3GPP TSG-RAN WG4 Meeting #114bis	R4-2504276

Wuhan, China, 7 – 11 April 2025

Source:	Ericsson, Verizon

Title:	TP for TR 38.719-03-01 to add UL CA_n66A-n77C for CA_n48-n66-n77

Agenda item:			6.3.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.719-03-01 to add UL CA_n66A-n77C for CA_n48A-n66A-n77C, CA_n48(2A)-n66A-n77C, CA_n48A-n66(2A)-n77C and CA_n48(2A)-n66(2A)-n77C.

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n48-n66-n77

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

.

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n48 | 3550 MHz – 3700MHz | 3550 MHz – 3700MHz | TDD |
| n66 | 1710 MHz –1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

| Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n48+n66+n77NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n48A-n66A-n77C | n777,9 CA_n48A-n66A CA_n66A-n77A7 CA_n77C |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 5012, 6012, 7012, 8012, 9012, 10012 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n48A-n66A CA_n66A-n77A CA_n66A-n77C CA_n77C |  | n48 | n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48(2A)-n66A-n77C | n777,9 CA_n77C CA_n48A-n66A CA_n66A-n77A7 |  | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS0 |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n77 | CA_n77C_BCS1 |  |
|  | CA_n77C CA_n48A-n66A CA_n66A-n77A CA_n66A-n77C |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48A-n66(2A)-n77C | CA_n77C CA_n48A-n66A CA_n66A-n77A CA_n66A-n77C |  | n48 | n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |
| CA_n48(2A)-n66(2A)-n77C | CA_n77C CA_n48A-n66A CA_n66A-n77A CA_n66A-n77C |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  |  | n66 | CA_n66(2A)_BCS4 and 5 |  |
|  |  |  | n77 | CA_n77C_BCS4 and 5 |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

##### 5.x.2.1.1	Co-existence studies for 2UL band with 1CC per band

Co-existence study has been performed in Rel-18.

##### 5.x.2.1.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.x.2.1.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products into band n48 interference analysis for CA_n66A-n77C with n77C transmitting with a 200 MHz maximum instantaneous bandwidth.

Table 5.x.2.1.2-1: Two UL band with intra-band ULCA Triple beat analysis.

| Band / CA1 | Single Carrier |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | flow | fhigh | fy_low | fy_high |
| fUL (n66, n77) | 1710 – 1780 |  | 3300 – 4200 |  |
| fDL (n48) | 3550 – 3700 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 – 200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 1510 – 1710 |  | 1780 – 1980 |  |
| Analysis | No Issue |  |  |  |
| Note 1: If the third band (nZ) are not part of the same or adjacent band groups, as defined in Table A-1, of band one (nX) or band two (nY), the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### 5.x.2.2	REFSENS requirements

Based on the triple beat analysis of the added ULCA, there are no triple beat IMD issues and no need to define MSD values.

---End of changes---

# Reference

[1] 		RP-250214, Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells; Ericsson, ZTE, Huawei