3GPP TSG-RAN WG4 Meeting #114bis                                    R4-2504288

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:              General discussion on AI general and scope

Agenda item:			7.21.1

Document for:	Discussion

Introduction

Some AI mobility general aspects were discussed extensively in last meeting. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

- Sub-use-case priorities

Observations from RAN2 simulation results

- Based on the observations summerized in our Tdoc submitted in the previous meeting, for RRM measurement prediction, at least 5 dimensional discussions can be involved in AI mobility (Evaluation scenarios, Sub-use-cases including cell-specific and/or cluster approaches, Sliding/Non-sliding filtering, OW and PW including MRRT/MRRS ratio), for each dimension, multiple elements are contained. According to the simulation results, the variation in each element in each dimension can indirectly/directly affects the prediction accuracy. However, considering full combinations of above all dimensions to define RRM requirements are unnecessary. In this sense, it is necessary to down select and focus on some/one of them to define requirements.

  - The simulation results evaluation for different Sub-use-cases based on simulation assumptions had begun in RAN2. Based on their simulation analyses and observations, majority of companies suggest to focus on L3 filtered cell level RSRP prediction since it can achieve excellent performance intuitively at least with shorter prediction window. It is most likely that RAN2 decide to study the spec. impact also for beam-level prediction. Meanwhile, for the sub-use-case 3, the model should additionally learn the logic of the beam consolidation mechanism, which does not need to be part of AI/ML model inference.

- Observations/ Analyses from RAN2 agreement on sptial domain prediction

（FR2 to FR2 intra-frequency spatial domain）

RAN2 #129 - Agreements on priority - RLF prediction will not be studied in Rel-19 - Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

First, we admit that scenario 6 is an important scenario, but we prefer to start the discussions in terms of scenario 2, 3, 4 agreed in previous meeting, which aligns to the mainstream discussions in RAN2. And, the intra-cell spatial RRM prediction can be categorized as the extension of Rel-18 AI/ML BM, which it is already studied in R18 AI/ML Air SI. Besides, if spatial RRM prediction is considered, from our understanding at least the following issues would be paid attention on: I. How to derive cell quality combined with predicted beam qualities and measured beam qualities; II. The feasibility of defining the requirements including cell detectability, measurement, side conditions and accuracy based on different beam related observation/prediction data, which are intricate.

- Observations from RAN2 agreement on measurement event prediction

RAN2 #129 - Agreements on priority - RLF prediction will not be studied in Rel-19

In accordance to the above analyzes, and to consistent with RAN2 simulation observations, we have the following proposals:

Proposal 1: As a starting point, RAN4 to consider the Sub-use-case 2 for the study of scenarios 2, 3, and 4 for cell-level prediction.

Proposal 2: RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility

Proposal 3: Accommodating with RAN2 guidance, there is no need for RAN4 to define RLF prediction requirement in Rel-19

3 Conclusion

In this contribution, we provided our viewpoints on general discussion on AI general and scope. The following observations and proposals are obtained:

Proposal 1: As a starting point, RAN4 to consider the Sub-use-case 2 for the study of scenarios 2, 3, and 4 for cell-level prediction.

Proposal 2: RAN4 to deprioritize studying the impacts on requirements for beam level spatial domain prediction (i.e., FR2 to FR2 intra-frequency spatial domain scenario 6) in AI mobility

Proposal 3: Accommodating with RAN2 guidance, there is no need for RAN4 to define RLF prediction requirement in Rel-19

4 References

[1] R4-2502594 “WF on AI/ML Mobility”, Nokia