3GPP TSG-RAN WG4 Meeting #114bis 	R4-2504295

Wuhan, China, 7 – 11 April, 2025

Source: 	Huawei, HiSilicon

Title: 	Further discussion on Expected EIRP test

Agenda Item:	7.13.5.1

Document for:	Approval

1 Introduction

In the last meeting RAN4#114 it was agreed to use the 21 point test direction set to measure the EEIRP in conformance and the CRs to introduce the EEIRP requirement and conformance tests were finalised.

A few open issues remain however;

- the measurement uncertainty budget and how to introduce the accuarccy introduced by the use of the test beam direction set to the measurement uncertainty calculation.

- the coordinate alignment text in the CR to 38.141-2 is still in square brackets

2 Discussion

2.1	Measurement uncertainty

2.1.2	Data set

In the WF [1] issue 2.6 it was agreed to collect the estimated errors between the agreed 21 point test directions set and the reference condition from different companies for the different BS configurations.

There are a limited set of configurations that have been agreed upon during the work to decide upon the test beam direction set. These have been collected in the TR.

During the process we have also investigated a number of other antennas and steering configurations.

The uncertainty in this case is defined as the error due to the use of the test direction set with different BS/ANT configurations and steering ranges. As such the larger number of BS configurations sampled then the better estimate of MU will be achieved. Initial investigations indicate that the standard deviation of both sets of data are similar so it is not strictly necessary for the data set to be the same for all contributions if the results are coming out with the same result.

2.1.2	Ellipse steering range

It was also agreed to consider the elliptical steering range for the analysis, there have been some questions on the type of ellipse assumed.

In our analysis we have assumed a standard ellipse

x2a2+y2b2=1

Where a and b are the peak steering directions in azimuth and elevation respectively.

If we compare this to the 3dB steering range of ANT2 for example:

Figure 1.1-1 Standard Ellipse (left) vs 3dB steering range (right) ANT2

We can see 2 things, 1. The steering range defined in elevation is greater than the 3dB gain of the antenna. 2. The 3dB steering range is slightly squarer than a standard ellipse.

The exact nature of the 3dB steering range is of course dependent on the antenna geometry, however as both the Ellipse and the rectangular steering ranges are considered the standard ellipse is useful as a corner point.

Proposal 1. The standard Ellipse can be used in the evaluation

As previously stated however, the intention of the statistical analysis is to consider a range of configurations and steering ranges, as such if different ellipse types are analysed we do not see this as a problem.

2.1.3	Analysis

The following cases were considered.

The test cases agreed so far are to use what we refer to as ANT4 (see table 2.1.2-1) with 2 different steering range configurations.

Table 2.1.2-1: Antenna parameters

The steering range configurations are:

Steering range 1

Mechanical tilt = 0deg

Electrical steering range = 0 to -12deg (0deg is perpendicular to the antenna, negative direction is towards the ground)

Electrical pre-set = -6deg

Steering range 2

Mechanical tilt = 0deg

Electrical steering range = 0 to -16deg (0deg is perpendicular to the antenna, negative direction is towards the ground)

Electrical pre-set = -6deg

It has been discussed that in practice Steering range 2 may need some additional mechanical tilt applied as such we have derived Steering range 3 which has 4 deg mechanical tilt.

Steering range 3

Mechanical tilt = 4deg

Electrical steering range = 0 to -16deg (0deg is perpendicular to the antenna, negative direction is towards the ground)

Electrical pre-set = -6deg

Looking at the errors for the ellipse steering range we have:

Figure 2.1.3-1: Histogram of EEIRP error (Ellipse steering rage)

If the rectangular steering range results are included as well then the standard deviation increases to 0.54dB.

If the cross company results captured in TR 38.908 are analysed then we gave the following distribution of errors:

Figure 2.1.3-2 – Histogram of EEIRP Errors – cross company results from TR 38.908

With standard deviations of:

- 0.3dB for the ellipse results

- 0.5dB for all cases.

The results from the two cases are very similar. Clearly considering more antennas configurations and steering ranges will increase the robustness of the analysis, however calculating the reference cases for each case is a lengthy process, so a large expansion of the test cases is not possible without allowing for a significant time window. However we believe the 5 antennas which have been considered represent a good sample of the antennas and steering ranges which will be used in this frequency range and provide enough range. Combined with the results form the cross company collection in TR 38.908 [2] which have a very similar standard deviation we can conclude that a the errors have an uncertainty of 0.5dB.

Proposal 2: The standard deviation of the EEIRP errors is 0.5dB.

2.2	Confidence interval

The confidence interval issues has been highlighted as a separate issue from the measurement uncertainty, however we believe it is the same issue.

The issue is based on the following statement in the regulation:

The averaging processes in steps 1 and 2 shall allow for accurate averaging of the expected e.i.r.p (e.g. to the confidence interval of 95%).

The averaging process in steps 1 and 2 refer to both the averaging of the angular measurement directions on the sampling grid and the number of beam steering directions inside the steering range.

Inaccuracies in these averaging processes occur when we move from an infinite number of sampling steps identified by the closed for equations and a discrete number of samples which are necessary for the practical measurement of the EEIRP.

As such the accuracy referred to in the regulatory text is part of the measurement uncertainty we have been estimating.

The example of a 95% confidence interval is not a separate requirement it is a statistical measure of how to represent that accuracy and is the same measure we apply to all our measurement uncertainty calculations.

As the EEIRP error due to averaging has a standard deviation of 0.5dB, we can say that within a confidence interval of 95% the accuracy of the averaging is within 1dB (0.5 *1.96).

However this figure in isolation is not of much use, the standard deviation (and statistical distribution) of the error is added to the measurement uncertainty calculation with the other measurement uncertainties so an MU for the whole measurement can be found.

2.3	Measurement uncertainty calculations

The EEIRP measurement is very similar to the in-band TRP measurements, as such the in-band emission MU budgets can be used as a starting point for the EEIRP budgets.

Additional sources of uncertainty are:

Dynamic range uncertainty

The dynamic range uncertainty has been identified as having a value of 0.5dB with a rectangular distribution

EEIRP sampling Error

The EEIRP sampling error has a standard deviation of 0.5dB with a gaussian distribution.

The MU tables are updated as such:

As with the TRP cases the CATR is slightly higher error so we can concentrate on this when considering the final MU value.

This results in a measurement uncertainty (at 95% CI) of 1.73dB

2.4	Coordinate alignment text

The text in the CR is currently in square brackets:

[If non-zero mechanical down-tilt is considered, the measured EEIRP profile needs to be shifted as:

θH=-θ+θMT

, where

θH – angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

 is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°.

MT is the down-tilt angle representing the mechanical tilt defined between -90° and +90°. ]

It has been pointed out that this is not the correct equation to apply tilt at every angle, and that the maths to tilt the full sphere of co-ordinates is more complex. This is true but is not really the purpose of the description. The text has been modified and the purpose of the description has been lost, originally it stated:

OTA spatial emissions are defined with respect to the horizon. The general BS coordinate system is defined with respect to the BS enclosure. BS orientation and the horizon are related by the mechanical tilt as follows:

This made the point the equations described alignment of the coordinate systems (which is really the issues rather than how to apply a transformation to the entire sphere of points as is not implied. The purpose of the text would be clearer if this was corrected.

Its also the case that there is an inversion in the sign of θ so the equation is required for alignment in all case not just when there is a non zero mechanical tilt

Observation 1: As there is an inversion in the sign of θ the equation applies at all times not just when there is a non-zero mechanical tilt.

As the equations are really only intended to describe the alignment of the coordinate systems this is done when φ=0, it can be note that the mechanical tilt is declared as a single value:

| D.64 | The mechanical tilt for EEIRP | The declaration of mechanical tilt together with D.10.(NOTE 21) |
| --- | --- | --- |

It is not stated but the implication is it is perpendicular to the antenna where φ=0.

The purpose of the text is not to describe mathematically how all points on the sphere are transposed but only to describe the differences between the 2 coordinate systems. This is most easily done with φ=0.

The different coordinate systems can cause confusion so it is better to keep the explanation of their alignment as simple as possible as such to keep the simple description but to make it clear that it is at φ=0.

Proposal 3: Add the clarification that φ=0 and modify the coordinate alignment text as follows:

OTA spatial emissions in table 6.9.5.1-1 are defined with respect to the horizon. The general BS coordinate defined in sub-clause 4.14 system is defined with respect to the BS enclosure. BS orientation and the horizon are related when φ=0, by the mechanical tilt as follows:

θH=-θ+θMT

, where

θH – angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

 is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°.

MT is the down-tilt angle representing the mechanical tilt defined between -90° and +90°.

Observation 2: Potentially the mechanical tilt definition should be clarified as being at φ=0.

3 Summary

In this paper we have further analysed the statistical distribution of the errors caused by the use of the 21 point test beam direction set with additional antenna configurations.

We have investigated bot ellipse and rectangular steering range distributions as such we propose to use the standard ellipse.

Proposal 1. The standard Ellipse can be used in the evaluation.

Statistical analysis of our own extended antenna set and steering ranges yield a standard deviation of the errrors of 0.5dB which is almost the same as that for the cross company set of results captured in TR 38.908 as such we propose:

Proposal 2: The standard deviation of the EEIRP errors is 0.5dB.

Finally the EEIRP measurement uncertainty budgets for the IA chamber and the CATR are updated with this uncertainty to give a measurement uncertainty for the EEIRP measurement of 1.7dB.

For the coordinate system alignment text in the CR we make the following proposal and observation:

Observation 1: As there is an inversion in the sign of θ the equation applies at all times not just when there is a non-zero mechanical tilt.

Proposal 3: Add the clarification that φ=0 and modify the coordinate alignment text as follows:

OTA spatial emissions in table 6.9.5.1-1 are defined with respect to the horizon. The general BS coordinate defined in sub-clause 4.14 system is defined with respect to the BS enclosure. BS orientation and the horizon are related when φ=0, by the mechanical tilt as follows:

θH=-θ+θMT

, where

θH – angle with respect to the horizon defined between +90° and -90°, above the horizon is positive below the horizon is negative. Note the orientation is opposite to 

 is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between -90° and +90°.

MT is the down-tilt angle representing the mechanical tilt defined between -90° and +90°.

Observation 2: Potentially the mechanical tilt definition should be clarified as being at φ=0.

References

[1]	R4-2502309 Way Forward for [114][302] NR_BS_RF_Part1_E_EIRP_final– ZTE

[2]	TR 38.908 Protection of fixed satellite service (FSS) UL within 6425 to 7125 MHz