3GPP TSG-RAN WG4 Meeting #114bis 	R4-2504300

Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom

Title: 	Deployment scenarios for FR1 SBFD macro BS

Agenda Item:	7.23.2

Document for:	Approval

Introduction

After the comprehensive study carried out by the Rel-18 SI for SBFD, the BS RF requirements for SBFD have been extensively discussed with good progress since the first meeting of the Rel-18 WI for SBFD. However, there are some disagreements on FR1 WA SBFD deployment scenario and corresponding RX in-band blocking and TX unwanted emission requirements. To facilitate RAN4 discussion, we provide our views in this contribution.

Discussion

We think the potential SBFD deployment scenarios especially for FR1 WA could be quite limited if RAN4 only focus on SBFD-TDD co-existence and define the RF requirements accordingly. Following analyses are mainly based on the Rel-18 SBFD SI outcomes.

- Per the conclusions for Case 3 SBFD UL co-existence evaluation in TR 38.858 clause 11.3, the throughput degradation is increased with grid-shift lower than 100%.

Case 3 SBFD UL co-existence conclusions

| Deployment Scenario (Aggressor -> Victim) | Frequency range | Co-existence conclusion |
| --- | --- | --- |
| Urban Macro -> Urban Macro Scenario 1 | FR1 | Under baseline assumptions, SBFD UL throughput degradation is observed only for cell edge throughput, and minor degradation but acceptable to some companies for average throughput. With other assumptions (higher BS Tx power and lower grid-shifts), the degradation is increased for cell edge throughput and average throughput. |

But it should be noted that the 100% grid-shift might be too ideal for macro BS comparing with real deployment. Operators in adjacent channels in the same band have been collocating macro BS since the beginning of cellular wireless to minimize the near-far effect on ACIR and IMD. When collocation is not possible, sites in the vicinity of each other will be acquired. As a result, we assumed lower grid-shift, e.g., 20% considering macro BS deployment today. Thus the inevitably increased CLI due to such deployment-dependent site distance would be worthy of attention from this point of view.

- Per those findings in TR 38.858 clause 11.2, the exercises on enhancing ACIR had been conducted by RAN4. With the principle that no change on ACLR/ACS from legacy TDD BS or UE can be expected, the ACIR gain respective to each concerned case can be found as below.

ACIRvictim=11ACLRaggressor+1ACSvictim

  - Case 1 (Legacy UE as a victim and SBFD network as an aggressor): The BS-to-UE ACIR enhancement cannot exceed 0.3 dB in FR1 and 1.2 dB in FR2-1 based on the limitation of the UE ACS simulation assumption.

  - Case 2 (Legacy BS as a victim and SBFD network as aggressor): The BS-to-BS ACIR enhancement cannot exceed 6.2 dB in FR1 and 1.5 dB in FR2-1 based on the limitation of the legacy BS ACS simulation assumption.

  - Case 3 (SBFD-capable BS as a victim and legacy BS as aggressor): The BS-to-BS ACIR enhancement cannot exceed 1.2 dB in FR1 and 5.5 dB in FR2-1 based on the limitation of the legacy BS ACLR simulation assumption.

  - Case 4 (SBFD-aware UE as a victim and legacy UE as aggressor): The UE-to-BS ACIR enhancement is 0 dB in FR1 and cannot exceed 3.0 dB in FR2-1 based on the limitation of the legacy UE ACS.

Obviously, The ACIR gain would be subject to the legacy RF requirements and has limited help for CLI mitigation.

However, for a new wide TDD band without existing deployment of legacy TDD base stations, it would be more beneficial to deploy SBFD BS to increase UL coverage and reduce latency to realize the full potential of SBFD technology In this case improved adjacent channel co-existence between an SBFD BS with another SBFD BS would be feasible by enhancing the ACIR, i.e. defining additional requirements both transmitter side and receiver side. As shown in below figure, for protection of SBFD UL of operator B, the SBFD DL of operator B can be adopted as guard band for filter implementation of adjacent operator A. It would not impose additional implementation difficulty for taking the advantage of per operator filter.

More specifically, we reran the RAN4 co-ex simulation with 20% grid-shift to see how much UL throughputs degradation can be reduced with the help from the increased ACIR. Note that the baseline UL throughput loss i.e. applying the existing 42.5dB ACIR determined by 45dB BS ACLR and 46dB BS ACS, is 100% at cell-edge and 22.88% at cell-average, respectively.

| 20% grid shift UL throughput loss |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ACIR(dB) UL Thr. Loss @X%-tile | 43 (baseline) | 62.5 | 65 | 67.5 | 70 | 72.5 |
| @5% (cell-edge) | 100% | 22.18% | 18.31% | 9.96% | 4.29% | 2.23% |
| @50% (cell-average) | 22.88% | 2.64% | 1.91% | 1.40% | 0.98% | 0.64% |

From the above table, it is observed that around 30 dB improvement on ACIR can be enough to guarantee <3% UL throughput loss at cell edge comparing to the baseline, while for cell average the loss is almost reduced to none.

In conclusion, we think SBFD-SBFD co-existence for FR1 macro BS is an important deployment scenario and should be considered in Rel-19. Although Rel-19 may still be an early experiment of flexible duplexing, it is reasonable to expect the new SBFD BS hardware come with enhanced ACIR for better co-existence and performance with an adjacent SBFD BS to come a later. This way we can continuously evolve flexible duplexing technologies in 5G Adv. and 6G for better performance and deployment flexibility. The enhanced requirements in the analysis above are only for SBFD-capable BS w/o any legacy TDD impact, the final requirement should be based on RAN4 co-ex simulation practise and realistic consideration regarding grid-shift and other parameters if necessary.

Proposal 1: At least for FR1 macro BS it is confirmed the deployment scenario of SBFD-SBFD co-existence in adjacent channels shall be within the scope of Rel-19.

Conclusion

In this contribution we discussed the deployment scenarios for FR1 SBFD macro BS.

Proposal 1: At least for FR1 macro BS it is confirmed the deployment scenario of SBFD-SBFD co-existence in adjacent channels shall be within the scope of Rel-19.

- References

- TR 38.858.