3GPP TSG-RAN WG4 Meeting #114bis 	R4-2504302

Wuhan, China, 7th – 11st April, 2025

Source: 	Huawei, HiSilicon

Title: 	On modification of existing RX RF requirements for SBFD

Agenda Item:	7.23.2.3

Document for:	Approval

1 Introduction

During RAN4#114, BS RF requirements for SBFD have been discussed [8]. In this contribution, we would like to share our views regarding the open issues.

2 Discussion

2.1 ACS requirement

Per WF in [6],

WF:

- FFS on the followings:

  - The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD.

For ACS and in-band blocking, both requirements are defined out of the BS channel bandwidth instead of uplink subband bandwidth. ACS and in-band blocking are defined with different interferer frequency offsets at adjacent channel(s). For channel bandwidth wider than 20 MHz, ACS interferer frequency offset is ~9.5 MHz and in-band blocking interferer frequency offset is 30 MHz, using 20 MHz NR interfering signal. And value of the interferer level is 9 dB offset. Hence, we propose to consider the two requirements together.

Proposal 1: it is proposed to consider the ACS and in-band blocking requirements together. The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level.

2.2 In-band blocking

Per WF in [7],

Agreement: - For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS.

There are some consideration on the consideration on the coexistence scenarios in [9] and it concludes that SBFD-SBFD co-existence for FR1 macro BS is an important deployment scenario and should be considered in Rel-19. From the discussion in last meeting, it is clear that it would result different in-band blocking requirements for SBFD-TDD adjacent operator scenario and SBFD-SBFD adjacent operator scenario.

Proposal 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

Firstly for SBFD-TDD adjacent operator scenario, due to the principle that no change on the ACLR/emission from legacy TDD BS, improving the blocking capability of SBFD only would not improve the co-existence performance further. Assuming the actual ACLR/emission is ideally 10 dB lower than that in the specification, higher than -27 dBm (-43 dBm-10-6) may not be helpful, which is already higher than the simulation results with 100% grid shift. Hence from our results with 100% grid shift, it is 17 dB higher than existing in-band blocking limits.

Proposal 3: As a general requirement, it is proposed to adopt 100% grid shift to derive the in-band blocking level for SBFD-TDD adjacent operator scenario.

Secondly, we provide the study on SBFD-SBFD adjacent operator scenario.

In our view eventually the same grid shift assumption should be adopted for both TX and RX requirements derivation since both requirements are defined for SBFD BS and the same performance shall be guaranteed under the same deployment.

As those requirements are actually the guarantee of SBFD network performance, we feel the need to collect some measurements based on real deployment in order to cross-check the reliability of RAN4 co-ex simulation. To be specific, we extract the site location from a region where typical co-existence among operators with active NR traffic can be identified. Note that the concerning deployment is dense urban macro in C band, co-location case is also included.

According to the CDF of site distance, following characteristics for such real deployment can be observed:

- Site distance is smaller than 10m @11%-tile of the CDF

- Site distance is smaller than 50m @45%-tile of the CDF

While in RAN4 co-ex simulation, 10% grid shift (site distance sets to 500m) means that the adjacent BS is only 28.8m away, the distance would increase to 57.6m for 20% grid shift and to 144m for 50% grid shift.

Based on base station locations in one example region, the CDF of in-band blocking is simulated and the results are shown below. It is should be noted the statistics the co-location cases is not excluded. If it does the number will further lower.

And our RAN4 simulation results are also put here for comparasion, details can be seen in clause 2.4. So it is obvious that using grid shift less than 20% as RAN4 co-ex assumption will lead to an over stringent requirement.

Table 2.1-1: blocking level @ 99%CDF

| Blocking level(dBm) |  |
| --- | --- |
| Grid-shift | 99% CDF |
| 10% | -5.1 |
| 20% | -12.4 |
| 50% | -22.4 |
| 100% | -25.8 |

Observation 1: using grid shift less than 20% as simulation assumption will result over stringent requirement.

For some scenarios which could lead smaller site distance than 20% grid shift, we think it can be addressed by co-location limits.

Observation 2: For some scenarios which could lead smaller site distance than 20% grid shift, we think it can be addressed by co-location limits.

Proposal 4: It is proposed to adopt 20% grid shift to be used for both TX and RX requirements definition for SBFD-SBFD adjacent operator scenario.

From the simulation result of RAN4 simulations and the simulation based on the BS locations of a real deployment at C band, it is proposed to define in-band blocking interferer signal power to -15 dBm for FR1 WA BS.

Proposal 5: It is proposed to define in-band blocking interferer signal power to -15 dBm for FR1 WA BS.

2.3 Receiver intermodulation

Per WF in [4],

Necessity of New RX intermodulation requirement with 1 interfering signal WF: - FFS on the following options: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to define new RX intermodulation scenario with self-interference and 1 interfering signal. Necessity of New RX intermodulation requirement with 2 interfering signals WF: - FFS on the following: - Option 1: - for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - for receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: new IMD requirement with 2 interfering signal scenario is not needed. - Option 3: No need to define new RX intermodulation requirement with 2 interfering signals if new requirement with self-interference and 1 interfering signal is defined. - Option 4: The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity. For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

The motivation of the two new RX intermodulation requirements are to consider BS2BS cross link interference. For general in-band blocking case as discussed in clause 2.4, the stronger intermodulation product from closer in-band blocking blocker and self sub-band DL TX is already covered. Hence there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s).

Proposal 6: there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s). The two new requirement is implicitly captured by the SBFD RX blocking requirement.

2.4 Text Proposals for RX

At last meeting in [10], it was agreed to use a separate chapter for requirements for SBFD-capable BS. Some baseline proposals are agreed in [8]. It includes the BS type 1-H, 1-O and 2-O, so both conducted requirements and radiated requirements should be defined. To keep the same levels for SBFD requirements, the following clauses skeleton are proposed for RX requirements.

Proposal 7: the following clauses skeleton are proposed for RX requirements.

=================== Start of Text Proposal ===================

14	Conducted transmitter characteristics for SBFD capable BS

14.1	General

14.2	Reference sensitivity level for SBFD

14.3	Dynamic range for SBFD

14.4	In-band selectivity and blocking for SBFD

14.5	Out-of-band blocking for SBFD

14.6	Receiver spurious emissions for SBFD

14.7	Receiver intermodulation for SBFD

15	Radiated receiver characteristics for SBFD capable BS

15.1	General

15.2	OTA sensitivity for SBFD

15.3	OTA reference sensitivity level for SBFD

15.4	OTA dynamic range for SBFD

15.5	OTA in-band selectivity and blocking for SBFD

15.6	OTA out-of-band blocking for SBFD

15.7	OTA receiver spurious emissions for SBFD

15.8	OTA receiver intermodulation for SBFD

15.9	OTA in-channel selectivity for SBFD

=================== End of Text Proposal ===================

3 Conclusion

In this contribution we discussed on the BS RX requirement for SBFD. We have the following observations and proposals:

Proposal 1: it is proposed to consider the ACS and in-band blocking requirements together. The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level.

Proposal 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

Proposal 3: As a general requirement, it is proposed to adopt 100% grid shift to derive the in-band blocking level for SBFD-TDD adjacent operator scenario.

Observation 1: using grid shift less than 20% as simulation assumption will result over stringent requirement.

Observation 2: For some scenarios which could lead smaller site distance than 20% grid shift, we think it can be addressed by co-location limits.

Proposal 4: It is proposed to adopt 20% grid shift to be used for both TX and RX requirements definition for SBFD-SBFD adjacent operator scenario.

Proposal 5: It is proposed to define in-band blocking interferer signal power to -15 dBm for FR1 WA BS.

Proposal 6: there is no need to define new RX intermodulation requirement with 1 or 2 interfering signal(s). The two new requirement is implicitly captured by the SBFD RX blocking requirement.

Proposal 7: the clauses skeleton in clause 2.4 are proposed for RX requirements.

References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.

- R4-2409958, Way Forward for [111][313] NR_duplex_evo,	Samsung

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Huawei, HiSilicon

- R4-2416528, Topic summary for [112bis][307] NR_duplex_evo_BSRF, Moderator (Huawei)

- R4-2419908, Way Forward for [113][307] NR_duplex_evo_BSRF, Huawei

- R4-2419245, On modification of existing RX RF requirements for SBFD, Huawei

- R4-2502380, Way Forward for [114][307] NR_duplex_evo_BSRF, Huawei

- R4-2504300, Deployment scenarios for FR1 SBFD macro BS, Huawei, HiSilicon, Samsung, Qualcomm, Verizon, Vodafone, China Telecom, China Unicom

- R4-2502310, Way Forward for [114][306] NR_duplex_evo_General, Samsung