3GPP TSG-RAN WG4 Meeting # 114bis	R4-25xxxxx

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Simulation results on BS MMSE-IRC requirements with inter-cell interference

Source: 	Huawei, HiSilicon

Agenda item:	7.18.3

Document for:	Discussion

Background

According to WF R4-2419780 [1], we provide our simulation results for BS MMSE-IRC requirements with inter cell interference.

- Simulation results

In this clause we provide our initial simulation results。

Table 3-1: Simulation assumptions

| Assumptions |  | Serving cell | Interference cell 1 | Interference cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | 5MHz/15kHz, 20MHz/15kHz, 10MHz/30kHz 100MHz/30kHz |  |  |
| MCS |  | 16 | 16QAM random symbols | 16QAM random symbols |
| Propagation conditions: |  | TDLC300-100 for DIP set1 TDLA30-10 for DIP set2 | TDLC300-100 Low | TDLC300-100 Low |
| Antenna configuration |  | 1T2R, 1T4R,1T8R |  |  |
| Interference model |  |  | DIP set1: DIP1=-1.11dB DIP set2: DIP1=-0.43dB | DIP set1: DIP2=-10.91dB DIP set2: DIP2=-13.78dB |
| DM-RS | DM-RS configuration type | 1 | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB | -3 dB |
|  | DM-RS port | {0} | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Receiver assumption |  | MMSE-IRC | N/A | N/A |

The simulation results are captured in Table 3-2:

Table 2-2: Simulation results

| Bandwidth/SCS | DIP Set | Propagation conditions | MCS | Antenna configuration | SNR(dB)@70% of maxTP |
| --- | --- | --- | --- | --- | --- |
| 5MHz/15kHz | DIP Set1 | TDLC300-100 | MCS16 | 1T2R | 11.5 |
|  |  |  |  | 1T4R | 7.2 |
|  |  |  |  | 1T8R | 4.7 |
|  | DIP Set2 | TDLA30-10 | MCS16 | 1T2R | 8.8 |
|  |  |  |  | 1T4R | 5.2 |
|  |  |  |  | 1T8R | 3.2 |
| 20MHz/15kHz | DIP Set1 | TDLC300-100 | MCS16 | 1T2R | 10.5 |
|  |  |  |  | 1T4R | 6.3 |
|  |  |  |  | 1T8R | 3.6 |
|  | DIP Set2 | TDLA30-10 | MCS16 | 1T2R | 9.3 |
|  |  |  |  | 1T4R | 5.5 |
|  |  |  |  | 1T8R | 3.1 |
| 10MHz/30kHz | DIP Set1 | TDLC300-100 | MCS16 | 1T2R | 9.7 |
|  |  |  |  | 1T4R | 6.1 |
|  |  |  |  | 1T8R | 3.5 |
|  | DIP Set2 | TDLA30-10 | MCS16 | 1T2R | 9.7 |
|  |  |  |  | 1T4R | 5.8 |
|  |  |  |  | 1T8R | 3.2 |
| 100MHz/30kHz | DIP Set1 | TDLC300-100 | MCS16 | 1T2R | 10.5 |
|  |  |  |  | 1T4R | 6.3 |
|  |  |  |  | 1T8R | 3.8 |
|  | DIP Set2 | TDLA30-10 | MCS16 | 1T2R | 9.8 |
|  |  |  |  | 1T4R | 5.9 |
|  |  |  |  | 1T8R | 3.3 |

- Conclusion

This contribution provides our simulation results on BS MMSE-IRC requirements.

- Reference

[1]   R4-2502272 Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS