3GPP TSG-RAN WG4 Meeting #114bis	R4-25xxxxx

Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Overviews on 6Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.3

Document for:	Discussion

Background

According to the TU assignment, RAN4 shall start working on 6Rx performance requirements in RAN-114 bis meeting. This contribution provides our overviews.

- Discussions

General

According to [1], following objectives related to performance part are made:

6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx

It can be seen that test scope is limited to PDSCH requirements with rank4/6 and SDR requirements. Therefore, RAN4 shall follow the objective and preclude any other test case.

Proposal 1: RAN4 to define following test cases for R19 6Rx performance requirements and don’t consider other test cases:

PDSCH requirements with 4 layers and 6 layers

SDR requirements

According to [2], it was agreed to capture the fact that “ 6 layers is only supported for FWA shall be explicitly captured at least in the RAN4 specification”:

Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

Considering performance part shall follow the core part, hence it is necessary to define the capability rules to capture this agreement.

Proposal 2: RAN4 to define the applicability rules that all test cases corresponding to 6layers are only applicable for FWA devices. The wording can be further discussed.

According to [1], 6Rx is only supported for FDD and single carrier scenario:

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104

Hence, performance part shall follow the WID to only focus on FDD and single carrier scenario.

Proposal 3: Performance requirements shall only be defined for TDD and single carrier scenario.

PDSCH requirements

We propose to use MCS13 and TDLA30-10 as start point for 4layers and 6layers performance requirements definition, which are typical test setups for high rank test cases.

Proposal 4: RAN4 to use MCS13 and TDLA30-10 as starting point for 4layers and 6layers tests.

Other simulation assumptions could be reused from existing PDSCH test, which is captured in Table 2.2-1:

Table 2.2-1: Simulation assumptions for PDSCH test

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | 30/40 |
| MCS |  |  | 13 |
| MIMO layer and Antenna configuration |  |  | Rank 4: 4T6R Rank 6: 8T6R |
| Propagation condition and antenna correlation |  |  | TDLA30-10 Low |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Rank 4: Single-symbol Rank 6: Double-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| Overhead |  |  | 0 |
| N1 and N2 configurations for 8Tx cases |  |  | For 8Tx test: (N1,N2) = (4,1), (O1, O2) = (4,1) For 4Tx test: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 8 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Test metric |  |  | SNR@70% max TP |

Proposal 5: Use simulation assumptions in Table2.2-1 for PDSCH test

SDR requirements

Static channel model

For 1T6R, it’s easy to get the static channel as follows:

H=111111

For 2T6R, it’s easy to get the static channel as follows:

H=111j-jj111-jj-j

For 4T6R, it’s easy to get the static channel as follows:

H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

For 8T6R, channel matrix can’t be designed as column orthogonal since the rank is limited by number of Rx (row), which is smaller than Tx(column), which will cause inter-layer interference when random precoding is assumed.  It shall be guaranteed that no additional interference is existing during the SDR test. To address this issue, our propose is to use fixed precoder instead and derive the specific static matrix by the following procedure:

          - Fixed precoder w at i1,1=i1,2=i2=0

          - Derive the static channel model for 8T6R as follows:

H=wHw-1wH=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

It can be easily proved that the final channel matrix from layer to Rx is Hw=wHw-1wHw=I, which means layers to Rx is one to one mapping and there is no inter layer interference.

Observation 1: For 8T6R, static channel matrix can’t be designed as column orthogonal since the rank is limited by number of Rx (row), which is smaller than Tx(column), which will cause inter-layer interference when random precoding is assumed.

Proposal 6: RAN4 to use following static channel matrix and precoding for 6Rx SDR test:

For 1T6R(Random precoding):

H=111111

For 2T6R(Random precoding):

H=111j-jj111-jj-j

For 4T6R (Random precoding):

H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Others:

Up to 1024QAM is defined for 2Rx,4Rx and 8Rx, it can also be applied for 6Rx. For 1024QAM, same as 8Rx, 2 layers can be used as starting point. For 64QAM and 256QAM, it is proposed to reuse existing MCS values for 4 layers and focus on the study for 6 layers. Same as legacy procedure for SDR test, RAN4 evaluates the maximum achievable MCS for MCS Table 1 and MCS Table 2. Meanwhile, we propose to reuse the legacy TxEvm assumptions to derive maximum test SNR. I.e. 6% for 64QAM, 3% for 256QAM and 2.5% for 1024QAM.

Proposal 7:  RAN4 to define the 6Rx SDR requirements with following assumptions as starting point:

1024QAM: 2layers

For 64QAM and 256QAM, reuse existing MCS values for 2 and 4 layers and define new requirements for  6layers

Proposal 8:  RAN4 to use following TxEvM assumptions to derive maximum test SNR:

6% for 64QAM, 3% for 256QAM and 2.5% for 1024QAM.

For other assumptions we propose to reuse the existing configurations, which is shown in Table 2.3-1:

Proposal 9: RAN4 to use assumptions in Table 2.3-1 for SDR test.

Table 2.3-1: Simulation assumptions for SDR test:

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) DL |  | Rank2: Single-symbol Rank 8: Double-symbol |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1005} for 6 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2layers 2 for 6 layers |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Number of HARQ processes |  |  | 8 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen for 2 layers. Single Panel Type I, fixed precoding with i1,1=i1,2=i2=0 for rank6 |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 6 1024QAM: 2 |
| Test metric |  |  | 85% of maximum throughput |
| Antenna configuration | 2 layers |  | 2x6 |
|  | 8 layers |  | 8x6 |

- Conclusion

This contribution provides our overviews on 6Rx performance requirements, the proposals and observations are:

General:

Proposal 1: RAN4 to define following test cases for R19 6Rx performance requirements and don’t consider other test cases:

PDSCH requirements with 4 layers and 6 layers

SDR requirements

Proposal 2: RAN4 to define the applicability rules that all test cases corresponding to 6layers are only applicable for FWA devices. The wording can be further discussed.

Proposal 3: Performance requirements shall only be defined for TDD and single carrier scenario.

PDSCH requirements:

Proposal 4: RAN4 to use MCS13 and TDLA30-10 as starting point for 4layers and 6layers tests.

Proposal 5: Use simulation assumptions in Table2.2-1 for PDSCH test

SDR requirements:

Observation 1: For 8T6R, static channel matrix can’t be designed as column orthogonal since the rank is limited by number of Rx (row), which is smaller than Tx(column), which will cause inter-layer interference when random precoding is assumed.

Proposal 6: RAN4 to use following static channel matrix and precoding for 6Rx SDR test:

For 1T6R(Random precoding):

H=111111

For 2T6R(Random precoding):

H=111j-jj111-jj-j

For 4T6R (Random precoding):

H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Proposal 7:  RAN4 to define the 6Rx SDR requirements with following assumptions as starting point:

1024QAM: 2layers

For 64QAM and 256QAM, reuse existing MCS values for 2 and 4 layers and define new requirements for 6layers

Proposal 8:  RAN4 to use following TxEvM assumptions to derive maximum test SNR:

6% for 64QAM, 3% for 256QAM and 2.5% for 1024QAM.

- Proposal 9: RAN4 to use assumptions in Table 2.3-1 for SDR test.

- Reference

[1] RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4. Huawei, HiSilicon

[2] R4-2503014 WF on 6Rx requirements. AT&T