3GPP TSG-RAN WG4 Meeting #114bis	R4-2504314

# Wuhan, CN, 07 - 11 Apr, 2025

Title: 	Discussion on NTN testing for NGSO channel model

Source: 	Huawei,HiSilicon

Agenda item:	7.10.4.1

Document for:	Discussion

Background

During RAN4#114 meeting, Way Forward [1] for NTN_testing_NGSO_channel_model was approved. In this contribution, we share our view on NTN testing for NGSO channel model.

Discussion

TE emulated channel model for satellite mobility

Correction of Propagation Delay Calculation

- WF - RAN4 discuss whether the Earth's rotation is considered during signal flight time for more accurate estimations of propagation delay - Considering the earth rotation, the propagation delay, DelayERtR can be derived as DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR> Where tR is the signal’s received time, rtRECEFand vtRECEF are the satellite position and velocity vectors at time tR , ρtRis the difference in satellite and UE position vectors at time tR, K is the unit vector in standard z-axis, (a×b) is the cross-product and <a, b> is the dot product between a and b - The proponent is encouraged to provide the detail information, e.g., reference, if possible, about the propagation delay calculation with considering the signal flight time before RAN4#114bis meeting - Participating companies are encouraged to examine this issue from their perspective.

Our evaluation results are shown as following based on initial ephemeris information agreed in last meeting. We can see that the DL delay error is rather small, this is because the inclination angle is near to 90°. In the real deployment, a larger DL delay error value can be observed if the inclination angle is smaller or UE is moving with large speed. For the uplink, the delay error is obviously larger, this is not because earth rotation but because satellite motion during the signal transmission. To make the TE emulated channel model more accurate, we propose to also consider delay error caused by satellite motion in uplink.

The uplink delay error is larger than the downlink delay error due to satellite motion rather than earth rotation.

Consider downlink delay error caused by earth rotation and uplink delay error caused by satellite motion.

For the propagation delay calculation, the following figure is illustrated. For downlink, the delay is calculated from the satellite at the time t to the UE at the time t+ΔT; For the uplink, the delay is calculated from the UE at the time t-ΔT to the satellite at the time t; where ΔT is the signal transmission time.

For downlink, calculate delay from the satellite at the time t to the UE at the time t+ΔT; For the uplink, calculate delay from the UE at the time t-ΔT to the satellite at the time t; where ΔT is the signal transmission time.

Accuracy requirement to solve Kepler’s equation based on Newton-Raphson method

- Proposals - Option 1: E-E0< ϵ=10-8

The accuracy requirement to solve the Kepler’s equation based on Newton-Raphson method is an intermediate step to calculate the Doppler and delay. From our understanding, it is better up to implementation and no accuracy requirement to solve Kepler’s equation based on Newton-Raphson method.

No accuracy requirement to solve Kepler’s equation based on Newton-Raphson method.

Number of iterations to solve Kepler’s equation based on Newton-Raphson method

- Proposals - Option 1: 4 - Option 2: Do not consider the number of iterations as a stopping criterion for Newton-Raphson

Here we list the impact of the different number of iterations. We can see that high accuracy can be achieved with a smaller number of iterations, e.g. 2.

| Number of iterations | Accuracy | Doppler | Delay |
| --- | --- | --- | --- |
| 1 | 1e-7 | 1e-5 | 1e-8 |
| 2 | 1e-12 | 1e-12 | 1e-12 |
| 3 | 1e-12 | 1e-12 | 1e-12 |
| 4 | 1e-12 | 1e-12 | 1e-12 |

The iteration to solve Kepler’s equation based on Newton-Raphson method is an intermediate step to calculate the Doppler and delay. From our understanding, it is better up to implementation and not consider the number of iterations as a stopping criterion for Newton-Raphson

Do not consider the number of iterations as a stopping criterion for Newton-Raphson.

    - Interpolation method to obtain the propagation delay and Doppler shift trajectory

- Proposals - Option 1: Cubic interpolation - Option 2: linear interpolation - Option 3: Open to not restrict the implementation on the TE side. However, RAN4 should discuss and determine an acceptable error value introduced by interpolation - Option 4: With the assumption of UE updates the satellite location/velocity at least every 1 second. The TE may update Doppler Shift and Propagation delay every slot linear interpolation or cubic spline interpolation. It is up to TE implementation which interpolation method to be used.

From our understanding, the interpolation method to obtain the propagation delay and Doppler shift trajectory should be up to implementation. Some accuracy requirement about the Doppler and delay should be fulfilled at the TE side.

The interpolation method to obtain the propagation delay and Doppler shift trajectory should be up to implementation. Some accuracy requirement about the Doppler and delay should be fulfilled at the TE side.

    - Accuracy requirement of maximum delay errors to obtain the propagation delay and Doppler shift trajectory

- Proposals - Option 1 - Max ¦delay-error¦~5e-10 s - Max ¦delay-rate error¦~1e-10 s/s - Option 2 - RAN4 to define a simulation assumption/TE requirement on interpolation accuracy when quantifying the NGSO demodulation for dynamic conditions so all test equipment can provide equivalent results - Option 3 - Do not consider accuracy requirements for the estimation of propagation delay and Doppler Shift

For the accuracy requirement of maximum delay errors to obtain the propagation delay and Doppler shift trajectory, we think Option 1 is acceptable.

Consider Max |delay-error|~5e-10 s and Max |delay-rate error|~1e-10 s/s.

Assumption of Elevation angle range

- FFS whether to define the ephemeris information for NGSO LEO 1200 - For RF test - Evaluation angle: FFS - Initial ephemeris information: FFS - FFS on how to solve the problem of long testing time for IoT NTN - Define the test applicability rule for the tests which are applied for TE-emulated time varying channel model testing

For the RF tests, we propose to use the elevation angle range starting from 30° to align with RRM/demod.

Use the elevation angle range starting from 30° to align with RRM/demod.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- The uplink delay error is larger than the downlink delay error due to satellite motion rather than earth rotation.

- Consider downlink delay error caused by earth rotation and uplink delay error caused by satellite motion.

- For downlink, calculate delay from the satellite at the time t to the UE at the time t+ΔT; For the uplink, calculate delay from the UE at the time t-ΔT to the satellite at the time t; where ΔT is the signal transmission time.

- No accuracy requirement to solve Kepler’s equation based on Newton-Raphson method.

- Do not consider the number of iterations as a stopping criterion for Newton-Raphson.

- The interpolation method to obtain the propagation delay and Doppler shift trajectory should be up to implementation. Some accuracy requirement about the Doppler and delay should be fulfilled at the TE side.

- Consider Max |delay-error|~5e-10 s and Max |delay-rate error|~1e-10 s/s.

- Use the elevation angle range starting from 30° to align with RRM/demod.

Reference

R4-2502314, Way Forward for [114][325] NTN_testing_NGSO_channel_model, RAN4#114, Samsung