ye3GPP TSG-RAN WG4 Meeting #114bis	R4-2504327

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.18.2

Source:	Ericsson

Title:	On MMSE-IRC receiver for interference mitigation with 8Rx

Document for:	Discussion

# 1	Introduction

Following the objectives of the WID, companies have reached several agreements below with respect to the test scope:

Test scope - RAN4 to introduce PDSCH demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx - RAN4 to introduce CQI reporting requirements under ICI for 8Rx

To establish parameter assumptions for the test cases, it was encouraged to share initial evaluations during this meeting to assess the performance gain of the IRC receiver compared to the MMSE receiver. Following the last meeting’s discussion of down-selecting parameter assumption candidates, we shared our simulation results for PDSCH demodulation performance and CQI reporting based on the agreed evaluation cases in our companion simulation results paper. In this contribution, we present our perspectives along with observations obtained from the evaluations to facilitate better determination of the parameter assumptions.

# 2	Inter-cell interference

In this section, we discussed the scenario of inter-cell interference cancellation with 8Rx reception.

## 2.1	PDSCH requirements with inter-cell interference

As a result of last RAN4 #114 meeting’s discussion, following parameter assumptions are further agreed on top of the previous agreements:

Issue 1-2-1: MCS and rank Agreement: - Do not introduce requirements for Rank 1. - Introduce requirements for Rank 2 with HomNet (2 interference cells) and Rank 4 with HetNet (1 interference cell) - Do not introduce applicability rules to have UE only being tested for either rank 2 or rank 4.

From our simulation results, we have observed ideal gain from IRC receiver over MMSE receiver for the following agreed test cases below.

Table 2.1-1 PDSCH simulation results for 2 and 1 interference cell (baseline)

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 2 interference cells |  |  |  |  |  |  |  |  |
| 2 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 7.26 | 13.22 | 5.96 |
| 1 interference cell |  |  |  |  |  |  |  |  |
| 6 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 7.30 | 12.83 | 5.53 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |

Therefore, we confirm to define PDSCH requirement with the following MCS plus rank combo configurations:

- MCS17 + Rank 2

- MCS13 + Rank 4

Proposal 1: Confirm to define PDSCH requirements with MCS17+rank 2 and MCS13+rank 4

## 2.2	CQI reporting

It has been agreed to include CQI reporting requirements for inter-cell interference scenario following legacy assumptions from Rel-17 that is to assume fading condition for the target cell and Static channel condition for the interference cell. It’s encouraged for companies to bring simulation results for both rank 1 and rank 2 (for the target cell) to evaluate the performance to facilitate the main open issue for the CQI reporting requirements: rank.

Figure 2.2-1 Throughput for interference model and white noise model

We provided the figure of gamma for both rank 1 and rank 2 as above. Gamma value (Gain to AWGN) refers to the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source.

From the simulation results above, we can observe that the performance of IRC receiver with both rank 1 and rank 2 showed optimal gain over the performance of MRC, and good gamma over the AWGN model.

Observation 1: Both rank 1 and 2 are feasible for the requirement definition

According to the following agreements reached in RAN4 #114 meeting, we propose to consider rank 2 for the requirement definition.

Issue 1-4-2: Rank for serving cell Agreement: - If simulation results show both rank 1 and rank 2 to be feasible for requirement definition, then Rank 2 will be selected.

Proposal 2: Consider rank 2 for CQI reporting requirement definition under ICI scenario

More CQI reporting results can be found in our companion paper [2].

We also provided the comparison between performance of baseline receiver and simplified receiver:

Figure 2.2-2 Gain comparison for baseline & simplified receiver

Based on the results, there is only minor performance difference between two different receiver types.

In this case, we propose to only define one CQI reporting requirement and apply it to both receiver types.

Proposal 3: RAN4 to only define one CQI reporting requirement applying to both receiver types.

# 3	Intra-cell inter-user interference

In this section, we discussed the scenario of intra-cell inter-user interference cancellation with 8Rx reception.

## 3.1	PDSCH requirements with intra-cell inter-user interference

As a result of last RAN4 #114 meeting’s discussion, following parameter assumptions are further agreed on top of the previous agreements:

Issue 1-3-1: MCS for Rank 2+2 Agreement: - For Rank 2+2, use MCS17 for requirement definition. Issue 1-3-2: Rank 4+4 Proposals: - Option 1: Do not define Rank 4+4 requirements. - Option 2: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

One open issue is specified and discussed below.

Whether to consider the rank 4+4 case

We have evaluated the possibility of introducing requirements for rank 4+4 from simulation point of view.

Following are the expected simulation cases and results for rank 4+4 from our side.

Table 3.1-1 PDSCH simulation results (baseline)

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 3 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | 13.9 | N/A | INF |

Table 3.1-1 PDSCH simulation results (simplified)

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 3 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | N/A | N/A | N/A |

It can be observed that the baseline receiver works fine for canceling the interference even with higher rank, while the simplified receiver not.

Observation 2: For the case with rank 4+4 plus baseline receiver, the operating SNR for IRC receiver is suitable for requirements, and the gain over MMSE receiver is infinity so that it is quite easier to differentiate when testing

Observation 3: For the case with rank 4+4 plus simplified receiver, the maximum Tput can not be reached by both IRC and MMSE receivers

From the simulation results above, we can conclude that simplified receiver does not work well under a rank 4+4 MU-MIMO transmission, the reason might be that the interference model is too strict for rank 4+4 (should be less severe than rank 2+2 case), and the channel model is not realistic (should consider SCM). It is reasonable to suspect the feasibility of good performance for rank 4+4 with simplified receiver.

From the requirement perspective, it is valuable to evaluate and has a knowledge of rank 4+4 performance captured in the specification during the transmission of a real Network, there is no way that the Network knows about the UE receiver because there is no such UE capability  and those tests with advanced receivers e.g. IRC, R-ML are all based on declaration so no signaling will be sent to help the Network. Thus, Network’s innocence of the receiver type is not the bottle neck of introducing PDSCH requirement for rank 4+4.

Given the facts above, we are of the opinion that it’s beneficial to have this requirement.

Observation 4: Network’s innocence of the receiver type is not the bottle neck of introducing PDSCH requirement for rank 4+4.

Considering the possibility of downlink transmission with paired rank 4+4, we are open to discuss the necessity of introducing the corresponding PDSCH requirement.

# 4	Applicability rule and release independent

Test applicability to skip 2/4Rx requirements

Issue 1-5-1: Test applicability to skip 2/4Rx requirements Proposals: - Option 1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. TBD: Clarification of what “corresponding 8Rx requirements” covers.

For UEs supporting both 8Rx and 2/4Rx, in Rel-18, we have already introduced test applicability rules to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed as shown below. We prefer to apply the same approach for the upcoming PDSCH requirements with 8Rx plus IRC.

Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list. UE supports 8Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2.

According to the agreed test applicability above, if a UE (supports 8Rx, 4Rx and 2Rx or both 8Rx and 4Rx) passes tests 8Rx requirements, then the following cases will be skipped.

Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4:

Table 5.2.3.1.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x4, ULA Low | 70 | 13.5 |
| 2-2 | R.PDSCH.2-1.1 FDD | 20 / 30 | 64QAM, 0.50 | TDLA30-10 | 2x4, ULA Low | 70 | 13.7 |

Table 5.2.3.2.1-4: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.2-3.1 TDD | 40 / 30 | 64QAM, 0.50 | FR1.30-1 | TDLA30-10 | 2x4, ULA Low | 70 | 13.6 |
| 2-2 | R.PDSCH.2-9.1 TDD | 20 / 30 | 64QAM, 0.50 | FR1.30-4 | TDLA30-10 | 2x4, ULA Low | 70 | 13.7 |

Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6:

Table 5.2.3.1.1-6: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.1-2.4 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | 15.6 |
| 4-2 | R.PDSCH.1-12.2 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | 15.2 |

Table 5.2.3.2.1-6: Minimum performance for Rank 4

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.2-2.4 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | 15.4 |
| 4-2 | R.PDSCH.2-26.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | 15.5 |

Note that it is not every 4Rx cases will be passed. Following the same approach, it is acceptable for us to define an applicability rule that only skip ‘corresponding’ tests but keep other tests.

Observation 5: Test applicability rule have been defined in Rel-18 to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed

Proposal 4: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Release independent

There are three options left for further discussion:

Issue 1-5-3: Release independency Proposals: - Option 1: Discuss release independence requirements at a later stage in the WI. - Option 2: The new Rel-19 requirements for 8Rx should be release independent from Rel-17. - Option 3: The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17. - Option 4: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18. - Option 5: The requirement should at least be released independent from Rel-18 if no new signalling or UE capability introduced.

Since the 8Rx UE feature is introduced in Rel-18, we are of the opinion that the upcoming PDSCH and CQI requirements for both inter-cell interference and intra-cell interference scenarios should be release independent from Rel-18.

Proposal 5: The new requirements for 8Rx should be release independent from Rel-18.

# 5	Summary

In this contribution, we shared our views and results on left open issues for introducing PDSCH and CQI reporting requirement. Our proposals are summarized as follows:

Proposal 1: Confirm to define PDSCH requirements with MCS17+rank 2 and MCS13+rank 4

Observation 1: Both rank 1 and 2 are feasible for the requirement definition

Proposal 2: Consider rank 2 for CQI reporting requirement definition under ICI scenario

Proposal 3: RAN4 to only define one CQI reporting requirement applying to both receiver types

Observation 2: For the case with rank 4+4 plus baseline receiver, the operating SNR for IRC receiver is suitable for requirements, and the gain over MMSE receiver is infinity so that it is quite easier to differentiate when testing

Observation 3: For the case with rank 4+4 plus simplified receiver, the maximum Tput can not be reached by both IRC and MMSE receivers

Observation 4: Network’s innocence of the receiver type is not the bottle neck of introducing PDSCH requirement for rank 4+4

Observation 5: Test applicability rule have been defined in Rel-18 to skip the 2/4Rx requirements if the corresponding 8Rx requirements are passed

Proposal 4: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

Proposal 5: The new requirements for 8Rx should be release independent from Rel-18

# References

[1]	R4-2502275: Way forward for [114][324] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #114, Athens, Greece

[2]	R4-2504328: Simulation results for MMSE-IRC receiver for 8Rx reception, RAN4 #114bis, Wuhan, China