3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504330

# Wuhan, China, 7th - 11th April, 2025

Title:	Potential impacts on spec and TR from HPUE

Source:	Samsung

Agenda item:	7.10.1

Document for:	Approval

1	Introduction

In this paper we will go through the impacted TR/TS listed in the WID, and disuss the potential impacts on each of them.

2	TS 38.101-5

After discussing several meetings, the impact on TS 38.101-5 is quite clear, and we listed it as follows:

| TS 38.101-5 | Notes |
| --- | --- |
| 6.2 Transmitter power |  |
| 6.2.1 UE maximum output power | Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN Non-Redcap HPUE |
| 6.2.2 MPR | Impacts pending on agreements. |
| 6.2.3 A-MPR | Impacts pending on agreements. |
| 6.2.4 Configured transmitted power | May have impact for SAR. |
| 6.5 Output RF spectrum emissions |  |
| 6.5.2.2 Spectrum emission mask | Apply SEM from NR-NTN PC3 to other PCs for NR-NTN as baseline. |
| 6.5.2.4.1 NR ACLR | Agreement: n PC2 ACLR is [30 or 31] dBc for NR-NTN UE Agreement: n PC1.5 ACLR is [30 or 31] dBc for NR-NTN UE Agreement: n PC1 ACLR is [31 or 37] dBc for NR-NTN UE • Further study averaging value, i.e. 34. |
| 6.5.3.3 Additional spurious emissions | Impacts pending on agreements. |
| 7.3 Reference sensitivity |  |
| 7.3.2 Reference sensitivity | For PC2 NR NTN UE, the REFSENS test can adopt the same UL configuration i.e. Table 7.3.2-2 in TS 38.101-5. For PC2 on n256: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx. For PC2 on n254: 0dB RSD for PC2 NR-NTN for 1Tx and 2Tx. |

Proposal 1: To review and comment on the potential impacts to TS 38.101-5 as provided in above table.

3	TS 36.102

Similarly, the table below contains an overview of the impacted sections for TS 36.102:

| IoT-NTN UE Tx requirement - TS 36.102 | Notes |
| --- | --- |
| 6.2A Transmitter power for category M1 |  |
| 6.2A.1 UE maximum output power |  |
| 6.2A.2 MPR |  |
| 6.2A.3 A-MPR |  |
| 6.2A.4 Configured transmitted power |  |
| 6.2B Transmitter power for category NB1 and NB2 |  |
| 6.2B.1 UE maximum output power |  |
| 6.2B.2 MPR |  |
| 6.2B.3 A-MPR |  |
| 6.2B.4 Configured transmitted power |  |
| 6.5A Output RF spectrum emissions for category M1 |  |
| 6.5A.2.2 Spectrum emission mask | Reusing SEM requirements defined for PC3 for eMTC based Iot-NTN |
| 6.5A.2.4.1 NR ACLR |  |
| 6.5A.3.3 Additional spurious emissions |  |
| 6.5B Output RF spectrum emissions for category NB1 and NB2 |  |
| 6.5B.2.2 Spectrum emission mask | Reusing SEM requirements defined for PC3 for NB-Iot based Iot-NTN |
| 6.5B.2.4.1 NR ACLR | Remove GSM_ACLR for NB-IoT based NTN HPUE. Re-use current PC3 ACLR (UTRA_ACLR = 37dBc) for PC1/2. |
| 6.5B.3.3 Additional spurious emissions |  |
| IoT-NTN UE Rx requirement - TS 36.102 | Notes |
| 7.3A Reference sensitivity for category M1 |  |
| 7.3A.2 Reference sensitivity |  |
| 7.3B Reference sensitivity for category NB1 and NB2 |  |
| 7.3B.2 Reference sensitivity |  |

Proposal 2: To review and comment on the potential impacts to TS 36.102 as provided in above table.

4	TR 38.863

For TR 38.863, besides the new section of 6b we suggested in one of our separate tdoc, the following sections will be impacted to introduce the RF requirement analysis by introducing HPUE.

| TR 38.863 | Notes |
| --- | --- |
| (new) 6b Co-existence study |  |
| 6b.2 Co-existence simulation assumption | To update with NTN HPUE co-ex assumptions based on previous agreements. |
| 6b.4 Co-existence simulation results | To amend NTN HPUE results from collected table. |
| 6b.5 Summary of co-existence study | To amend NTN HPUE conclusions based on previous agreement. |
| 7.4 NTN UE requirements |  |
| 7.4.2 UE Tx requirements | To update the TR accordingly with TS 38.101-5 agreements 7.4.2.2.1 MOP 7.4.2.2.2 MPR/AMPR 7.4.2.2.6 SEM 7.4.2.2.7 ACLR 7.4.2.2.8 Spurious emissions |
| 7.4.3 UE Rx requirements | To update the TR accordingly with TS 38.101-5 agreements 7.4.3.2.2 Refsens |

Proposal 3: To review and comment on the potential impacts to TR 38.863 as provided in above table.

5	TR 38.741, TR 36.764

The TR 38.741 and TR 36.764 can accomodate the specific analysis over L/S NTN bands for NR-NTN and IoT-NTN respectively. Based on previous agreements, these impacts are contribution driven, and not mandate by the WID objectives.

| NR L/S band TR 38.741 | Notes |
| --- | --- |
| 6.2 UE requirements |  |
| 6.2.1 UE Tx characteristics | To update the TR accordingly with TS 38.101-5 agreements 6.2.1.1 MOP 6.2.1.2 MPR/AMPR ... etc |
| 6.2.2 UE Rx characteristics | To update the TR accordingly with TS 38.101-5 agreements 6.2.2.1 Refsens |
| IoT-NTN L/S band TR 36.764 | Notes |
| 6.2 UE requirements for b254 |  |
| 6.2.1 UE Tx characteristics | To update the TR accordingly with TS 36.102 agreements 6.2.1.1 MOP 6.2.1.2 MPR/AMPR ... etc |
| 6.2.2 UE Rx characteristics | To update the TR accordingly with TS 36.102 agreements 6.2.2.1 Refsens |
| 7.2 UE requirements for b253 |  |
| 7.2.1 UE Tx characteristics | To update the TR accordingly with TS 36.102 agreements 7.2.1.1 MOP 7.2.1.2 MPR/AMPR ... etc |
| 7.2.2 UE Rx characteristics | To update the TR accordingly with TS 36.102 agreements 7.2.2.1 Refsens |

Proposal 4: To review and comment on the potential impacts to TR 38.741 and TR 36.764 as provided in above table.

6	Conclusion

For NTN HPUE Co-ex study aspects, we propose the following:

Proposal 1: To review and comment on the potential impacts to TS 38.101-5 as provided in above table.

Proposal 2: To review and comment on the potential impacts to TS 36.102 as provided in above table.

Proposal 3: To review and comment on the potential impacts to TR 38.863 as provided in above table.

Proposal 4: To review and comment on the potential impacts to TR 38.741 and TR 36.764 as provided in above table.

7	Reference

[1] R4-2502954, WF on HPUE requirements for NR-NTN, Samsung