3GPP TSG-RAN WG4 Meeting # 114-bis	R4-2504334

# Wuhan, China, 7th - 11th April, 2025

Title:	Discussion on NR-NTN HPUE Rx requirements

Source:	Samsung

Agenda item:	7.10.2.2.2

Document for:	Approval

1	Introduction

In last #114 meeting, the WF[1] had progressed on down scope the HPUE architectures and UE types for different power classes.

Issue 2-1: NR-NTN PC2 Agreement: - Define the requirements for NR-NTN PC2 covering the following cases and try to define the common requirements as much as possible - 1Tx handheld - 2Tx handheld - 1Tx non-handheld - 2Tx non-handheld - If the difficulty is identified to define the requirements for the cases above, discuss how to down-select the cases in Rel-19 Issue 2-3: NR-NTN PC1.5 Agreement: - Define the requirements for PC1.5 for non-handheld NR-NTN UE with 2Tx in Rel-19. - FFS on other cases if time is allowed Issue 2-3: NR-NTN PC1 Agreement: - RAN4 to consider non-handheld UE with 1Tx and [4Tx] in Rel-19.

And also, the WF had reached agreements for n256 and n254 RSD in PC2.

In this paper, we will continue to discuss the remaining open issues for Rx requirements of NR-NTN HPUE.

2	RSD

The WF[1] had agreed that for PC1 and PC1.5, this release will focus on non-handheld NR-NTN UE.

For PC1.5, the current options are:

Issue 3-2: RSD requirements for PC1.5 - Proposals - Option 1: For PC1.5, 2Tx RSD are the same for handheld and non-handheld, while 1Tx RSD is likely to consider non-handheld only if defined. (vivo) - Option 2: Regarding NR NTN HPUE operated in PC1.5, discuss whether the RX RSD values as shown in tables below could be used for bands n256, n255 and n254. (MediaTek) Table 4 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE not supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [1.8]; [1.8]; [1.8]; [1.8]. n255; [2.4]; [2.4]; [2.5]; [2.5]. n254; [1.8]; [1.8]; [1.8]. Table 5 RSD from PC3 to PC1.5 for FDD bands for NR NTN UE supporting Tx Diversity Operating Band; 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB). n256; [2.2]; [2.2]; [2.2]; [2.2]. n255; [3.2]; [3.2]; [3.3]; [3.3]. n254; [2.2]; [2.2]; [2.2]. - Option 3: For the NR-NTN, as the starting point consider not to specify RSD requirements for PC1.5 and PC1. (Ericsson)

And for FWA and vehicle-mounted devices, 3GPP TR 37.880[2] had already studied and concluded that the RSD is not required. I quote the relevant content below for reference:

6.3 UE self-dense Only receiver requirement that has been defined for PC1 UE is the exception due to the high-power leakage or blocking issue for band 20, see Table 7.3.1-1b below. From [12] we can learn that REFSENS exception due to PC1 operation relates IQ-image and carrier leakage components distorting on the PA and 5th order IM products hit on own Rx band. In PC3 operation this is handled with NS-10 and with special UL allocation during REFSENS test which avoids MSD. … 8.3 Receiver requirements Apart from possible REFSENS exception which is discussed in clause 6.3 no receiver requirements are needed for PC1.

Hence, we propose the meeting to consider to not define RSD requirements for PC1.5 and PC1 given non-handheld UE is considered in this release.

Proposal 1: To not define RSD requirements for PC1.5 and PC1 given only non-handheld UE is to be defined in this release.

3	Conclusion

For NR-NTN HPUE Rx requirements, we propose the following:

Proposal 1: To not define RSD requirements for PC1.5 and PC1 given only non-handheld UE is to be defined in this release.

4	Reference

[1] R4-2502954, WF on HPUE requirements for NR-NTN, Samsung

[2] TR 37.880