# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504339

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Views on general aspects for SBFD

Source:	Qualcomm Incorporated

Agenda item:	7.23.1

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#114, a WF was agreed in [2] entailing several aspects regarding SBFD general system design and deployment. In this contribution we share our views on the general aspects for Rel-19 SBFD WI within RAN4.

# General aspects

## SBFD applicability for NR TDD bands

In [2], the following was agreed regarding FR2-1 channel bandwidth and SCS support for SBFD

- In FR2-1 bands, the BS channel bandwidth for SBFD operation shall be no less than 100MHz. - RAN4 to further study 100MHz for DUD case to identify if any issue.

Recall that for FR2-1, the sync raster step size is 17.28 MHz while the SSB bandwidth for SCS=240 KHz is 28.8 MHz. In the case that SSBs are allowed to be transmitted in SBFD slots, for DU/UD SBFD configuration one DL subband bandwidth would need to be larger than 46.08 MHz. Thus, with the agreement that the minimum BS channel bandwidth for SBFD operation is 100MHz, no issues are observed. For the DUD configuration, SSBs has to be sent contiguously within only a single DL subband, which has to be larger than 46.08 MHz. From RAN1 perspective, the following was agreed in RAN1#118bis:

RAN1 Working Assumption(#118bis) For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

Thus, from RAN1 perspective, SBFD symbols containing the SSB symbols are still considered as SBFD symbols. Additionally, since the UL subband size has to follow one of the defined bandwidth configurations given in Table 5.3.2-2 in TS 38.104, and given that RAN4 has agreed that DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB, deploying DUD configuration with 100MHz channel bandwidth for FR2-1 will not be possible, since the only way to enable this by having DL subband 1 and 2 of different sizes.

Observation 1: Given that UL subband size needs to be at least 50 MHz and following RAN4 agreement that that DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB, DUD configuration for FR2-1 with 100MHz channel bandwidth would not be feasible.

Proposal 1: In FR2-1 bands, the BS channel bandwidth for DUD configuration shall be no less than 200MHz.

## Declaration and flexibility for SBFD operation

In [2], the following was agreed regarding how to indicate which bands could support SBFD deployment

- The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS

In our view, it is not possible to impose such a restriction on field operation of a given SBFD configuration. Typically, an infra-vendor will declare which SBFD configuration its product support and the testing spec will provide a number of configurations to be tested. However, from operation in the field, it is not common that 3GPP specs would enforce that operation of other-than-declared configurations shall not be operated. Such an enforcement comes typically from the regulatory bodies and the regulatory specs in different regions (e.g., Harmonized standards in Europe).

Observation 2: Restrictions on in-field operation of certain configurations is not within the scope of RAN4 specs, but could be captured via regional regulations (e.g., Harmonized Standards in Europe).

With focus on the SBFD DL/UL subband configuration, RAN4 had extensive discussions on specifying certain configurations, specifically for wide area BSs in Rel-19 work item. To progress on this, we understand the rational behind such proposals for wide area BSs. However, for medium range and local area BSs, we believe that such restriction is not needed. To recall from the study item phase, TR 38.858 Section 9.3.4 summarizes the conclusion of the feasibility of medium range BS as follows:

Based on RAN4 feasibility study on FR1 medium range BS, specifically the analysis on self-interference, co-site inter-sector co-channel inter-subband interference and inter-site inter-subband interference, RAN4 concluded that: - For self-interference analysis, the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects mentioned in clause 9.3.1. Based on the different assumptions and/or technique adoption for the above-mentioned implementation aspects, based on 5 companies’ technical inputs, companies have come to the following conclusions: - 1dB receiver desensitization target is achievable according to 4 companies. - 1dB receiver desensitization target is not achievable for a base station that strictly meets the RAN4 minimum requirements and/or operates with the maximum BS transmit power according to 2 companies. - Note: Among 5 companies, 1 company shows different feasibility results based on different assumptions. - Sectorized deployments of medium range BS are by no means universal. Dependent on the deployment needs, single sector MR BS are often deployed at least in some locations. In case a three sector deployment is used, for co-site inter-sector co-channel inter-subband interference, it can be observed that the implementation feasibility of controlling the co-site inter-sector co-channel inter-subband interference to meet the target (i.e., being less than certain level of receiver desensitization) depends on the implementation aspects mentioned in clause 9.3.2. Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 3 companies’ technical inputs companies concluded that: - The total interference caused by two co-site sectors would be above the noise floor, causing a desensitization of the receiver, according to 2 companies. - The residual co-site inter-sector interference can be controlled, causing reasonable residual self-interference, causing up to 1 dB desensitization of the receiver, according to 1 company.

As we can observe, majority of companies during the study item showed the feasibility of SBFD in terms of self-interference handling to meet the 1dB Rx desense. In addition, RAN4 requirements represent minimum requirements, so, actual performance is expected to be better that the minimum requirements. One of the challenges of not being able to converge on a one view in RAN4 regarding the medium range feasibility during the study item is the dependency on the underlying implementation aspects. It is important to recall that requirements should be developed in a neutral manner without having a single/ specific architecture/ implementation solution in mind. Thus, the proposed WF that was captured in RAN4#113 that each RF front end operational configuration from the supported SBFD configurations shall be tested is not realistic as RAN4 was not able to agree on a benchmark architecture during the study item phase, due to the implementation differences.

For local area BSs, it was concluded in TR 38.858, Section 9.4.3 that from implementation point of view, it is feasible to have SBFD-capable local area BS. Similar conclusion was reached for FR2 BSs, for wide area, medium range, and local area classes. Accordingly, the restriction that the SBFD capable BS product shall not operate any SBFD configuration other than those that have been declared and tested should not be applicable to medium or local area base station classes.

Observation 3: It is challenging for RAN4 to agree on a reference RF solution for interference handling in SBFD-capable BS to be used to test SBFD configurations for medium range base stations.

Observation 4: In TR 38.858, no challenges were reported from implementation point of view on the feasibility of SBFD-capable local area BS.

Proposal 2: If it is feasible to capture RAN4 specs, RAN4 to consider the below only for wide area base stations

- The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared and tested.

## Dl/UL subband configuration support for SBFD

In [2], the following was agreed following RAN4#113 agreements

- For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

Regarding the definition of the UL/DL subband configurations for RF requirements, it is viable to consider a configuration-agnostic approach, such as the RF requirements are defined independent of the subband configuration. The supported configuration will impact the conformance work, which is one of the main driving factors of the above discussion. On one extreme, leaving the DL subband, UL subband, and guardband fully flexible might impose additional testing challenges, as conveyed by some companies within RAN4. On the other hand, specifying only one configuration will limit the feature deployment as operators will be restricted to deploy such a configuration.  Regarding the two alternatives above on top of the agreed option 2b from RAN4#112, it is important to ensure within RAN4 that the above discussion should be restricted to WA FR-1 SBFD-capable gNB. For other base station classes and FR2-1 gNB, from such a restriction might not be needed, since there might be more flexibility in term of switching between different SBFD configurations as different (and more flexible) interference cancellation techniques are feasible, in contrast, to less flexible cancellation for WA BS.

Proposal 3: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation.

# Conclusion

In this contribution, we provided our views on general aspects for SBFD operation with the following observations and proposals:

Observation 1: Given that UL subband size needs to be at least 50 MHz and following RAN4 agreement that that DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB, DUD configuration for FR2-1 with 100MHz channel bandwidth would not be feasible.

Proposal 1: In FR2-1 bands, the BS channel bandwidth for DUD configuration shall be no less than 200MHz.

Observation 2: Restrictions on in-field operation of certain configurations is not within the scope of RAN4 specs, but could be captured via regional regulations (e.g., Harmonized Standards in Europe).

Observation 3: It is challenging for RAN4 to agree on a reference RF solution for interference handling in SBFD-capable BS to be used to test SBFD configurations for medium range base stations.

Observation 4: In TR 38.858, no challenges were reported from implementation point of view on the feasibility of SBFD-capable local area BS.

Proposal 2: If it is feasible to capture RAN4 specs, RAN4 to consider the below only for wide area base stations

- The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared and tested.

Proposal 3: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2502310, “Way Forward for [113][306] NR_duplex_evo_General”, RAN4#114, Samsung.