# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504340

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.23.2.2

Source: 	Qualcomm Incorporated

Title: 	Views on existing BS RF requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#114, a WF was agreed in [2] based on some of the topics listed in the moderator summary, that is summarized in [3]. In this contribution, we share our views on existing BS RF requirements for Rel-19 SBFD.

# Discussion

## In-band blocking and additional emission

In [2], the following was agreed:

- For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS.

For FR2-1 WA SBFD case, it is important to recall the adjacent channel coexistence work that was carried out in Rel-18 study item and summarized in TR 38.858. In particular the following case is of relevance to the discussion, where the victim was SBFD UL and aggressor was TDD DL. Based on the adjacent channel coexistence, it was concluded that under baseline assumptions, no degradation on SBFD UL was observed for both cell edge and average performance.

Observation 1: For FR2-1, based on conclusions in TR 38.858, for TDD DL interfering on SBFD UL, no degradation was observed for cell edge and average user under baseline assumptions.

Moreover, following the same framework to determine the in-band blocking value for FR2-1 WA BS, we have run adjacent channel coexistence simulations considering TDD DL interfering on SBFD UL assuming baseline assumptions, following that of TR 38.858. The metric of interest is the CDF of the absolute power level of interference from adjacent channel and particularly focusing on the 99% percentile point of the curve. Based on our findings, it was observed that the 99% assuming 50% grid shift is around -65 dBm. Note that the OTA in-band blocking requirement given in Table 10.5.2.3-1 in TS 38.104, will result in interfering signal mean power equal to -62 dBm assuming 200MHz channel bandwidth. Thus, it is a viable way forward to reuse legacy FR2-1 WA BS in-band blocking requirements for the SBFD-capable gNB.

Observation 2: Based on baseline simulation assumptions captured in TR 38.858 and 50% grid shift, the 99% CDF of the total received interference power for FR2-1 WA is observed to be -65 dBm.

Proposal 1: Based on our simulation findings, RAN4 to reuse legacy FR2-1 WA BS in-band blocking requirement given in Section 10.5.2.3 in TS 38.104.

Regarding the in-band blocking requirements for FR1 WA BS, we believe that SBFD is a key feature for future deployments, especially in new TDD bands, thus, SBFD-SBFD coexistence should be considered and thus, RAN4 has to define general requirements for SBFD-TDD adjacent operator scenario, based on ongoing discussions while also defining an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. The latter is important to ensure that Rel-19 work is robust for SBFD deployments in the future. 

Proposal 2: RAN4 to adopt option 2 from RAN4#114 WF, where RAN4 to define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

Regarding SBFD-TDD in-band blocking requirements, in our previous contribution (R4-2411514), we have provided our in-band blocking requirement for 50% GS around -26 dB, considering AAS configuration considering two panels with 46 dBm conducted power per polarization. Regarding discussion on Grid shifts, we propose to define the new requirement considering 50% grid shift since it presents a compromise between the two extremes being discussed currently in RAN4. With such assumptions, our simulation results showed that -26 dBm would be a reasonable new in-band requirement for SBFD-capable gNB.

Proposal 3: RAN4 to consider grid shift values between 20% and 50% when defining the in-band blocking requirements for SBFD-TDD coexistence case.

## ACS

In [2] the following was agreed

- FFS on the following proposals. - Proposal 1: Prioritize discussing the scenarios under non-colocation case first. - Proposal 2: The same definition of ACS could be reused for SBFD, and the interference level of [9] dB offset should be further checked. - Proposal 3: The definition of ACS is reused for SBFD. The interference level keeps 9 dB offset to the general in-band blocking interference level defined for SBFD. - Proposal 4: It is needed to determine whether interference signal level of BS to BS can be defined as ACS interference signal level. - Proposal 5: The requirement level of ACS should be derived from the BS-BS co-existence simulation results obtained in SI, instead of keeping 9 dB offset to the general in-band blocking interference level defined for SBFD. - Proposal 6: Derive ACS requirement level based on the simulation results using 5% GS in SI. - Proposal 7: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

In contrary to the ACLR discussion in Rel-19, no need to distinguish between SBFD gNB coexisting with legacy TDD or with SBFD in the adjacent channel, when considering the ACS requirement. From coexistence point of view, case 3 should be considered as a good reference to understand the impact of the adjacent channel interference while considering the legacy ACLR/ACS. Based on the outcome of the coexistence results, we can consider case 3 (i.e., Case 3: aggressor NR TDD DL victim SBFD DU), as a starting point to understand the target ACS, with starting with 9 dB offset relative to the in-band blocking requirement. Note that from ACIR point of view, ACIR enhancement will be limited by legacy TDD BS ACLR. Nevertheless, the same definition of ACS should be used for SBFD.

Proposal 4: RAN4 to adopt the same definition of ACS for SBFD. Case 3 in TR 38.858 could be adopted to investigate the feasibility of 9 dB offset relative to the in-band blocking requirement.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: For FR2-1, based on conclusions in TR 38.858, for TDD DL interfering on SBFD UL, no degradation was observed for cell edge and average user under baseline assumptions.

Observation 2: Based on baseline simulation assumptions captured in TR 38.858 and 50% grid shift, the 99% CDF of the total received interference power for FR2-1 WA is observed to be -65 dBm.

Proposal 1: Based on our simulation findings, RAN4 to reuse legacy FR2-1 WA BS in-band blocking requirement given in Section 10.5.2.3 in TS 38.104.

Proposal 2: RAN4 to adopt option 2 from RAN4#114 WF, where RAN4 to define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario.

Proposal 3: RAN4 to consider grid shift values between 20% and 50% when defining the in-band blocking requirements for SBFD-TDD coexistence case.

Proposal 4: RAN4 to adopt the same definition of ACS for SBFD. Case 3 in TR 38.858 could be adopted to investigate the feasibility of 9 dB offset relative to the in-band blocking requirement.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2502380, “Way Forward for [114][307] NR_duplex_evo_General”, RAN4#114, Huawei, HiSilicon.

- R4-2500568, “Topic summary for [114][307] NR_duplex_evo_BSRF”, RAN4#114, Huawei, HiSilicon.