# 3GPP TSG-RAN WG4 Meeting #114bis		R4-2504342

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Views on BS demodulation requirements for MMSE-IRC

Source:	Qualcomm Incorporated

Agenda item:	7.18.3

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on BS advanced receiver performance was agreed in [1]. One of the objectives of the WI is as follows:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In [2], a WF was agreed regarding simulation parameters, scenarios and  that are the basis for the demodulation requirements to be defined in this work item. In this contribution, we provide our views on the open items captured in [2].

# 2 	Discussion

## 2.1	Number of interferers for 2Rx

In [2], below options were agreed:

- For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx. - For the number of interferers for 2Rx under HetNet, further decide between the following: - Option 1: 2 interferers - Option 2: 1 interferer - For HomNet scenario, further decide among the following: - Option 1: 2 interferers - Option 2: 1 interferer - Option 3: Not to consider 2Rx requirements for HomNet scenario.

During RAN4#113, several companies have submitted simulation results showing gains of MMSE-IRC over MMSE receiver for both 1 and 2 interferers. Such results are expected, since it was already captured in [3], § 8.1, that for the 2Rx cases, that both 1 and 2 interferers provide gains, however, considering 2 interferers did not show additional gain when compared to the single interferer case. Thus, it was agreed to only consider a single interferer case for the 2Rx scenario. Similar observations were captured in other RAN4#113 initial simulation results (e.g., R4-2417782).

Observation 1: Since the interference profile follows what was considered in TR 36.884, it is expected that gains trends between 1 and 2 interferers for 2Rx will be applicable.

In our views, having 2Rx requirements (whether based on 1 or 2 interferers) could be motivated to provide consistency with legacy PUSCH requirements, however, it is not clear if it reflects practical deployments. Nevertheless, to ensure consistency with LTE MMSE-IRC requirements defined in TS 36.104 and with existing NR BS demodulation requirements, it is proposed to consider HomNet with 1 interferer, given that

Proposal 1: Since the gains of MMSE-IRC over MMSE receiver for both 1 and 2 interferers are similar, RAN4 to consider 1Rx interferer for the 2Rx case.

## 2.2	Test parameters and simulation assumptions

### 2.2.1	Propagation condition for the serving cell

In [2], below options were agreed:

Select between the following options: - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet - Option 2: TDLC300-100 for all cases - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Proposed during RAN4#114 ad-hoc session as a compromise between option 1 and option 2)

There are mainly two options were discussed during RAN4#114, either follow LTE work on MMSE-IRC [3], where two different propagation models were agreed to define the PUSCH demodulation requirements (see Table 8.2.6.1-4 in TS 36.104). The rationale behind this is that for heterogenous scenario, smaller cell size is assumed and thus, lower UE mobility and low Doppler configurations are prioritized. On the other hand, for the homogenous scenario, larger cell size is considered where higher UE mobility and higher Doppler configurations are existent. Accordingly, PUSCH demodulation requirements with MMSE-IRC need to reflect such discrepancies. The other option is related to the MCS, since in NR, the propagation condition is selected following the agreed MCS (see Table 8.2.2.2-1 in TS 38.104). Since in RAN4#114, as captured in [2], MCS was agreed for 2Rx, 4Rx, and 8 Rx, TDLC300-100 would be the propagation channel for both heterogeneous and homogeneous scenarios.

Observation 2: TDLA30-10 for HetNet and TDLB100-400 for HomNet follows LTE assumptions where different propagation conditions were defined for heterogeneous and homogeneous scenarios, while TDLC300-100 follows NR propagation condition for the agreed MCS 16.

Given that RAN4 has agreed in RAN4#113 to reuse FRCs defined in TS 38.104, option 2 is the straightforward option to ensure consistency of FRCs considered. With options 1 and 3, new FRCs might need to be introduced. Accordingly, it is proposed to consider option 2 as a first priority within RAN4. If not agreeable, then option 3 will be a good compromise with RAN4 to further check what are the additional FRCs that might need to be introduced.

Proposal 2: RAN4 to adopt option 2 (TDLC300-100 for all cases) as a first priority and if not agreeable, consider option 3 (TDLA30-10 for HetNet and TDLC300-100 for HomNet) with further discussions on the FRCs impact.

## 2.3	Manufacturer declarations and applicability rules

### 2.3.1	Manufacturer declaration on MMSE-IRC receiver

In [2], below options were agreed:

- Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. D.xxx; MMSE-IRC receiver; Declaration of BS supporting MMSE-IRC receiver.; x; x. - Proposal 2: Do not need to introduce a new Manufacturer Declaration on MMSE-IRC

In RAN4#114, there were discussions on ensuring that MMSE-IRC Receiver would be a Rel-19 mandatory feature or not. It is important to recall how the MMSE-IRC feature was handled at the UE side. In TS 38.307, MMSE-IRC was applicable from Rel-17 and optional for both Rel-16 and Rel-15, as shown below.

Figure 1 Snippet from TS 38.307 version 18.2.0 Release 18

Considering also several comments that were made in RAN4#114 regarding the wide adoption and deployment of MMSE-IRC receivers, it is a viable option to consider MMSE-IRC as a Rel-19 mandatory feature. In case RAN4 did not agree on introducing the feature as mandatory, optional declaration needs to be introduced to align with how MMSE-IRC requirements were introduced in LTE (See Section 8.2.6.1 in TS 36.141).

Observation 3: For the UE, MMSE-IRC is applicable from Rel-17 and optional for both Rel-16 and Rel-15, as captured in Table 5.4-1 in TS 38.307.

Proposal 3: RAN4 to consider MMSE-IRC as a Release-19 mandatory feature to be aligned with how MMSE-IRC requirements were considered for the UE. If not agreeable, RAN4 to introduce a manufacture declaration to allow optional deployment of the feature following how it was introduced in LTE specs.

### 2.3.2	Applicability rule for different test scenarios

In [2], below options were agreed:

- Candidate options on test requirement applicability for different BS classes and test scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS - Other options are not precluded.

In [3], the demodulation requirements for HomNet scenario was not appliable for Local Area BS and Home BS, as such deployment was envisioned for macro urban deployments and not small cell or indoor deployments. On the other hand, the HetNet entail mixture of wide area and local area deployments. In this regard, it is viable to follow the same approach within this work item and consider only HomNet for wide area and medium range deployments (i.e., proposal 1 above).

Proposal 4: RAN4 to confirm that HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS.

# Conclusion

In this contribution we provide our views on the remaining issues of BS demodulation aspects for MMSE-IRC receiver. Our observations and proposals can be summarized for that frequency range as follows:

Observation 1: Since the interference profile follows what was considered in TR 36.884, it is expected that gains trends between 1 and 2 interferers for 2Rx will be applicable.

Proposal 1: Since the gains of MMSE-IRC over MMSE receiver for both 1 and 2 interferers are similar, RAN4 to consider 1Rx interferer for the 2Rx case.

Observation 2: TDLA30-10 for HetNet and TDLB100-400 for HomNet follows LTE assumptions where different propagation conditions were defined for heterogeneous and homogeneous scenarios, while TDLC300-100 follows NR propagation condition for the agreed MCS 16.

Proposal 2: RAN4 to adopt option 2 (TDLC300-100 for all cases) as a first priority and if not agreeable, consider option 3 (TDLA30-10 for HetNet and TDLC300-100 for HomNet) with further discussions on the FRCs impact.

Observation 3: For the UE, MMSE-IRC is applicable from Rel-17 and optional for both Rel-16 and Rel-15, as captured in Table 5.4-1 in TS 38.307.

Proposal 3: RAN4 to consider MMSE-IRC as a Release-19 mandatory feature to be aligned with how MMSE-IRC requirements were considered for the UE. If not agreeable, RAN4 to introduce a manufacture declaration to allow optional deployment of the feature following how it was introduced in LTE specs.

Proposal 4: RAN4 to confirm that HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS.

# Reference

[1]		RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

[2] 	R4-2502272, “Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS”, RAN4#114, China Telecom.

[3]	TR 36.884, “Performance requirements of MMSE-IRC receiver for LTE BS” 3GPP V13.1.0