# 3GPP TSG-RAN WG4 Meeting #114bis		R4-2504343

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	Views on BS FR1 transmitter co-existence spurious emission requirements

Source:	Qualcomm Incorporated

Agenda item:	7.13.4

Document for:	Approval

# 1.	Introduction

In RAN4, a discussion triggered by a Liaison Statement (LS) from ETSI TC MSG/TFES [1] on the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in the upper region of FR1. This is based on the discussions within ETSI MSG/TFES on Rel-17 versions of Harmonized Standards for BS specifications, where it was observed that the coexistence requirements for Tx spurious emissions might needs further checking to ensure their relevance and applicability. Note that those limits were mainly established and derived for lower FR1 bands and then were reused for upper FR1 bands.  Foe example, -52 dBm is used for 450 MHz frequency bands, as well as for the high frequency bands above 3.5 GHz and up to 7 GHz. The reader is kindly referred to [3], for more information on the derivation of those legacy requirements.

In response, it was agreed in [2] that RAN4 Add the new objective of co-existence transmitter spurious emission limits according to the request from ETSI/TFES to the ongoing Rel-19 NR BS RF requirement evolution work item, with the following objectives.

- Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed.

In this contribution, we share our preliminary views on how to study and assess the co-existence Tx spurious limits within RAN4.

# 2 	Discussion

## 2.1	Evaluation of Tx co-existence spurious emissions

The co-existence Tx spurious emission requirements are presented in BS RF specs to ensure protection of other carriers in adjacent bands within the same geographical area. There are BS-to-UE (i.e., -52 dBm/1MHz) and BS-to-BS (i.e., -49 dBm/1MHz) requirements existing in the specs and the requirement evaluation depends on the following parameters:

- Coupling loss between Tx and Rx (BS-to-UE or BS-to-BS case)

- BS or UE noise figure

- Bandwidth (Typically requirement is given per 1MHz)

- Additional protection criteria (only applicable to BS-to-BS protection)

TR 25.942, section 5.1.4.1, entails the assumptions that were adopted to derive the above legacy requirements for lower FR1 bands. Several assumptions were considered when such levels were derived, for example, passive antennas were assumed with fixed antenna gain and the minimum coupling loss considered only a single BS type or deployment.

Observation 1: Legacy co-existence Tx spurious emissions were based on low FR1 bands deployments and assumptions. Noticeably, passive antennas were assumed with fixed antenna gain.

## 2.2	Simulation assumptions and evaluation methodology

To re-evaluate co-existence requirement levels with the intention to find relevant requirement levels a study will be required. For a successful co-existence evaluation in RAN4 relevant parameter assumption for the BS antenna and the UE antenna, Propagation model assumption (including BS-to-UE propagation, BS-to-BS propagation) and deployment scenario should be established. As a starting point, it is a viable option to evaluate the MCL following system level simulations in order to understand the statistical implications of AAS deployments, which is the tailored deployment and assumption for high FR1 bands.

Proposal 1: RAN4 to consider system level simulations to derive the new emission requirements.

It is also important to confirm what assumptions from legacy requirements could be reused, for example, reusing NF values for both BS and UE from lower FR1 bands might be a good starting point.

Proposal 2: RAN4 to reuse BS and UE NF values that were used to derive legacy requirement (i.e., BS NF = 7 dB and UE NF = 9 dB).

Proposal 3: RAN4 to reuse 0.5 dB degradation of victim receiver sensitivity to be used as a starting point.

In terms of AAS configurations, it is viable to have a split between low and high FR1 bands, where for the former, AAS and non-AAS could be both considered at the BS side, while for the latter, only AAS configurations to be studied. For the UE side, following what was agreed in TR 38.922, it is proposed to consider an omnidirectional antenna model with 0 dBi at the UE side.

Proposal 4: RAN4 to consider both AAS and non-AAS at the BS side for low FR1 bands, while consider only AAS at the BS side for high FR1 bands.

Proposal 5: RAN4 to consider no beamforming at the UE side (omnidirectional antenna model with 0 dBi).

# Conclusion

In this contribution we provide our views on  BS FR1 transmitter co-existence spurious emission requirements. Our observations and proposals can be summarized as follows:

Observation 1: Legacy co-existence Tx spurious emissions were based on low FR1 bands deployments and assumptions. Noticeably, passive antennas were assumed with fixed antenna gain.

Proposal 1: RAN4 to consider system level simulations to derive the new emission requirements.

Proposal 2: RAN4 to reuse BS and UE NF values that were used to derive legacy requirement (i.e., BS NF = 7 dB and UE NF = 9 dB).

Proposal 3: RAN4 to reuse 0.5 dB degradation of victim receiver sensitivity to be used as a starting point.

Proposal 4: RAN4 to consider both AAS and non-AAS at the BS side for low FR1 bands, while consider only AAS at the BS side for high FR1 bands.

Proposal 5: RAN4 to consider no beamforming at the UE side (omnidirectional antenna model with 0 dBi).

# Reference

[1]		RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[2]		RP-243288, “Moderator's summary of discussion about R19-RAN4-BSRF”, RAN4 Chair (Huawei), RAN #106, Madrid, Spain.

[3] 	R4-2418396, “On the topic of the need to reevaluate requirement levels for BS transmitter co-existence spurious emission for bands in upper region of FR1”, Ericsson