3GPP TSG-RAN WG4 Meeting #114bis	R4-2504344

Wuhan, Hubei, China, 7 April – 11 April 2025

Agenda Item:	7.10.4.1

Source:	Ericsson

Title:	TE-emulated channel model for NTN UE tests

Document for:	Discussion

# 1	Introduction

RAN4#114 agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1]. In the WF, RAN4 agreed with the satellite propagator model to estimate the satellite position/velocity user for Doppler shift and propagation delay calculation.

This contribution continues the discussion how to derive the time varying Doppler shift and propagation delay.

# 2	NGSO NTN test environment

Figure 1	Assumed test environment for NGSO NTN UE testing.

Figure 1 illustrates an overview of the NGSO NTN test environment. During the test, DUT (UE) estimates the Doppler shift and propagation delay based on the satellite position/velocity (ephemeris) information and UE’s location information at a given time. In the DUT, the UE location information is pre-configured or provided via AT command, as specified in TS 38.508 (See C.1). The estimated Doppler shift and propagation delay are used to perform the pre-compensation before the baseband processing in the DUT.

As agreed in RAN4#114, the TE calculates the satellite position/velocity based on the satellite propagator model, which can be based on the Kepler’s equation or equation of motion. To reduce the computational complexity, the TE may update the satellite position/velocity every Tsat (sec). However, since the TE needs to apply to the Doppler shift and propagation delay every slot to the transmit signals before the fading simulator, RAN4#114 agreed that the TE need to update the Doppler shift and propagation delay by interpolation once per 1ms. It was also confirmed that the TE does not need to synchronize the updated timing with the slot boundary of the transmitted signal. To derive the Doppler shift and propagation delay, it is assumed the TE knows the same UE location information configured to the DUT.

During the test, the TE generates the satellite ephemeris information to signal to the DUT (UE) with SIB19 ephemerisInfo-r17 for NR or SIB31 ephemerisInfo-r17 for IoT-NTN periodically, every Tsib (sec), where Tsat < Tsib. together with epochTime. RAN4 assumes the ephemeris information in SIB is derived from the propagator model used for the Doppler shift and propagation delay calculation.

Figure 2 summarizes the update timing of the satellite ephemeris information, that is, ephemerisInfo, Doppler shift and propagation delay.

Figure 2	Update timing of satellite position/velocity, epehmerisInfo, Doppler shift, and propagation delay.

| Parameters | Unit | Values |
| --- | --- | --- |
| Tsat | sec | 1.024 |
| Tsib | sec | 10.24 for RF and demodulation tests 2.56 for RRM tests |

# 3	Discussion

## 3.1	Initial ephemeris information

### 3.1.1	LEO-600 satellite scenario

RNA4 so far focused on LEO-600 satellite scenario and it was agreed to set the initial evaluation angle to 30 degrees for RRM and demodulation tests. Table 1 is the initial ephemeris information shown in [1], which is derived from TS 38.508 Table 6.4.1_2.4.3-1.

Table 1	Initial ephemeris information for LEO-600 stating from elevation angles of 30 degrees.

| Parameters | Unit | Values |
| --- | --- | --- |
| positionX-r17 | km | -3451.1273 |
| positionY-r17 | km | 5703.5635 |
| positionZ-r17 | km | 2071.0809 |
| velocityVX-r17 | km/s | 0.87360 |
| velocityVY-r17 | km/s | -2.06700 |
| velocityVZ-r17 | km/s | 7.21231 |

Figure 3	Elevation angle for LEO-600 maximum positive Doppler scenario.

Figure 3 shows the elevation angles of the satellite derived from the UE with Table 1. Note the procedure to calculate the elevation angle is shown in Appendix A.2.3. It is observed that the initial elevation angle is 30 degrees. The figure also illustrates the simulation time becomes at most 250 seconds to keep the elevation angle larger than 30 degrees.

Proposal 1: Use Table 1 as the initial ephemeris information for LEO-600 satellite scenario.

Observation 1: Simulation time is at most 250 seconds for LEO-600 satellite scenario to keep the elevation angle of satellite larger than 30 degrees.

Figure 4 shows the Doppler shift and propagation delay using LEO-600 maximum positive Doppler test parameters, for information.

Figure 4	Doppler shift and propagation delay for LEO-600 maximum positive Doppler scenario.

### 3.1.2	LEO-1200 satellite scenario

RAN4 also discussed the time-varying Doppler shift and propagation delay model for LEO-1200 satellite scenario. For LEO-1200 satellite scenario, we can reuse TS 38.508 Table 7.6.2.1-3a, whose condition is the serving cell 30 degrees elevation angle condition as described in the caption, as shown in Table 2.

Table 2	Initial ephemeris information for LEO-1200 stating from elevation angles of 30 degrees (Copy from TS 38.508 Table 7.6.2.1-3a, serving cell 30 degrees elevation angle conditions).

| Parameters | Unit | Values |
| --- | --- | --- |
| positionX-r17 | km | -3816.2176 |
| positionY-r17 | km | 6364.098 |
| positionZ-r17 | km | 1544.9954 |
| velocityVX-r17 | km/s | 0.51036 |
| velocityVY-r17 | km/s | -1.39884 |
| velocityVZ-r17 | km/s | 7.09518 |

Figure 5	Elevation angle for LEO-1200 maximum positive Doppler scenario.

Figure 5 shows the elevation angles of the satellite from the UE with Table 2. From the figure, it is observed the simulation time is about 500 seconds to keep the elevation angle larger than 30 degrees, which is double compared with LEO-600 satellite scenario as expected.

Proposal 2: If RAN4 specify the time-varying Doppler shift and propagation delay model for LEO-1200 satellite scenario, reuse TS 38.508 Table 7.6.2.1-3a as the initial ephemeris information.

Figure 6	Doppler shift and propagation delay for LEO-1200 maximum positive Doppler scenario.

Figure 6 shows the Doppler shift and propagation delay using Table 2. It is observed that the maximum Doppler shift is about 35kHz. Although the maximum Doppler shift with LEO-1200 is lower than LEO-600, the minimum propagation delay is double compared with LEO-1200.

Observation 2: Simulation time is about 500 seconds for LEO-1200 satellite scenario to keep the elevation angle of satellite larger than 30 degrees. For the LEO-1200 satellite scenario, the range of the Doppler shift is between -35kHZ to +35kHz, and the minimum propagation delay is 4.05us.

If the simulation time is not enough for LEO-600, RAN4 could consider using LEO-1200 than can ensure the simulation time up to 500 seconds.

Proposal 3: RAN4 specify the time-varying Doppler shift and propagation delay model for LEO-1200 satellite scenario if there are test cases whose test time becomes more than 250 seconds.

## 3.2	Propagation delay calculation

RAN4#114 discussed whether the earth rotation is considered for propagation delay calculation.

Issue 1-1-2 Correction of Propagation Delay Calculation WF - RAN4 discuss whether the Earth's rotation is considered during signal flight time for more accurate estimations of propagation delay

When the UE estimate the propagation delay at tR, the UE first estimates the satellite position at tR, given by rtRECEI. However, the propagation delay should be derived from the signal path between the satellite position at tT, given by rtTECEI, and UE position at tR, given by UEtRECEI, as illustrated in Figure 7 (a).

| (a) Illustration of signal path | (b) How to derive the actual distance |
| --- | --- |

Figure 7	Example of signal path from the time the satellite transmits signal at tT to the time the UE receives the signal at tR.

If the propagation delay is given by Δt, Δt is given by the difference between tR and tT, that is, Δt=tR-tT. During the propagation delay period, the satellite position changes by vtTECIΔt. When the speed of light is given by c, we can get the following equation (Figure 7 (b)).

UEtRECEF-rtTECEF=cΔt

From rtRECEF=rtTECEF+vtRECIΔt, rtTECEF is replaced by rtRECEF-vtRECIΔt, and thus,

UEtRECEF-rtRECEF-vtRECIΔt=cΔt

We should emphasize the velocity vectors, vtRECI, shall be given by inertial reference frame, and given by:

vtRECI=vtRECEF+00ωE×rtRECEF

By defining, ρtR ECEF:=rtRECEF-UEtRECEF

cΔt=UEtRECEF-rtRECEF+vtRECIΔt

c2Δt2=UEtRECEF-rtRECEF+vtRECIΔt2

cΔt2=-ρtRECEF+vtRECIΔt2=ρtRECEF-vtRECIΔt2

≈ρtRECEF2-2vtRECI⋅ρtRECEFΔt

By taking a square root of the equation,

cΔt=ρtRECEF2-2vtRECI⋅ρtRECEFΔt

=ρtRECEF1-2vtRECI⋅ρtRECEFρtRECEF2Δt

≈ρtRECEF1-vtRECI⋅ρtRECEFρtRECEF2Δt=ρtRECEF-vtRECI⋅ρtRECEFρtRECEFΔt

Finally, we get the equation below.

cΔt≈ρtRECEF-vtRECI⋅ρtRECEFρtRECEFΔt

Solve the above equation by Δt,

c+vtRECI⋅ρtRECEFρtRECEFΔt=ρtRECEF

Δt=DelayERt=ρtRECEFc+vtRECI⋅ρtRECEFρtRECEF=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF

Figure 8	Difference of propagation delays between Delay(t) and DelayER(t).

RAN4 so far assumed to use the following formula to derive the propagation delay (See Appendix A.2.2), which does not include the effect of the earth rotation.

ρtRECEF=rtRECEF-UEtRECEF

Delay(t)=ρtRECEFc

Figure 8 shows the difference between two propagation delays, that is Delayt-DelayER(t) in the case of the initial ephemeris information with Table 1. As we agreed in RAN4#114, we assume the initial evaluation angle is 30 degrees for RRM and demodulation tests. In the case of LEO-600 satellite scenario, the duration the UE receives the signal from one satellite is 250 seconds. This figure implies that the propagation delay mismatch is between -0.07us to +0.07us if the TE applies the propagation delay with Delay(t), but the UE estimate the propagation delay with DelayER(t), or vice versa.

Considering the SCS=15kHz, we don’t see the impact to demodulation requirements because the cyclic prefix is 4.7us. On the other hand, as we discussed in RAN4#114, one concern is the RRM UE UL transmit timing requirements. According to TS 38.133 Table 7.1C.2, the UE initial transmission timing error shall be less than or equal to +/-Te_NTN, and Te_NTN is 0.944us (29*64*Tc) for FR1-NTN SCS=15kHz. If the TE calculates the propagation delay with Delay(t) but UE estimates the propagation delay with DelayER(t), we cannot say the mismatch of propagation delay is entirely negligible regarding Te_NTN.

We slight prefer to consider the earth rotation to calculate the propagation delay in the TE regardless of RRM or demodulation tests, since it is more realistic.

Proposal 4: Propagation delay model should include the impact due to the earth rotation since it is more realistic.

# 4	Summary

Observations:

Observation 1: Simulation time is at most 250 seconds for LEO-600 satellite scenario to keep the elevation angle of satellite larger than 30 degrees.

Observation 2: Simulation time is about 500 seconds for LEO-1200 satellite scenario to keep the elevation angle of satellite larger than 30 degrees. For the LEO-1200 satellite scenario, the range of the Doppler shift is between -35kHZ to +35kHz, and the minimum propagation delay is 4.05us.

Proposals:

Proposal 1: Use Table 1 as the initial ephemeris information for LEO-600 satellite scenario.

Proposal 2: If RAN4 specify the time-varying Doppler shift and propagation delay model for LEO-1200 satellite scenario, reuse TS 38.508 Table 7.6.2.1-3a as the initial ephemeris information.

Proposal 3: RAN4 specify the time-varying Doppler shift and propagation delay model for LEO-1200 satellite scenario if there are test cases whose test time becomes more than 250 seconds.

Proposal 4: Propagation delay model should include the impact due to the earth rotation since it is more realistic.

# References

- R4-2502314, “Way Forward for [114][325] NTN_testing_NGSO_channel_model”, Samsung.

# Appendix

## A.1	Satellite position/velocity estimation

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| r0ECEF=r0,xECEFr0,yECEFr0,zECEF | km | Initial satellite position state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| v0ECEF=v0,xECEFv0,yECEFv0,zECEF | km/s | Initial satellite velocity state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004418 x 105 | km2/s2 |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

### A.1.1	Keplerian model based estimation

#### Step 1	Derive the orbital parameters (a, e, i, Ω, ω, M0)

##### Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0ECI=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0ECI=v0,xECIv0,yECIv0,zECI=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

##### Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0ECI=r0,xECI2+r0,yECI2+r0,zECI2

v=v0ECI=v0,xECI2+v0,yECI2+v0,zECI2

vr=v0ECI⋅r0ECIr=r0,xECIv0,xECI+r0,yECIv0,yECI+r0,zECIv0,zECIr

h=hxhyhz=r×v=r0,yECIv0,zECI-r0,zECIv0,yECIr0,zECIv0,xECI-r0,xECIv0,zECIr0,xECIv0,yECI-r0,yECIv0,xECI

h=h=hx2+hy2+hz2

##### Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

##### Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 38.331 ephemerisInfo-r17.

##### Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0ECI-r0ECI⋅v0ECIv0ECI=1μv2-μrr0,xECIr0,yECIr0,zECIT-r⋅vrv0,xECIv0,yECIv0,zECIT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

##### Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

##### Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0ECIe⋅r=cos-1exr0,xECI+eyr0,yECI+ezr0,zECIe⋅rvr≥02π-cos-1exr0,xECI+eyr0,yECI+ezr0,zECIe⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

#### Step 2	Determine the satellite position and velocity at time t (sec)

##### Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

##### Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton’s method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If Et(n+1)-Et(n)<ϵ, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n := n+1.

Note ϵ=10-6, for example.

Note n=0, 1, 2, …

##### Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

##### Step 2-4	Convert the orbital elements to the state vector in ECI frame

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

### A.1.2	Equation of Motion based estimation

##### Step 1: Initialization

n:=0

rnΔt=rnΔt,xrnΔt,yrnΔt,z=r0,xECEFr0,yECEFr0,zECEF=r0ECEF

vnΔt=vnΔt,xvnΔt,yvnΔt,v=v0,xECEFv0,yECEFv0,zECEF=v0ECEF

Set time step size of position/velocity updates. Set Δt=1 (sec), for example, if the satellite position/velocity is updated every 1 second.

##### Step 2	Solve the equation of motion with the 4th-order Runge-Kutta method.

k1,r=k1,r,xk1,r,yk1,r,z=vnΔtΔt

k1,v=k1,v,xk1,v,yk1,v,z=frnΔt,vnΔtΔt

k2,r=k2,r,xk2,r,yk2,r,z=vnΔt+12k1,vΔt

k2,v=k2,v,xk2,v,yk2,v,z=frnΔt+12k1,r,vnΔt+12k1,vΔt

k3,r=k3,r,xk3,r,yk3,r,z=vnΔt+12k2,vΔt

k3,v=k3,v,xk3,v,yk3,v,z=frnΔt+12k2,r,vnΔt+12k2,vΔt

k4,r=k4,r,xk4,r,yk4,r,z=vnΔt+k3,vΔt

k4,v=k4,v,xk4,v,yk4,v,z=frnΔt+k3,r,vnΔt+k3,vΔt

rn+1Δt=rn+1Δt,xrn+1Δt,yrn+1Δt,z=rnΔt+16k1,r+2k2,r+2k3,r+k4,r

vn+1Δt=vn+1Δt,xvn+1Δt,yvn+1Δt,z=vnΔt+16k1,v+2k2,v+2k3,v+k4,v

Where

frt,vt=frt,x,rt,y,rt,z,vt,x,vt,y,vt,z=-μrt3rt,x+2ωEvt,y+ωE2rt,x-μrt3rt,y-2ωEvt,x+ωE2rt,y-μrt3rt,zT

rt=|rt|=rt,x2+rt,y2+rt,z2

##### Step 3: Determine the satellite position/velocity at time t.

If t=n+1Δt:

rtECEI=rn+1Δt=rn+1Δt, xrn+1Δt, yrn+1Δt, z

vtECEI=vn+1Δt=vn+1Δt, xvn+1Δt, yvn+1Δt, z

Otherwise, set n≔n+1, and go to Step 1.

## A.2	Varying Doppler shift and propagation delay generation

### A.2.1	Determine UE position

This step calculates the UE position according to the earth angular speed. Since the UE location is configured with a format of latitude, longitude, and altitude, we need to convert the values to ECEF format to align with the satellite position/velocity state vectors.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.6943799014 x 10-3 |  |

ϕ=π180UElatitude

λ=π180UElongitude

N=aE1-eE2sin2ϕ

Since it is assumed that UE is stationary during the tests, UE position does not change regardless of time t.

UEt,xECEF=N+UEaltitudecosϕcosλ UEt,yECEF=N+UEaltitudecosϕsinλUEt,zECEF=N1-eE2+UEaltitudesinϕ

### A.2.2	Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vt=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| Delay | sec | Propagation delay at time t |

Constant parameters:

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 299792.458 | km/s |

ρtECEF=ρt,xECEFρt,yECEFρt,zECEF=rt,xECEF-UEt,xECEFrt,yECEF-UEt,yECEFrt,zECEF-UEt,zECEF

ρtECEF=ρt,xECEF2+ρt,yECEF2+ρt,zECEF2

RangeRate(t)=ρtECEF⋅vtECEFρtECEF=ρt,xECEF⋅vt,xECEF+ρt,yECEF⋅vt,yECEF+ρt,zECEF⋅vt,zECEFρtECEF

Delay(t)=ρtECEFc

Doppler(t)=-RangeRate(t)×fcc

If the earth rotation is considered, the propagation delay is calculated as follows:

DelayERt=ρtECEF/c1+1cvtECI⋅ρtECEFρtECEF

### A.2.3	Elevation angle and Azimuth angle

The elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t can be derived as follows:

e=-sinλcosλ0, n=-sinϕcos λ-sinϕsin λcosϕ, u=cosϕcosλcosϕsinλsinϕ

Note e, n, and u are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρtECEF⋅e=ρt,xECEF⋅ex+ρt,yECEF⋅ey+ρt,zECEF⋅ez

lt,n=ρtECEF⋅n=ρt,xECEF⋅nx+ρt,yECEF⋅ny+ρt,zECEF⋅nz

lt,u=ρtECEF⋅u=ρt,xECEF⋅ux+ρt,yECEF⋅uy+ρt,zECEF⋅uz

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

Azimuth angle, Az (degrees), is also derived as follows.

Azt=180πtan-1lt,elt,n

## B.1	SIB19 NTN Ephemeris Information for zero Doppler conditions specified in TS 38.508

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 | -3315.1053 (km) |  |
| positionY-r17 | 4151097 | 5396.4261 (km) |  |
| positionZ-r17 | 2263034 | 2941.9442 (km) |  |
| velocityVX-r17 | 21970 | 1.3182 (km/s) |  |
| velocityVY-r17 | -48164 | -2.88984 (km/s) |  |
| velocityVZ-r17 | 114109 | 6.84654 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 | -3603.0046 (km) |  |
| positionY-r17 | 4509243 | 5862.0159 (km) |  |
| positionZ-r17 | 2455572 | 3192.2436 (km) |  |
| velocityVX-r17 | 20837 | 1.25022 (km/s) |  |
| velocityVY-r17 | -46176 | -2.77056 (km/s) |  |
| velocityVZ-r17 | 109528 | 6.57168 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

Table 7.6.2.1-3a: SIB19- NR NTN serving cell Ephemeris information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2935552 | -3816.2176 |  |
| positionY-r17 | 4895460 | 6364.098 |  |
| positionZ-r17 | 1188458 | 1544.9954 |  |
| velocityVX-r17 | 8506 | 0.51036 |  |
| velocityVY-r17 | -23314 | -1.39884 |  |
| velocityVZ-r17 | 118253 | 7.09518 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in NR NTN RRM test cases listed in Table A.3.36.3-1 in TS38.133 [13]. |  |  |  |

## B.2	SIB19 NTN Ephemeris Information for zero Doppler conditions for UL frequency error test specified in TS 38.521-5

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 | -3532.9021 (km) |  |
| positionY-r17 | 4550419 | 5915.5447 (km) |  |
| positionZ-r17 | 852799 | 1108.6387 (km) |  |
| velocityVX-r17 | 6164 | 0.36984 (km/s) |  |
| velocityVY-r17 | -19424 | -1.16544 (km/s) |  |
| velocityVZ-r17 | 124281 | 7.45686 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 | -2859.0536 (km) |  |
| positionY-r17 | 3404229 | 4425.4977 (km) |  |
| positionZ-r17 | 3535794 | 4596.5322 (km) |  |
| velocityVX-r17 | 35394 | 2.12364 (km/s) |  |
| velocityVY-r17 | -74414 | -4.46484 (km/s) |  |
| velocityVZ-r17 | 94682 | 5.68092 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

## C.1	UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0