3GPP TSG-RAN WG4 Meeting #114bis	R4-2504346

Wuhan, Hubei, China, 7 April – 11 April 2025

Agenda Item:	7.19.2

Source:	Ericsson

Title:	Discussion on CSI prediction

Document for:	Discussion

# 1	Introduction

RAN4#114 agreed with the way forward on AI/ML for NR air interface [1], where the way forward captured the several agreements on CSI prediction as follows:

Issue 1-2: Reference model for CSI prediction Agreement: - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model. Issue 1-3: Training and test data consistency Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations Issue 1-4: Generalization Agreement: - Postpone this discussion until the requirement and test definition is clear

RAN#107 revised WID on AI/ML for NR air interface [2] and clarified to conder the following aspects for CSI prediction as well as beam management and positioning accuracy enhancements.

Note: the following aspects may be considered: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g. CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

This contribution discusses the performance monitoring, generalization aspects, and test conditions.

# 2	Discussion

## 2.1	Simulation results

RAN4#114 agreed with the simulation assumption to verify the companies’ CSI predictor results regarding the average of SGCS between the SVD of model output and SVD of the ground truth Raw channel [3]. Table 1 summarizes the average SGCS values from our simulation results.

Table 1	Summary of average SGCS between the SVD of model output and SVD of ground-truth.

| Number of CSI-RS ports | Channel model | Rank | Average of SGCS values |
| --- | --- | --- | --- |
| 16 | TDLA30-20 | 1 |  |
| (4x2x2) |  | 2 |  |
|  | TDLA30-50 | 1 |  |
|  |  | 2 |  |
|  | TDLA30-100 | 1 |  |
|  |  | 2 |  |
| 32 | TDLA30-20 | 1 | 0.80 |
| (4x4x2) |  | 2 | 0.74 |
|  | TDLA30-50 | 1 | 0.67 |
|  |  | 2 | 0.58 |
|  | TDLA30-100 | 1 | 0.27 |
|  |  | 2 | 0.22 |

Observation 1: Higer Doppler results in lower average SGCS values.

Observation 2: Average SGCS of rank 2 (average over two layers) is lower than that of rank 1.

Since RAN1 agreed to reuse Rel-18 eTypeII-Doppler codebook configuration for AI-based CSI prediction, we propose to discuss the simulation assumption to evaluate the follow PMI with Rel-18 eTypeII-Doppler.

Proposal 1: RAN4 should discuss the simulation assumption to evaluate the follow PMI performance with AI-based PMI prediction.

## 2.2	Performance monitoring

RAN1 is going to specify the performance monitoring framework so that the gNB can activate/deactivate the proper CSI reporting functionality, as shown in Figure 1. RAN1 is discussing the detailed performance monitoring framework, e.g., CSI-RS resource configuration, reported quantity, etc.

Figure 1	Performance monitoring of AI/ML-based CSI prediction model.

RAN1 is discussing three options for performance monitoring reporting for CSI prediction [4], and RAN1 has not yet selected the option(s) [5].

Agreement For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support - Based on intermediate KPI - Down select between SGCS and NMSE by RAN1#120bis - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

According to [4], in the performance monitoring Type 1 and Type 3, UE calculates the performance metric(s). In Type 3, the UE calculates the measurement quantity like SGCS or NMSE between the predicted CSI and ground-truth CSI, and then the UE reports the measurement quantity to gNB. In Type 1, before the reporting, the UE derives the monitoring output based on the configured threshold, and then the UE reports the monitoring outputs.

In Type 2, on the other hand, the UE reports both predicted CSI and/or ground-truth CSI so that the gNB can calculate the metric.

Since the required RAN4 core requirement depends on the RAN1 agreements, it is early for RAN4 to discuss the detailed core requirements. However, from our experience on CSI reporting requirements, RAN4 may need at least the following requirements.

- CSI prediction performance monitoring reporting delay requirements

- CSI prediction performance monitoring reporting accuracy requirements if type 3 is supported.

  - For example, accuracy of monitoring quantity such as SGCS/NMSE.

In our understanding, some monitoring quantities may be difficult to verify the accuracy. For example, SGCS/NMSE is derived from the predicted PMI and ground-truth PMI (PMI derived from the measured CSI-RS) at the UE. To evaluate the accuracy, TE need to know the ground-truth PMI to calculate the ideal SGCS/NMSE. If we follow the existing measurement quantities like L1-RSRP/SINR specified in TS 38.215, the measurement reference point is at the UE antenna connectors. If the measurement reference point of SGCS/NMSE is also at the UE antenna connectors, we are wondering if the TE could get the ground-truth PMI.

We propose RAN4 discuss the testability of each method, and RAN4 could inform RAN1 so that RAN1 chooses a method that is testable.

Proposal 2: RAN4 discuss the testability of monitoring reporting options, and RAN4 could inform RAN1 if RAN1 does not make decision.

If RAN1 choose the option that is not testable, alternatively, we may check the reported quantity at the PMI reporting performance requirements together with the throughput ratio. For example, we may add the following requirements in the PMI reporting performance requirements.

- Reported SGCS/NMSE shall be more than X at least Y% of the test time (e.g., X=0.7 and Y=80%)

## 2.3	Generalization aspects, static/non-static scenarios/conditions, and propagation conditions for testing

Unlike the legacy non-AI-based CSI reporting, the AI/ML-based CSI reporting performance depends on the dataset used to train the CSI prediction model. For the non-AI-based PMI reporting performance, RAN4 usually define one PMI reporting test case per precoder type. It is because we believe the UE applies the same PMI reporting algorithm regardless of the test scenarios (e.g., antenna configuration, PMI configuration, SNR, PDSCH FRC, channel model).

On the other hand, the AI/ML-based CSI prediction performance depends on the training dataset. Even if a UE reports very accurate predicted PMI in one specific scenario, this UE may report PMI that degrades the performance in other scenarios. To ensure the generalization of CSI prediction model, we propose RAN4 define several PMI reporting requirements. At least RAN4 consider several channel models, such as TDLA30 and TDLC300, and antenna configurations, 16 CSI-RS ports and 32 CSI-RS ports.

Proposal 3: RAN4 should consider more than one conditions to guarantee the generality of the AI-based CSI prediction model. At least RAN4 consider several channel models (e.g., TDLA30 and TDLC300) and antenna configurations (e.g., 16 CSI-RS ports and 32 CSI-RS ports).

In RAN4#113, RAN4 agreed to use TDL channel model for AI/ML-based CSI prediction tests. We think the TypeII PMI reporting test should be verified with more realistic channel model than TDL channel model, such as CDL channel model. We understand RAN4 is still discussing the feasibility for performance requirements with CDL channel model in Rel-19 SI. Therefore, RAN4 should consider the AI/ML-based PMI reporting requirements with CDL channel model when a suitable model is available.

Proposal 4: RAN4 should consider defining AI/ML-based CSI prediction requirements using CDL channel model when a suitable CDL model is available.

Regarding the static/non-static scenarios/conditions for testing, RAN4 has already agreed to use static conditions as baseline for CSI prediction [6], but FFS on whether to have non-static condition.

We understand the motivation for non-static condition to verify the generality of CSI prediction model. Since the current PMI reporting tests use the fixed MCS/rank and fixed SNR test point corresponds to 90% of the peak rate with follow PMI, the determinations of SNR test point and PDSCH throughput are difficult if the test conditions are changed during the test.

One possible way is to mix more than one fixed condition tests. This is an example of test procedure:

- Step1: In time period T1, the TE sets TDLA30-10 and schedules the fixed MCS with 64QAM/0.5 and measures the PDSCH throughput with follow PMI. TE then decides the SNR test point (=SNR1) based on the throughput in T1. In time period T2, the TE sets TDLC300-100 and schedules fixed MCS with QPSK/0.3 and decides the SNR test point (=SNR2) based on the throughput in T2. In time period T3, …

- Step 2: After determining the test points SNR1, SNR2, …, TE reruns the same tests with random TypeI PMI with TDLA30-10/64QAM/0.5 with SNR1 in T1, with TDLC30-100/QPSK/0.3 with SNR2 in T2, …

- Step 3: TE derives the throughput ratios of follow PMI and random PMI for each time period T1, T2, …

- Step 4: UE passes the test if the throughput ratio exceeds the required gamma for each time period.

Since the channel model and MCS can be changed without RRC reconfiguration, we don’t think the test time is increased significantly.

Table 2 is an example of PMI reporting test requirements with semi-static condition. In this example, TE sets different scenarios in time duration T1, T2, and T3.

Table 2	Example of PMI test with semi-static scenario/condition

|  | Channel model | MCS | Gamma |
| --- | --- | --- | --- |
| T1 (e.g., 1000 slots) | TDLA30-10 | 64QAM, 0.5 | 2.0 |
| T2 (e.g., 1000 slots) | TDLC300-100 | QPSK 0.3 | 1.3 |
| T3 (e.g., 1000 slots) | TDLA30-50 | 16QAM 0.5 | 1.7 |

Proposal 5: For CSI prediction performance requirements, RAN4 discuss the feasibility of mixing several static condition tests to ensure the generality of the prediction model.

# 3	Summary

Proposals:

Proposal 1: RAN4 should discuss the simulation assumption to evaluate the follow PMI performance with AI-based PMI prediction.

Proposal 2: RAN4 discuss the testability of monitoring reporting options, and RAN4 could inform RAN1 if RAN1 does not make decision.

Proposal 3: RAN4 should consider more than one conditions to guarantee the generality of the AI-based CSI prediction model. At least RAN4 consider several channel models (e.g., TDLA30 and TDLC300) and antenna configurations (e.g., 16 CSI-RS ports and 32 CSI-RS ports).

Proposal 4: RAN4 should consider defining AI/ML-based CSI prediction requirements using CDL channel model when a suitable CDL model is available.

Proposal 5: For CSI prediction performance requirements, RAN4 discuss the feasibility of mixing several static condition tests to ensure the generality of the prediction model.

# References

- R4-2502856, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated.

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm.

- R4-2503026, “Baseline Simulation assumptions for AIML based CSI prediction”, OPPO, et al.

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

- Report of 3GPP TSG RAN WG1 #120, 3GPP RAN WG1.

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm Incorporated.

# Appendix

Performance monitoring in TR [4].

For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).