3GPP TSG-RAN WG4 Meeting #114bis	R4-2504349

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.10.4.1

Source:	MediaTek Inc.

Title:	Discussion on NSGO channel model for NTN testing

Document for:	Discussion

# Introduction

At RAN4#114 meeting, a WF for NGSO testing was agreed [1]. In this contribution, we provide our views on outstanding issues.

# Discussion

Issue 1-1-1 Satellite position/velocity estimation based on equation of motion - WF - RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay based on the Keplerian ’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test. Specify both the satellite position/velocity calculation methods in TS 38.101-5/TS 36.102 - Note: the details models for Keplerian ’s equation and equation of motion can be referred in Appendix - RAN4 will introduce the accuracy requirement applied for two models compared with GMAT - Companies are encouraged to provide the calibration and comparison results with two models before RAN4#114bis meeting, based on calibration and comparison results, further discuss how to define the related accuracy requirement in the next meeting. - FFS how to capture the TE-emulated time varying doppler shift and propagation delay model in the specification - Option 1: capture into the normative part of Appendix, details need to be further discussed

In RNA4, two distinct methods are discussed to generate varying Doppler shifts and delays for satellite communication simulations. The first method, based on Keplerian orbital mechanics, utilizes the laws of planetary motion to accurately predict the satellite's position and velocity in its orbit. This method requires a coordination transformation between the Earth-Centered Inertial (ECI) frame and the Earth-Centered Earth-Fixed (ECEF) frame to calculate the Doppler effect and signal delay. Additionally, it relies on numerical methods, such as the Newton method, which necessitates several iterations and a stopping criterion to achieve the desired precision.

On the other hand, the equation of motion method offers a different approach. It directly solves the ordinary differential equations that describe the satellite's motion without the need for coordination transformation between ECI and ECEF frames. This simplifies the process as it eliminates the complex step of converting coordinates from one frame to another. Furthermore, the RK4 method does not require the use of the Newton method or any iterative process with a stopping criterion, making it a more straightforward option for simulating complex orbital dynamics where analytical solutions are not feasible.

We show simulations results for the difference between GMAT, Keplerian method and equation of motion, where the derivation of Keplerian method and equation of motion are based on Appendix in [1] and initial ephemeris information is from the excel file LEO-600 in [2].

Figure 1 illustrates that the discrepancies in the position and velocity vectors when comparing the Keplerian method to the equation of motion are relatively minor. Consequently, as depicted in Figure 2, the resulting Doppler shifts and time delays, which are derived from these vectors, also exhibit a high degree of similarity.

Subsequently, we juxtapose the results from Keplerian method and the equation of motion with the results from GMAT as presented in [2]. Figures 3 and 4 reveal significant discrepancies, particularly in the position vectors. Figure 5 further demonstrates that the maximum Doppler difference between GMAT and the two methods is approximately 30Hz, and the maximum time delay difference is around 4ns.

| Figure 1. Comparison of position and velocity vector between Keplerian and equation of motion | Figure 2. Comparison of Doppler and time delay between Keplerian and equation of motion |
| --- | --- |

| Figure 3. Comparison of position vector between GMAT, Keplerian and equation of motion | Figure 4. Comparison of position vector between GMAT, Keplerian and equation of motion |
| --- | --- |

Figure 5. Comparison of Doppler and time delay between GMAT, Keplerian and equation of motion

Observation 1: The discrepancies in the position and velocity vectors, when contrasting the Keplerian method with the equation of motion, are notably slight.

Observation 2: The outcomes for time-varying Doppler shifts and time delays from the Keplerian method and equation of motion are almost the same.

Observation 3: The discrepancies in the position and velocity vectors, when contrasting the Keplerian method and equation of motion with GMAT, are non-negligible.

Observation 4: The outcomes for time-varying Doppler shifts and time delays from the Keplerian method and equation of motion compared to GMAT are quite different. The maximum Doppler difference between GMAT and the two methods is approximately 30Hz, and the maximum time delay difference is around 4ns.

The detailed procedures for generating satellite trajectories, Doppler effects and time delays using the GMAT, are outlined in [2]. Unlike the Keplerian method and the equation of motion, which require only the initial position and velocity vectors, GMAT necessitates the determination of numerous additional parameters, as illustrated in Figures 6 and 7. These parameters include the Gravity model, Atmospheric conditions, Drag area, SRP area, etc. It is presumed that this extensive parameterization in GMAT accounts for the notable discrepancies observed when comparing its results with Keplerian method and the equation of motion.

Figure 6. Setup for GAMT in [2]

Figure 7. Setup for GAMT in [2]

Observation 5: Compared to Keplerian method and equation of motion, GAMT consider more effects and depends on many parameters to generate varying Doppler shit and time delay.

Proposal 1: RAN4 to reconsider whether and how to use GMAT as a basis for introducing accuracy requirements.

# Conclusion

In this contribution, we provide our views on NTN testing for NGSO. Observations and proposals are summarized as below.

Observation 1: The discrepancies in the position and velocity vectors, when contrasting the Keplerian method with the equation of motion, are notably slight.

Observation 2: The outcomes for time-varying Doppler shifts and time delays from the Keplerian method and equation of motion are almost the same.

Observation 3: The discrepancies in the position and velocity vectors, when contrasting the Keplerian method and equation of motion with GMAT, are non-negligible.

Observation 4: The outcomes for time-varying Doppler shifts and time delays from the Keplerian method and equation of motion compared to GMAT are quite different. The maximum Doppler difference between GMAT and the two methods is approximately 30Hz, and the maximum time delay difference is around 4ns.

Observation 5: Compared to Keplerian method and equation of motion, GAMT consider more effects and depends on many parameters to generate varying Doppler shit and time delay.

Proposal 1: RAN4 to reconsider whether and how to use GMAT as a basis for introducing accuracy requirements.

# Reference

- R4-2502314, Way Forward for [114][325] NTN_testing_NGSO_channel_model, Samsung

- R4-2413333, Discussion on dynamic NTN testing for NGSO, THALES