3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504352

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.24.1

Source: 	ZTE Corporation, Sanechips

Title: 	Discussions on General aspect for A-IoT

Document for:	Approval

Introduction

In the RAN#106 meeting, New WID on Rel-19 Ambient IoT [7] was approved and it’s expected to start the normative work from the Feb, 2025 meeting. In this contribution, we would like to share some further views based on the SI outcome and scope reached during RAN#106 meeting.

- Topic 2-1: System parameters Issue 2-1-1: Operating band Agreement: - NR band n8 can be used as an example band, and other FDD bands below 1GHz are not precluded. Issue 2-1-2: R2D transmission bandwidth definition Agreement: - Reuse the following definition of R2D transmission bandwidth in TR38.769 - Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D. - RAN4 will re-visit the two agreed bullets above, if RAN1 has the different conclusion. Issue 2-1-3: R2D channel bandwidth definition Agreement: - Define R2D channel bandwidth as: - Channel bandwidth, BCH,R2D from a reader perspective: The frequency resources used for transmitting R2D, and potential guard band. Issue 2-1-4: Values of R2D bandwidth Agreement: - Define R2D transmission bandwidth configuration: - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS - FFS on other RB numbers - Further discuss the R2D channel bandwidth considering additional guard band. - If RAN1 has the agreement different than above, RAN4 will revisit the agreements Issue 2-1-5: R2D channel raster and channel spacing Agreement: - Do not define channel spacing for R2D transmission - Define channel raster for R2D transmission. - Details need further discussion. Issue 2-1-6: D2R bandwidth Agreement: - D2R channel bandwidth for A-IoT device - Define D2R channel bandwidth for A-IoT device considering the following aspects: - [1SB or] 2SB - D2R maximum data rate (depending on RAN1 conclusion) - SFO - FDMA, i.e., considering the different device 1 has different frequency shift - Reference Filter performance - Other aspects are not precluded Issue 2-1-7: D2R channel raster and channel spacing Agreement: Do not define channel raster and channel spacing for D2R

Discussion

- 2.1 General aspects for A-IoT BS in D1T1

- Regarding the operation frequency, it is indicated in the WID that band n8 would be used as example band, from our understanding, other NR bands close to band n8 is not precluded, however this is also dependent on the operator market demand. From A-IoT BS perspective, NR BS is capable to operate at both band n8 and band n5. From A-IoT device perspective, as far as we know, device 1 is capable to operate from 800MHz to 1000MHz, the applicable frequency range is very wide to accommodate the operating bands close to 900MHz, however the exact operating frequency could be limited into 20MHz considering antenna radiation performance. In other words, device need to have some antenna retunning capability to support the large applicable frequency range and the exact operating bandwidth is limited within 20MHz.

Figure 1. the candidate example bands for A-IoT operation

Proposal 1: propose RAN4 to discuss the supported frequency range at device side to further determine the supported NR bands for A-IoT system e.g. band n5.

- For the system parameters for R2D and D2R transmission, it’s expected to have more discussions on it. For R2D transmission, from our initial understanding, the minimum channel bandwidth is supposed to be defined based on the potential minimum PRB configuration for R2D and additional guard band on each side (e.g. 1/18 of R2D transmission bandwidth which is the same as LTE spectrum utilization) to define the ACLR and OBUE requirement.

- Proposal 2: propose to define the minimum carrier bandwidth for R2D transmission as following with 10kHz, 20kHz, 30kHz, 40kHz GB on each side of R2D transmission reserved for 1/2/3/4 PRB transmission.

Figure 2. the proposed carrier bandwidth for R2D transmission

- For D2R bandwidth, there were some initial discussion with the agreement captured following, however it’s still not clear how to decide the D2R bandwidth based on certain criteria.

- Table 2. potential bandwidth for D2R reflection

| - D2R bandwidth (kHz) | - Chip length (us) | - D2R - (1/2 Manchester code, kbps) |
| --- | --- | --- |
| - 15 | - 133.3 | - 3.75 |
| - 30 | - 66.67 | - 7.5 |
| - 60 | - 33.33 | - 15 |
| - 90 | - 22.22 | - 22.5 |
| - ... | - ... | - ... |
| - 1440 | - 1.389 | - 360 |
| - 1920 | - 1.042 | - 480 |
| - 2160 | - 0.926 | - 540 |
| - 2700 | - 0.741 | - 675 |

Agreement: - D2R channel bandwidth for A-IoT device - Define D2R channel bandwidth for A-IoT device considering the following aspects: - [1SB or] 2SB - D2R maximum data rate (depending on RAN1 conclusion) - SFO - FDMA, i.e., considering the different device 1 has different frequency shift - Reference Filter performance - Other aspects are not precluded

- Proposal 3: propose RAN4 to discuss how to use these above factors to be decide the D2R bandwidth e.g. any evaluation criteria.

Figure 3. the illustration of OOK signal generation for R2D [Other REs is assumed as 0 for Rel-19 A-IoT BS transmission]

- Regarding the carrier bandwidth for D2R transmission, as clarified in table 2, different D2R bandwidth might need be specified according to the corresponding D2R data rate, however considering the potential FDM operation between different devices, then carrier bandwidth could be larger than D2R bandwidth per device. In addition, D2R signal generation (e.g. OOK based or BPSK based approach) is different compared with R2D signal generation, therefore the PRB configuration or sub-carrier spacing related concept cannot be reused for D2R transmission from our understanding.

- Proposal 4: propose not to reuse the PRB configuration or sub-carrier spacing related concept for D2R transmission.

Figure 6. the illustration of OOK signal generation for D2R signal [freq error of D2R transmission should be considered within GB]

- Regarding the channel raster for A-IoT service, if the standalone spectrum is allocated for A-IoT R2D transmission, then channel raster for R2D transmission should be at least 100KHz. If the in-band spectrum is allocated for A-IoT R2D transmission, then channel raster could be defined as following:

- N*100kHz channel raster+delta_offset

- where offset could be -47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5KHz

Figure 7. the illustration of OOK signal generation for D2R signal [freq error of D2R transmission should be considered within GB]

- Proposal 5: for R2D transmission in Standalone operation mode, to define the channel raster as 100KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [-47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5]KHz.

Conclusions

In this contribution, we shared some initial views on the RF requirement for A-IoT BS in D1T1 and proposals are made as following:

Proposal 1: propose RAN4 to discuss the supported frequency range at device side to further determine the supported NR bands for A-IoT system e.g. band n5.

- Proposal 2: propose to define the minimum carrier bandwidth for R2D transmission as following with 10kHz, 20kHz, 30kHz, 40kHz GB on each side of R2D transmission reserved for 1/2/3/4 PRB transmission.

- Proposal 3: propose RAN4 to discuss how to use these above factors to be decide the D2R bandwidth e.g. any evaluation criteria.

- Proposal 4: propose not to reuse the PRB configuration or sub-carrier spacing related concept for D2R transmission.

- Proposal 5: for R2D transmission in Standalone operation mode, to define the channel raster as 100KHz; for R2D transmission in in-band operation mode, to define the channel raster as 100KHz*N+delta_offset where delta_offset could be as [-47.5,-37.5, -27.5, -17.5, -7.5, 0, 2.5,12.5, 22.5,32.5,42.5]KHz.

References

- RP-240826,Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR (CMCC), Approved.

- R4-2410597, WF on impacts of A-IoT on RF requirements, Approved.

- R4-2413282, RF requirements for Ambient IoT BS, Huawei,

- [4] R4-2417088, WF on RF requirement impact of A-IoT, Huawei, Approved.

- R4-2414305,WF on RF requirements for A-IoT, Huawei, approved.

- R4-2410597,WF on impacts of A-IoT on RF requirements, Approved.

- RP-243326, New WID on Rel-19 Ambient IoT, Approved.

- TR 38.769,Study on solutions for Ambient IoT (Internet of Things) in NR

- R4-2502858, WF on requirements for A-IoT device, CMCC, approved.

- R4-2502859, WF on requirements for A-IoT BS and CW, Huawei, approved.