3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504357

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.12.1.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirement for UL-MIMO for ATG UE in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. In last RAN4#110bis meeting, there were some initial discussions on ATG UE supporting UL MIMO, however there are lots of remaining issues. In this contribution, we would like to share further views on ATG UE supporting UL MIMO.

Issue 3-1: ULFPTx mode for UL-MIMO - Proposals - Option 1: ULFPTx Mode is not relevant to this feature. (ZTE) - Option 2: ULFPTx mode still applies for ATG UE to support different implementation (CMCC, LGE, Ericsson) - Option 3: Depend on ATG UE implementation whether to leverage the existing ULFPTx mode. (Huawei). WF: FFS Issue 3-2: whether preclude the single antenna port related requirement - Proposals - Option 1: no (CMCC, LGE, Ericsson, Huawei) - Option 2: not to consider the single antenna port transmission TxD for ATG UE operation. (ZTE). WF: FFS

Discussion

- 2.1. RF requirements for UL MIMO

- 2.1.1 Rel-16 Full power mode

During the previous RAN4 meetings, we have some initial discussion on ULFPTx mode, as shown in the following diagrams extracted from TR 38.837,

- Rel-16 ULFPTx mode 1 is targeted for single layer transmission with two antenna ports instead of two layers transmission.

- ULFPTx Mode 2 Mechanism-1 is targeted for SRS port virtualization and ULFPTx Mode 2 Mechanism-2 is targeted for single layer transmission with single antenna port.

- ULFPTx Mode 0 is targeted for single layer transmission with single antenna port.

From our understanding, ATG UE with UL MIMO is targeted for two layer transmission instead of single layer transmission to enhance the uplink capacity instead of uplink coverage improvement.

Fig. 5.3.3.1-1. Illustration of Rel-16 ULFPTx Mode 1

For Rel-16 ULFPTx mode 1, codebook as 1/sqrt(2) [1 1] for coherent transmission is agreed to be used for non-coherent transmission. Technically speaking, this is the same as TxD without CDD implementation to be considered.

ULFPTx Mode 2 Mechanism-1 (SRS port virtualization):

Fig. 5.3.3.2-1. Illustration of Rel-16 ULFPTx Mode 2 (Mechanism-1, SRS port virtualization)

For ATG UE implemented with different polarization, it is not sure whether such kind of SRS virtualization are needed for ATG UE.

ULFPTx Mode 2 Mechanism-2 (TPMI indication):

Fig. 5.3.3.2-2. Illustration of Rel-16 ULFPTx Mode 2 (Mechanism-2, TPMI indication)

For ATG UE, we don’t see the good justification why ATG UE need to be implemented with different PA capability for different RF chains which is different from normal smartphone with 23/26dBm dual PA and potentially need to reports existing TPMI(s) available for full power transmission e.g. Rel-16 power scaling factor s = 1 is applied. If the concerns is still about Rel-15 mechanism, s = ½ for TPMI = {1 0}, ATG UE could indicate the higher power class to the network could allow the maximum transmission from UE side. It should be noted that power class indication for ATG UE is in the range of 23-40dB instead of limiting to PC3/PC2 etc

Fig. 5.3.3.3-1. Illustration of Rel-16 ULFPTx Mode 0

For ATG UE, we don’t see the good justification why Rel-16 ULFPTx mode 0 should be applied since both RF chain should be available for the UL transmission. If the concerns is still about Rel-15 mechanism, s = ½ for TPMI = {1 0}, ATG UE could indicate the higher power class to the network could allow the maximum transmission from UE side. It should be noted that power class indication for ATG UE is in the range of 23-40dB instead of limiting to PC3/PC2 etc.

Proposal 1: ULFPTx Mode is not relevant to this feature.

- 2.1.2 Rel-17 TxD

From single antenna port related feature which is also called TxD in the specification or SCDD in the implementation, this is mainly designed for single port transmission with two physical transmission ports with some cyclic shifting between signals transmitted in the time domain to mitigate the fading response of the channel. The detailed example implementation could be found as following, however this is not useful for ATG UE operating almost in the AWGAN channel. In other words, the artificial channel response added by TxD or SCDD as shown in following figure might have negative impacts of ATG UE operation. As shown in the figure 2.1.2-4, we cannot see any performance gain for this TxD feature for ATG UE. In again, we don’t see its justification on the benefit or usefulness of this feature.

Figure 2.1.2-1. the frequency-domain spectrum of field data

Figure 2.1.2-2. cyclic shifting for TxD implementation

Figure 2.1.2-3. the channel response in the frequency domain

Figure 2.1.2-4. link level simulation results for TxD under the AGWN channel

Proposal 2: not to consider the single antenna port transmission TxD for ATG UE operation.

Conclusions

In this contribution, we shared some initial views on the RF requirements for ATG UE supporting CA and UL MIMO in Rel-19 and proposals are made as following:

Proposal 1: ULFPTx Mode is not relevant to this feature.

Proposal 2: not to consider the single antenna port transmission TxD for ATG UE operation.

References

- RP-240839, New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321919,Big CR for TS 38.101-1 for NR ATG, Agreed.

- R4-2403864, CR for 38101-1 to update ATG related signaling name, Agreed

- R4-2406594, WF on Rel-19 ATG UE requirements, Approved.

- R4-2410687,WF on Rel-19 ATG UE RF requirements, Approved.

- R4-2502952,WF on UE RF requirements for ATG, CMCC, approved.