3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504359

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.13.4

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on FR1 BS transmitter co-existence spurious emission requirements

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In the RAN#106 meeting, the following objective is added for the reply LS to ETSI TFES. In this contribution, we would like to share our initial understanding on this work and its corresponding work plan for it.

Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

Discussion

## 2.1. Background and general principle

For the co-existence spurious emission requirement defined in Table 6.6.5.5.1.3-1 of TS38.104, it requires the NR BS to comply with the same requirement (e.g.-52dBm/MHz) regardless of the victim system’s operating frequency range (e.g. from 450MHz to U6GHz), this looks a bit strange from the the practical deployment perspective.

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limit | Measurement bandwidth |
| --- | --- | --- | --- |
| E-UTRA Band 31 or NR Band n31 | 462.5 -467.5 MHz | -52 dBm | 1 MHz |
|  | 452.5 -457.5 MHz | -49 dBm | 1 MHz |
| E-UTRA Band 46 or NR Band n46 | 5150 – 5925 MHz | -52 dBm | 1 MHz |
| E-UTRA Band 47 | 5855 – 5925 MHz | -52 dBm | 1 MHz |
| E-UTRA Band 48 or NR Band n48 | 3550 – 3700 MHz | -52 dBm | 1 MHz |
| NR Band n102 | 5925 – 6425 MHz | -52 dBm | 1 MHz |
| NR Band n104 | 6425 – 7125 MHz | -52 dBm | 1 MHz |

In the following section, we provided the background how these co-existence requirement was defined in the past.

| UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 2110 – 2170 MHz For protection of UE DL at the coexisting band: -174dBm/Hz+9+53=-52dBm | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65 |
| --- | --- | --- | --- | --- |
|  | 1920 – 1980 MHz For protection of BS UL at the coexisting band: -174dBm/Hz+5+67-7=-49dBm | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.6.5.2.2. |

If the co-existence spurious emission requirement is to protect the uplink receiver performance at BS side, then it is necessary to make sure the received spurious emission at victim node 7dB lower than its noise figure. In addition, this requirement is based on the study outcome in 3G phase as captured in TR25.942. Based on the following deployment layout assumption and also antenna again from both aggressor and victim BS.

Figure 2.1. the network layout assumption for 3G phase from our interpretation in 7.4.1.2.3 of TR 25.942

MCL=FreeSpace-Tx_gain-Rx_gain= 87-10-10=67dBc

Where:

- Freespace is is corresponding to the pathloss of 577/2 meter;

- Tx gain and Rx gain is assumed as 10dB instead of 13dBi for some mechanical downtilt considered.

Based on the above information, then for the protection of the victim BS receiver performance, then it is equal to -174dBm/Hz+60+5+67-7=-49dBm;

If the co-existence spurious emission requirement is to protect the UE receiver performance in the downlink direction, then it could be calculated as following:

-174dBm/Hz+60+9+53=-52dBm

Where:

- Freespace is is assumed as 53dBc as worst case considering the MCL for Medium range BS class

- UE noise figure is assumed as 9dB;

The the generic principle how to specify the appropriate co-existence spurious emission requirement for the protection of BS receiver in the uplink direction and UE receiver in the downlink direction could be reused for Rel-19 U6GHz coexistence spurious evaluation.

Proposal 1: reuse the same principle to specify the co-existence spurious emission requirement for the protection of BS receiver and UE receiver for BS operating at 5.9GHz.

For FDD band:

- For DL UE protection, reuse the 3db degradation;

- For UL BS protection, reuse the 0.8dB degradation.

For TDD bands:

- For DL UE protection, reuse the 3db degradation;

- For UL BS protection, reuse the 0.8dB degradation.

- Further discuss how to pick up the appropriate value.

## 2.2. Evaluation methodology and simulation assumptions

As discussed in the previous clause, in addition to the noise figure of victim device, MCL between two node (e..g BS2BS or BS2UE) are also quite important to determine the appropriate co-existence spurious emission requirement. From our understanding, the following principle could be used as starting point for further co-existence evaluation.

- for UE DL protection, victim UE is closest to the aggressor BS.

- For UL BS2BS coexistence, 100% grid shift as starting point.

Proposal 2: for the network layout assumption to evaluate the MCL between the aggressor BS and victim UE/BS,  propose to use the following assumption as starting point.

- for UE DL protection, victim UE is closest to the aggressor BS.

- For UL BS2BS coexistence, 100% grid shift as starting point.

For coexistence spurious emission requirement, for the certain interested regions, lots of coexistence bands should be considered. As shown in the following diagram, the following coexistence bands should be considered from the practical deployment perspective. For 800MHz, 900MHz or 2.3GHz, these bands could be also been considered, however the evaluation results for MCL might be quite similar as adjacent bands, e.g. band 28 or n41.

Figure 2.2. MCL evaluation for additional coexistence requirements.

Proposal 3: for U6GHz coexistence spurious emission requirement evaluation, propose to consider the following example spectrum, 700MHz, 1.8GHz, 2GHz, 3.5GHz, 4.9GHz.

In the clause 2.1, for Tx gain assumption of U6GHz BS for spurious emission performance towards the victim bands, this needed more technical discussions in RAN4. In the past, we have some initial discussions for antenna pattern back to SI phase of AAS topic e.g. reusing the different correlation factors across different RF chains for unwanted emission requirement next to the wanted signal, however this might be not valid assumption for antenna pattern of the spurious emission signal. More simulation results would be provided later on for the information and also technical justification. In order to trigger the RAN4 evaluation work as soon as possible due to the limited TU budget, the following assumption could be used as starting point.

  - Option 1: to consider the antenna element pattern only without any sub-array and beamforming gain considered

  - Option 2: to consider the sub-array pattern only without beamforming included.

  - Option 3: to consider AAS modelling with different correlation level similar as AAS SI phase.

  - Option 4: to consider the simulated antenna pattern for spurious emission [to be provided later on]

Proposal 4: for Tx gain assumption for spurious emission signal of U6GHz BS, propose to consider the following options for the further simulations.

  - Option 1: to consider the antenna element pattern only without any sub-array and beamforming gain considered

  - Option 2: to consider the sub-array pattern only without beamforming included.

  - Option 3: to consider AAS modelling with different correlation level similar as AAS SI phase.

  - Option 4: to consider the simulated antenna pattern for spurious emission [to be provided later on]

For MCL calculation, Rx gain of victim system should be also considered, indeed both UE antenna gain and BS antenna gain should be considered. For the antenna gain/pattern of victim UE, it should be quite clear to use the omini antenna as simulation assumption. For antenna pattern of victim BS, the following factors could be considered:

- For low than 2GHz, both non-AAS antenna and AAS antenna should be considered since FDD massive MIMO is also supported.

  - To figure out the most stringent requirement between non-AAS and AAS antenna from victim perspective.

- For above than 2GHz, AAS antenna should be considered

  - MCL at the sub-array connector should be considered.

Proposal 5: for Rx gain assumption of victim BS subject to spurious emission interference, propose to consider the following principle for the further simulations.

- For low than 2GHz, both non-AAS antenna and AAS antenna should be considered since FDD massive MIMO is also supported.

  - To figure out the most stringent requirement between non-AAS and AAS antenna from victim perspective.

- For above than 2GHz, AAS antenna should be considered

  - MCL at the sub-array connector should be considered.

- For the detailed simulation assumption for co-existence spurious emission study, BS antenna configuration for 700MHz to 4.9GHz could be found as following table 1-4.

Proposal 6: for Tx antenna pattern and Rx antenna pattern, propose to use the antenna configuration in Table 1/2/3/4 for the initial simulation. For Tx gain at U6GHz, BS antenna gain could be assumed as 0dBi as starting point.

Table 1. BS type 1-H with AAS antenna configuration for 1710-4900MHz

|  |  | Rural | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |  | N/A |  |
| 1.2 | Element gain (dBi) (Note 2) | 7.1 | 7.1 | 6.4 | N/A | N/A |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 54º for V | 90º for H 54º for V | 90º for H 65º for V | N/A | N/A |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | N/A | N/A |  |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º | N/A | N/A |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 4 × 8 elements | 4 × 8 elements | 4 × 8 elements | N/A | N/A |  |
| 1.6a | Number of element rows in sub-array | 1x3 | 1x3 | 1x3 | N/A |  |  |
| 1.6b | Pre-set sub-array down-tilt (degrees) | 6 | 6 | 6 | N/A |  |  |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | N/A | N/A |  |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 | N/A | N/A |  |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) | 25 | 25 | 25 | N/A | N/A |  |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 | 120 | N/A | N/A |  |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-120 | N/A | N/A |  |
| 1.12 | Mechanical downtilt (degrees) | 3 | 6 | 10 | N/A | N/A |  |

Table 2. BS type 1-C with non-AAS configuration for 700MHz

| Parameter for BS | Values |  |
| --- | --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=−min12θ″−90°θ3dB2,SLAV,θ3dB=10°,SLAV=20dB |  |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=−min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |  |
| Combining method for 3D antenna pattern (dB) | A“θ",φ”=−min−AE,Vθ"+AE,Hφ”, Am |  |
| Maximum directional gain of an antenna GE,max | 17 dBi |  |
| Conducted power | 46 dBm |  |
| Mechanical downtilt in degrees | Rural | 3 |
|  | Urban | 10 |

Table 3. BS type 1-H with AAS configuration with sub-array for U6GHz [this depends on the conclusion on  the proposal 4]

| Parameter | Urban Macro |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna sub-array configuration (Row × Column) (Note 4) | 16 × 8 elements |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V |
| Number of element rows in sub-array | 3 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | 6 |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 26.76 |
| Mechanical down-tilt (degrees) | 0 |

Table 4. BS type 1-H with AAS configuration without sub-array for U6GHz [this depends on the conclusion on the proposal 4]

| Parameter | Urban Macro |
| --- | --- |
| Element gain (dBi) (Note 2) | 6.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) (Note 4) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.7 of wavelength for V |
| Number of element rows in sub-array | 1 |
| Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V |
| Pre-set sub-array down-tilt (degrees) | N/A |
| Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 26.76 |
| Mechanical down-tilt (degrees) | 0 |

For UE antennas, an omni-directional radiation pattern with antenna gain 0dBi is assumed.

Conclusions

In this contribution, we shared some initial views on the applicability improvement of TRP in Annex in RAN4 and proposals are made as following:

Proposal 1: reuse the same principle to specify the co-existence spurious emission requirement for the protection of BS receiver and UE receiver for BS operating at 5.9GHz.

For FDD band:

- For DL UE protection, reuse the 3db degradation;

- For UL BS protection, reuse the 0.8dB degradation.

For TDD bands:

- For DL UE protection, reuse the 3db degradation;

- For UL BS protection, reuse the 0.8dB degradation.

- Further discuss how to pick up the appropriate value.

Proposal 2: for the network layout assumption to evaluate the MCL between the aggressor BS and victim UE/BS,  propose to use the following assumption as starting point.

- for UE DL protection, victim UE is closest to the aggressor BS.

- For UL BS2BS coexistence, 100% grid shift as starting point.

Proposal 3: for U6GHz coexistence spurious emission requirement evaluation, propose to consider the following example spectrum, 700MHz, 1.8GHz, 2GHz, 3.5GHz, 4.9GHz.

Proposal 4: for Tx gain assumption for spurious emission signal of U6GHz BS, propose to consider the following options for the further simulations.

  - Option 1: to consider the antenna element pattern only without any sub-array and beamforming gain considered

  - Option 2: to consider the sub-array pattern only without beamforming included.

  - Option 3: to consider AAS modelling with different correlation level similar as AAS SI phase.

  - Option 4: to consider the simulated antenna pattern for spurious emission [to be provided later on]

Proposal 5: for Rx gain assumption of victim BS subject to spurious emission interference, propose to consider the following principle for the further simulations.

- For low than 2GHz, both non-AAS antenna and AAS antenna should be considered since FDD massive MIMO is also supported.

  - To figure out the most stringent requirement between non-AAS and AAS antenna from victim perspective.

- For above than 2GHz, AAS antenna should be considered

  - MCL at the sub-array connector should be considered.

Proposal 6: for Tx antenna pattern and Rx antenna pattern, propose to use the antenna configuration in Table 1/2/3/4 for the initial simulation. For Tx gain at U6GHz, BS antenna gain could be assumed as 0dBi as starting point.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2419797	Way Forward for [113][303] NR_BS_RF_Part2_CLTA

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP