3GPP TSG-RAN WG4 Meeting#114bis                                R4-2504370

Wuhan, China, 07th – 11th April, 2025

Agenda item:	7.23.2.2

Source: 	ZTE Corporation

Title: 	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

Discussion

## 2.1. Tx requirements

### 2.1.1. Additional OBUE requirement

Issue 2-3-2: OBUE - Agreement: - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency. Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

Similar as Co-located ACLR requirement, during the last RAN4 meeting, there were some discussions on how to define OBUE requirement for SBFD capable BS in order to ensure the coexistence performance for SBFD BSs operating in adjacent channel shown as following. During the Rel-18 SBFD SI phase, the following coexistence scenario was not evaluated, however the CLI interference is quite similar as Case 3 where NR DL interfering SBFD UL. Especially for Scenario 1, Case 3, the severe performance degradation could be observed for SBFD UL due to the strong CLI from NR DL in the adjacent channel. Even with ACIR requirement tightening 8dB, the 5% throughput loss is still up to 43.5% and the mean throughput loss could be minimized down to 4.67%. However as we know that, for AAS based SBFD BS, due to the higher operating frequency and larger channel bandwidth and small transceiver’s PA linearity, there are no much ACLR/OBUE requirement to be further improved. In other words, even though RAN4 is capable to agree to define more tightening requirement e.g. with 10dB tightening requirements than the legacy requirements , it is still not feasible to be achieved for SBFD capable BS by improving the front-end PA linearity.

As mentioned before, the RF filter for wide area SBFD BS and coupling loss between aggressor and victim system is important to verify its co-location coexistence performance. In addition, for band n41, single operator is holding the whole 160MHz spectrum. For n78, two operators are holding the contiguous 200MHz spectrum and do the spectrum sharing. For n79, two operators are holding the contiguous 160MHz spectrum and some spectrum sharing might be also feasible. For U6GHz in China, there are about 700MHz spectrum available to be allocated, it might be possible operating band to have multiple operator coexistence scenario in China.

In short, for SBFD vs SBFD coexistence scenario among different operators, we propose to focus on the band n104 or treat it as 1st priority if that agreeable.

Proposal 1: for SBFD vs SBFD coexistence scenario among different operators, propose to treat the band n104 as 1st priority, for other interested bands, it depends on the operator’s demand.

In addition, as we discussed in the previous RAN4 meeting, how to guarantee the co-location coexistence performance between two different SBFD carries is highly dependent on the transmitter analog performance and also its coupling loss between the aggressor and victim system.

If the analog filter attenuation at the certain freq offset away from the RF bandwidth edge could be achieved as 34dBc together with MCL as 67dB which will results in -116dBm/MHz. This leakage interference is supposed to be 7dB lower than victim’s noise figure which is aligned with the legacy principle for co-location spurious requirement. However we might need to more time to verify this coupling loss based on the U6GHz BS in the OTA chamber. Meanwhile more analysis might be needed to get the baseline/default value for the required freq offset to achieve 34dBc attenuation, otherwise it will be very ambiguous to define this requirement with the declared freq offset to meet the requirement which will cause lots of product fragmentation.

Some initial analysis for analog filtering design at U6GHz is provided as following for the information, more data would be provided later on. Regarding the coupling loss between different U6GHz BS could be potentially provided after the May meeting or during the May meeting.

|  | Freq offset to the operators’ spectrum edge |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS RF bandwidth | 10MHz | 15MHz | 20MHz | 40MHz | F1 | F2 |
| 100MHz | 2.2 | 3 | 10 | 20 | 34? | TBD |
| 200MHz | TBD | TBD | TBD | TBD | TBD | TBD |
| 300MHz | TBD | TBD | TBD | TBD | TBD | TBD |
| 400MHz | TBD | TBD | TBD | TBD | TBD | TBD |

Proposal 2a: if it is agreeable to define the co-location OBUE requirement, propose to get the baseline/default value for the required freq offset to achieve the required attenuation, otherwise it will be very ambiguous to define this requirement with the declared freq offset to meet the requirement which will cause lots of product fragmentation.

Proposal 2b: both the co-location requirement and also coexistence requirement between two SBFD operators should be guaranteed if the coexistence scenario is agreeable.

### 2.1.2. EVM

WF: - Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity.

In the last RAN4 meeting, for EVM measurement for SBFD BS was agreed in the last RAN4 meeting as above, some clarification might needed. For example, in the following SBFD configuration with 2.5ms dual TD DL-UL periodicity, the 1st flexible slot with 14 flexible symbols and last flexible slot with 14 flexible symbols within 2.5ms could be configured as SBFD symbols, however it should be known that some flexible symbol in the 1st and last flexible slots would be reserved as guard period or uplink transmission, in other words, RAN4 need to clarify whether these two slots should be considered or not for SBFD EVM measurement. If these two slots are not been considered for SBFD measurement, for EVM measurement, per 2.5ms, there are only 14 DL symbols available for SBFD EVM measurement.

Figure 1. some potential sub-band configuration in the time domain for the conformance testing.

Proposal 3: propose to further discuss whether the following two slots configured with flexible symbols could be used for EVM measurement

- Flexible symbols per slot is configured with both SBFD symbols and guard period symbols or

- Flexible symbols per slot is configured with SBFD symbols, guard period and also uplink symbols.

Proposal 4: propose to further consider the available frame for EVM measurement considering the limited available slots for EVM measurement, otherwise the EVM measurement uncertainty will increase (e.g. to get the equal slots for EVM samples as the legacy EVM measurement).

Conclusions

In this contribution, we want to share some initial views on RF requirements for SBFD BS and proposals/observations are made as following:

Proposal 1: for SBFD vs SBFD coexistence scenario among different operators, propose to treat the band n104 as 1st priority, for other interested bands, it depends on the operator’s demand.

Proposal 2a: if it is agreeable to define the co-location OBUE requirement, propose to get the baseline/default value for the required freq offset to achieve the required attenuation, otherwise it will be very ambiguous to define this requirement with the declared freq offset to meet the requirement which will cause lots of product fragmentation.

Proposal 2b: both the co-location requirement and also coexistence requirement between two SBFD operators should be guaranteed.

Proposal 3: propose to further discuss whether the following two slots configured with flexible symbols could be used for EVM measurement.

- Flexible symbols per slot is configured with both SBFD symbols and guard period symbols or

- Flexible symbols per slot is configured with SBFD symbols, guard period and also uplink symbols.

Proposal 4: propose to further consider the available frame for EVM measurement considering the limited available slots for EVM measurement, otherwise the EVM measurement uncertainty will increase (e.g. to get the equal slots for EVM samples as the legacy EVM measurement).

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413283, On modification of existing TX RF requirements for SBFD, Huawei

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2411640,On the potentially new requirements for SBFD operation for FR1 and FR2-1, Samsung

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.