3GPP TSG-RAN4 #114-bis	R4-2504372

Wuhan, China, 7th April 2025 – 11th April 2025

Title:	Discussion on the usage of the terminology “FWA form factor” and its application for 6Rx, 4Rx and 8Rx requirements

Source:	Anritsu Limited

Agenda Item:	7.1.1.3.1

Document for:	Discussion

# Introduction

The requirements for the “Four antenna port reference sensitivity allowance ΔRIB,4R” and “Eight antenna port reference sensitivity allowance ΔRIB,8R” exist in TS 38.101-1 [1] but there is an inconsistency in the terms used.

As requirements for 6Rx are likely to be added either at this meeting or the next one, it would be useful to bring coherence in the terms used to avoid some readers being puzzled by the terminology used.

For UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands. For operating band frequency range ≤ 1 GHz, the 4Rx operation is primarily for FWA form factor, and when 4Rx operation is supported by handheld UE, ∆RIB,4R as indicated in Table 7.3.2-2 NOTE 2 is applied.

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n13, n26, n28, n71, n85, n105 | -2.71 |
| n5, n8, n28, n71, n20, n26 | -2.42 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: When 4 Rx operation is supported by FWA form factor NOTE 2: When 4Rx operation is supported by handheld UE. |  |

For UE(s) equipped with 8 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2a for the applicable operating bands.

Table 7.3.2-2a: Eight antenna port reference sensitivity allowance ΔRIB,8R

| Operating band | ΔRIB,8R (dB) |
| --- | --- |
| n7 | -4.5 |
| n41 | -4.3 |
| n77, n78, n79 | -4.0 |
| NOTE 1: 8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor. |  |

# Discussion

The following observations can be made:

Observation 1: “NOTE 1:	8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor” in Table 7.3.2-2a of TS 38.101-1 is using “form factor” in singular not plural. Like if FWA/CPE/Vehicle/Industrial devices where sharing a relatively similar form factor and as if “FWA form factor” could apply to all those devices.

Observation 2: “NOTE 1:	When 4 Rx operation is supported by FWA form factor” in Table 7.3.2-2 of TS 38.101-1 is using “FWA form factor” somehow like the terminology used (“FWA UE”, “fixed wireless access (FWA) type of devices”, “FWA devices”, “FWA”) for the WID [2] related to 6Rx.

Based on them, we would like to propose:

Proposal 1: Clarify if “FWA form factor” has the same meaning as “FWA/CPE/Vehicle/Industrial devices form factor”.

Proposal 2: Do not use the term “CPE” but “FWA” in section 7.

Proposal 3: Add a definition of “FWA UE” or “FWA form factor” in sub-section 3.1, where CPE can be mentioned.

Proposal 4: Agree on an identical NOTE 1 in the three tables: Table 7.3.2-2 (4Rx), Table 7.3.2-2a (8Rx) and Table 7.3.2-2b (6Rx).

# Conclusion

Based on the discussion in section 2, the following can be noted, and the following proposal is proposed:

Observation 1: “NOTE 1:	8 Rx operation is targeted for FWA/CPE/Vehicle/Industrial devices form factor” in Table 7.3.2-2a of TS 38.101-1 is using “form factor” in singular not plural. Like if FWA/CPE/Vehicle/Industrial devices where sharing a relatively similar form factor and as if “FWA form factor” could apply to all those devices.

Observation 2: “NOTE 1:	When 4 Rx operation is supported by FWA form factor” in Table 7.3.2-2 of TS 38.101-1 is using “FWA form factor” somehow like the terminology used (“FWA UE”, “fixed wireless access (FWA) type of devices”, “FWA devices”, “FWA”) for the WID [2] related to 6Rx.

Proposal 1: Clarify if “FWA form factor” has the same meaning as “FWA/CPE/Vehicle/Industrial devices form factor”.

Proposal 2: Do not use the term “CPE” but “FWA” in section 7.

Proposal 3: Add a definition of “FWA UE” or “FWA form factor” in sub-section 3.1, where CPE can be mentioned.

Proposal 4: Use as much as possible identical NOTE 1 in the three tables: Table 7.3.2-2 (4Rx), Table 7.3.2-2a (8Rx) and Table 7.3.2-2b (6Rx).

# References

[1] TS 38-101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, version 19.0.0

[2] RP-240782, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Huawei, HiSilicon, TSG-RAN#104