TSG-RAN Working Group 4 (Radio) meeting #114-bis	R4-2504375

Wuhan, China, 7th to 11th April 2025

Agenda Item:	10

Source: 	Ericsson

Title: 	Views on Rel-20 RRM topics

Document for:	Discussion

# Introduction

In this contribution we present proposals for RRM enhancements related to RAN4 led items in Rel-20.

At the last RAN plenary there were brief presentations which were followed by question/answer sessions for selected set of Rel-20 proposals. The presentation also covered our Rel-20 proposal in [1]. Based on the contributions from other companies our observation is that several companies have also expressed their support for some of our proposals in our contribution [1].

The aim of this contribution is to collect further feedback on our proposals related to RRM enhancements in Rel-20 and propose way forward on the organization of these items.

# Discussions on Rel-20 RRM topics

In the area of RRM generally there are very large number of proposals. Therefore, our aim is to focus on those areas which are urgent and have substantial practical significance and cannot be covered in work items led by other working groups. Based on the arguments we have the following two proposals for RRM.

- Further enhanced RedCap

- Rel-19 leftover

## 4.1 Further enhanced RedCap

Firstly, RAN4 introduced requirements for relaxed Radio Link Monitoring (RLM) and Beam Failure Detection (BFD) in release 17 for regular NR UEs as part of the UE power saving WI [2]. This feature allows the UE to achieve power saving gain by measuring less often when fulfilling some criteria, see Figure 1.

The criteria introduced are related to radio conditions of the UE with respect to the measured cell and mobility state of the UE. However, these requirements are currently not applicable for RedCap devices. Power saving is a relevant feature for RedCap UEs as they have reduced capability and cost. Therefore, we propose to extend this CONNECTED mode power saving feature for both 1 Rx and 2 Rx RedCap.

It was noted from the last plenary discussions that many companies have expressed support for this particular enhancement for RedCap. Thus it was highlighted as candidate topic with common interest among the companies in RAN4#107, see the summary document in [3].

Secondly, RAN4 specified support for operation with less than 5 MHz CBW in FR1 for regular NR UEs as part of the Rel-18 WI on NR support for dedicated spectrum less than 5 MHz, in [4]. Based on the discussion in last few meetings and as captured in the summary document in [3], there is good support to extend this work for RedCap. Therefore, we are also positive to do the required in RAN4. The requirements shall be developed based on the requirements defined for less than 5 MHz (3MHz carrier) operation for regular NR UEs in less than 5 MHz Rel-18 WI. It is also important to note UE shall operate in 3 MHz carriers only. This way impact to existing deployments can be avoided, also the RAN4 workload can be reduced while it also simplifies the work since the principles of defining the requirements and impacted requirements are already identified. We also would like to avoid any further enhancements beyond what is supported in Rel-17/18 RedCap, i.e. the Rel-17/18 RedCap should be the reference, and any further enhancements should be out of the scope. This way the impact to the other WGs can be avoided.

Thirdly, RAN4 defined requirements to support L1/L2 triggered mobility for NR UEs in Rel-18 [5]. This feature enables low-latency handover while reducing the interruption time. But this feature is currently not supported for RedCap. Taking into consideration that RedCap comprises both 1Rx and 2Rx which in turn can support different use cases, we propose to develop requirements to support L1/L2 triggered mobility for RedCap by using the corresponding non-RedCap UE requirements as reference. From a UE complexity point of view, 2Rx RedCap is similar to non-RedCap UE and therefore it is possible reuse most of the existing requirements for 2Rx RedCap.

## 4.2 Other proposals

In addition to the RedCap related proposals in previous section, we are also interested in developing the RRM requirements to support on-demand SSB (OD-SSB) based SCell activation requirements for NR UEs. It is noted that only single SCell activation procedure is considered in ongoing release 19 NES work item. Given the deployment scenarios and commercial interest, extending this OD-SSB based SCell activation requirements for multiple SCell activation procedure can bring good power saving gain while it also reduces the SCell activation delays in multiple Scells. Therefore, in the interest of time, we propose RAN4 to also consider this work.

It was also observed from last plenary discussions that there are various proposals (e.g. A-TRS based delay reduction) to speed up SCell activation procedure. While we share the view that reducing the SCell activation delays is important, RAN4 should use the time to improve the delays for type of SCells activation procedures which are already used in the field and/or likely to be used in the field, especially given that the RAN4 TUs are to be shared between 5G advanced related topics and 6G, not many RAN4 led 5G advanced topics are to be agreed. Therefore, we propose to focus on improvements which are urgent and have substantial practical significance.

# Summary of proposals

The summary of proposals related to the RRM enhancement in Rel-20 are provided below:

Further RedCap enhancements:

- Proposal #1: Specify relaxed RLM/BFD evaluation period requirements based on the Rel-17 relaxed RLM/BFD requirements for non-RedCap UEs.

  - Relaxation is in time-domain and relaxation criteria (i.e. good serving cell- and low mobility criteria) from Rel-17 relaxed RLM/BFD work for non-RedCap UEs shall be reused

  - Relaxation condition (i.e. short DRX periodicity) from Rel-17 relaxed RLM/BFD requirements shall be reused.

- Proposal #2: Specify RRM requirements to enable 3 MHz CBW operation for 1 Rx and 2 Rx RedCap devices in 3 MHz carriers only

  - Requirements shall be developed based on the requirements defined for less than 5 MHz operations for non-RedCap UEs in less than 5 MHz Rel-18 WI.

- Proposal #3: Specify support for L1/L2 based mobility for RedCap

  - Based on the LTM requirements introduced in Rel-18 for non-RedCap UEs, define LTM requirements for RedCap UEs

Other topics

- Proposal #4: In interest of time, RAN4 to consider developing requirements to support OD-SSB based multiple SCell activation delay requirements.

- Proposal #5: RAN4 to focus on topics which are urgent and have substantial practical significance.

# References

- RP-250540, 5G-Advanced RAN4 led topics in Rel-20, Ericsson

- RP-200938, Revised WID UE Power Saving Enhancements for NR, MediaTek Inc.

- RP-250817, Moderator's summary for RAN4 led REL-20 topics, RAN4 Chair (Huawei)

- RP-234027, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Nokia Shanghai Bell

- RP-233970, Revised WID on Further NR Mobility Enhancements, Rapporteur (MediaTek Inc., Apple)