3GPP TSG-RAN WG4 Meeting #114bis	R4-2504378

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.10.2.3.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN HPUE TX RF requirements

Document for:	Approval

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable high power UE (HPUE) with defining RF requirements targeting at support of IoT-NTN UEs for FR1 single carrier scenario.

In this contribution, our views about HPUE TX RF requirements for IoT-NTN UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

2.2		Previous Meeting Agreements

In RAN4#114 meeting, the agreed WF [2] provides the following works for specifying TX requirements for IoT-NTN under HPUE operation. The partial contents are shown as follows.

Tx requirements Issue 1-1-1: ACLR for NB-Iot based Iot-NTN for PC1/2; Agreement: - Re-use current PC3 ACLR (UTRA_ACLR = 37dBc) for PC1/2. Issue 1-1-2: ACLR for eMTC based Iot-NTN for PC1/2; Agreement: o Option 1: Re-use PC3 eMTC based Iot-NTN ACLR 30dBc. o Option 2: 31dBc ACLR Issue 1-1-3: SEM for NB-Iot based NTN for PC1/2 Agreement: - Reusing SEM requirements defined for PC3 for NB-Iot based Iot-NTN Issue 1-1-4: SEM for eMTC based NTN for PC1/2 Agreement: - Reusing SEM requirements defined for PC3 for eMTC based Iot-NTN Issue 1-1-5: MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2 Agreement: - For MPR/A-MPR assumptions for NB-Iot based Iot-NTN for PC1/2 - For NB-IoT PC2, for A-MPR evaluations assume 25 dBc IQ-image and LO-leakage and 60 dBc CIM3 - For NB-IoT PC1, for MPR and A-MPR evaluations assume [28~34]dBc IQ-image and LO-leakage and 60 dBc CIM3. - For NB-IoT PC2, as a starting point consider re-use of PC3 MPR Issue 1-1-6: MPR assumptions for eMTC based Iot-NTN for PC1/2 Agreement: - For MPR/A-MPR assumptions for eMTC based Iot-NTN for PC1/2 - For Cat M1, in simulations calibrate PA to just meet ACLR with [30~31] dBc ACLR and 1 dB MPR with 100 RB DFT-s-OFDM waveform. Issue 1-1-8: The Basic MOP tolerance (Including eMTC and NB-Iot based) - Proposals - For PC2: - Option 1: +2 / -3dB - Option 2: +2 / -2dB - For PC1: - Option 1: +2 / -3dB - WF FFS Issue 1-1-9: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation. - Proposals - Option 1: Relaxation of TX MOP tolerance upper limitation for NTN HD-FDD EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). 256; 26; +2x, -2; 23; +2x, -2. 255; 26; +2x, -2; 23; +2x, -2. NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx].. - Option 2: Additional NW signal to control relaxation of TX MOP upper limitation for NTN HD-FDD. EUTRA band; Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). 256; 26x; +2, -2; 23x; +2, -2. 255; 26x; +2, -2; 23x; +2, -2. NOTE X: The power limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx].. - Option 3: New sub power classes (e.g., PC2NTN-HD-FDD) to relax the TX output power upper limitation. - Option 4: Other ways are not excluded - FD-FDD can also be considered. - WF - FFS.

2.3		UE RF requirements

2.3.1	IoT-NTN PC2 for Cat M1

Proposal 1: Regarding IoT-NTN Cat M1, consider reusing PC3(1TX) MPR and ACLR1 for PC2(1TX) MPR and ACLR1 respectively.

TS 36.102 Table 6.2A.2-1: Maximum Power Reduction (MPR) for category M1 UE for Power Class 2, 3, and 5

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) | MPR (dB) |
| --- | --- | --- |
|  | 1.4 MHz |  |
| QPSK | > 2 | ≤ 1 |
| QPSK | > 5 | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 1 |
| 16 QAM | >2 | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1 |  |  |

2.3.2	IoT-NTN PC1 for Cat M1

Proposal 2: There is no commercial PA to support PC1 IoT-TN Cat M1, consider ACLR1 of [31dBc] for IoT-NTN PC1 Cat M1 if there is no specific concern.

2.3.3	IoT-NTN PC2 for Cat NB1/NB2

The IoT-NTN Cat NB1/NB2 SEM is shown in TS 36.102 Table 6.5B.3.2-1.

Proposal 3: If IoT-NTN Cat NB1/NB2 PC2 ACLR1 is only equal to 24dBc, the IoT-NTN PC3 MPR table below cannot be directly reused as IoT-NTN PC2 MPR because the absolute SEM requirements below should be met.

Table 6.5B.3.2-1: Category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Table 6.2B.2-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-11 |  |  |  |
| MPR | 0 dB |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

2.3.4	IoT-NTN PC1 for Cat NB1/NB2

Proposal 4: Before deciding the final IoT-NTN PC1(1TX) MPR for Cat NB1/NB2, it is necessary to clarify whether the IoT-NTN PC1(1TX) ACLR1 is approximately equal to 24dBc for MPR evaluation.

2.3.5	Relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HPUE HD-FDD operation

Proposal 5: Discuss options for relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HPUE HD-FDD operation.

- Option 1: Relax TX MOP tolerance upper limitation for NTN HD-FDD TX power increment.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26 | +2x, -2 | 23 | +2x, -2 |
| 255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: Add the NTN HD-FDD TX power increment scheme by extending TS 38.101-1 power boosting framework as staring point.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26x | +2, -2 | 23x | +2, -2 |
| 255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE X: The power limit increment and related requirements in HD-FDD operation are specified in power boosting clause [xxx]. |  |  |  |  |

3 Conclusion

In this paper, our views on HPUE TX RF requirements for IoT-NTN UE are provided as follows:

Proposal 1: Regarding IoT-NTN Cat M1, consider reusing PC3(1TX) MPR and ACLR1 for PC2(1TX) MPR and ACLR1 respectively.

TS 36.102 Table 6.2A.2-1: Maximum Power Reduction (MPR) for category M1 UE for Power Class 2, 3, and 5

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) | MPR (dB) |
| --- | --- | --- |
|  | 1.4 MHz |  |
| QPSK | > 2 | ≤ 1 |
| QPSK | > 5 | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 1 |
| 16 QAM | >2 | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1 |  |  |

Proposal 2: There is no commercial PA to support PC1 IoT-TN Cat M1, consider ACLR1 of [31dBc] for IoT-NTN PC1 Cat M1 if there is no specific concern.

Proposal 3: If IoT-NTN Cat NB1/NB2 PC2 ACLR1 is only equal to 24dBc, the IoT-NTN PC3 MPR table below cannot be directly reused as IoT-NTN PC2 MPR because the absolute SEM requirements below should be met.

Table 6.5B.3.2-1: Category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Table 6.2B.2-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 1 Tone allocation | 0-11 |  |  |  |
| MPR | 0 dB |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

Proposal 4: Before deciding the final IoT-NTN PC1(1TX) MPR for Cat NB1/NB2, it is necessary to clarify whether the IoT-NTN PC1(1TX) ACLR1 is approximately equal to 24dBc for MPR evaluation.

Proposal 5: Discuss options for relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HPUE HD-FDD operation.

- Option 1: Relax TX MOP tolerance upper limitation for NTN HD-FDD TX power increment.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26 | +2x, -2 | 23 | +2x, -2 |
| 255 | 26 | +2x, -2 | 23 | +2x, -2 |
| NOTE X: The upper power tolerance limit relaxation and related requirements in HD-FDD operation are specified in sub-clause [xxx]. |  |  |  |  |

- Option 2: Add the NTN HD-FDD TX power increment scheme by extending TS 38.101-1 power boosting framework as staring point.

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 26x | +2, -2 | 23x | +2, -2 |
| 255 | 26x | +2, -2 | 23x | +2, -2 |
| NOTE X: The power limit increment and related requirements in HD-FDD operation are specified in power boosting clause [xxx]. |  |  |  |  |

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2503006, “WF on HPUE UE requirement for IoT-NTN”, Vivo