3GPP TSG-RAN WG4 Meeting #114-bis	R4-2504381

# Wuhan, China, April 7th – 11st 2025

Source:	Ericsson

Title:	Discussion on the methodology of NR SCM for demodulation

Agenda item:	7.16.2

Document for:	Discussion

# Introduction

In the RAN4#114 meeting, companies discussed remaining issues and furtherly propose some new test cases and channel models. The agreements and simulation parameters for eType-II are captured in WF [1].

In this contribution, the open issues, such as comparison metrices, alignment metrices and remaining configurations on CDL/TDL models, are furtherly discussed.

# Discussion

## 2.1	Common for all Methodologies

#### PDSCH PMI Selection

Way Forward:

Random PMI already agreed. Regarding potential fixed PMI:

- Option 1: Random PMI from sub-set

- Option 2: Collectively chosen fixed PMI

- Option 3: Independently chosen fixed PMI

Note: Companies to propose detailed method description for interested options for RAN4#114-bis.

In some contribution to the RAN4#114 meeting [3, 5], it is observed that the spatial preference during a relative long time is typical in both simulations and field measurements. In that case, it is possible to use a fixed PMI configuration rather than random PMI to check the receiver performance under a specified (randomness reduced) spatial channel model. But the problem is how to align the PMI hypothesis if companies want to get alignment on the simulation results on it.

Based on current SU PDSCH simulation results from all companies, the number of fixed PMI results are very limited and there is a obvious gap among companies. To get alignment, the first thing could be the alignment on the PMI hypothesis. The motivation of Option 1 is to reduce the misalignment possibility by reducing the random PMI set. But the consequence might be that every channel model needs to consider PMI sub-set selection specially which makes the simulation and implementation complex. The second step could be collecting the preferred PMI from companies to check the gap which is indicated by Option 2, but the collection condition should be clearly indicated (SNR/throughput range etc.). Once the PMI histograms from companies are well overlapped, then it is possible to choose the most preferred PMI for step 3 simulation to get final alignment.

Proposal 1	Consider following steps for fixed PMI simulation alignment.

- Step 1: Align PMI hypothesis.

- Step 2: Align PMI histograms from companies under high SNR or high normalized throughput range (>=80%).

- Step 3: Choose the highest possibility PMI from aligned PMI histogram for performance alignment.

#### Cases for MU-MIMO

Agreement:

SU-MIMO to take priority over MU-MIMO.

Simulation alignment spreadsheet will only include agreed alignment cases.

The necessary of MU-MIMO SCM channel derivation was not discussed much in RAN4#114 meeting. Similar view as in our contribution [2], we think the MU-MIMO scenario in demodulation requirement perspective is more focusing on the interference profile rather than the channel model for aggressor. A typical RAN1 study method on MU-MIMO performance evaluation is to use independent 38.901 CDL models for victim and aggressors and assume the different angular separation between victim and aggressors (based on certain BS antenna configuration and feasible network scheduling PMI), see figure 2.1-1. Then the IRC performance on victim UE will be checked by a large set of condition combinations. The corresponding simulations are setup with multiple UEs and the interference is calculated by the channel correlation automatically. For a demodulation requirement, only one victim UE will be used in the final test and all interferences are inserted to the channel between BS and victim UE. Thus, how to model a proper interference profile is more important here. Furthermore, network assistant information, such as aggressor’s layer info, is also needed for some advance spatial receivers.

Figure 2.1-1	An example of RAN1 link level simulation for MU-MIMO

The point is that IRC or other advanced receiver should be checked by an interference with a certain power level with enough stochastics. The power level of interference could be defined by system level simulations which can show a typical result in a stochastic deployment. But the angular stochastics of interference is much trickier to have due to it is very hard to model all possible relative angular between victim antenna and aggressor antenna. It is not just dominant by the distance between victim and aggressors but also the network scheduling. By selecting specific locations victim and aggressor, it could be used for a simple function checking but might not be good for a requirement definition. The performance can’t be guaranteed by just a special interference angular.

MU-MIMO scenario for demodulation requirements is more focus on the interference profile than the channel model for aggressor UEs. The network assistant information would also play a role in some cases.

By selecting specific locations for both victim and aggressors, the stochastics will be lacking for the interference angular and not good for a requirement definition.

Proposal 2 	The study of MU-MIMO scenario could focus on the angular distribution model for interference profile rather than deriving channel models for a specified aggressor UE condition (e.g., fixed location and antenna direction).

#### Speed/Doppler

Way Forward:

For Comparison:

TDL Channels use 10 and 100Hz

CDL Channels use 3kph and 30kph

For Alignment:

TDL Channels use 10Hz

CDL Channels use 3kph

Based on the simulations in previous meeting [4], the performance difference with 827 model between 3kph and 30kph is small (less than 2dB). To make the achieved channel model from this SI could be reused in the future, the Doppler impact of the preferred model should be evaluated if we still have time after alignment and comparison. It could help companies to know how to define velocity for a certain SCM model according to target scenario.

Proposal 3 	Companies to check the Doppler impact on performance for aligned or preferred SCM models if there is time.

## 2.2 	TDL based methodologies

#### TDL Model Extensions

Way Forward:

Companies to use following two enhanced TDL channel for study.

- Option 1: Multi-cluster TDL channel with implementation details in R4-2500337:

  - For proposed simulation configurations see Annex C, Table C.1 and Table C.2.

  - For cluster powers settings there are several examples simulated in R4-2500335. We suggest trying 2 different setups

    - Un-even layer powers and even CW-powers.

    - Even layer powers and un-even CW-powers.

- Option 2: Multi-beams TDL channel with implementation details in R4-2501738:

  - 4T4R,4Layers: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table C.3

  - PMI test: Enhanced TDLC300-100, XPL, high with 2 Tx-Rx beams as defined in Table C.3

8T8R, 8Layers: Enhanced TDLC300-100, XPL, high with 4 Tx-Rx beams as defined in Table C.4 and 6 Tx-Rx beams as defined in Table C.5

The proposed TDL based extension methods were not discussed in previous meeting. The most important benefit of proposed methods is “convenience” claimed by companies since the partial of the model implementation is from multiple PMI test, but the most obvious limitation is that the total artificial configurations (including beam directions and power split) make the method less realistic. It should be noted that a more realistic model should be one of the most important aims for this study.

Current method uses orthogonal beam separation on both Tx and Rx sides, and the power split of each beam has no field measurement reference. It makes these methods more subjective on spatial relevant parameter setting than CDL approach and might be suitable for function check rather than performance verification.

To improve these methods, the further investigation on how to configure more realistic beam steering directions, power split, “tap-to-cluster” classification and correlation matrix impact should be needed. For example, evaluate the non-orthogonal beam steering impact on the performance, evaluate the relationship between correlation matrix and beam steering matrix, etc.

However, RAN1 team have maintained CDL model in TR38.901 for many years. Some parameters would be modified in Rel-19 regarding to the latest measurement results. In that case, focusing CDL for SCM study seems a more feasible way from a long run if companies can get alignment with it. Once the alignability can be approved in the study, there might be no reason to take new created TDL based method.

The proposed TDL extension methods are more subjective on spatial parameter setting than CDL approach and might be suitable for function check rather than performance verification.

Once the alignability of CDL base methods can be approved in the study, there might no reason to take new created TDL based method.

Proposal 4 	For TDL extension, interested companies should furtherly investigate how to improve the model to be more realistic, such as non-orthogonal steering directions, uneven power split, “tap-to-cluster” classification and correlation matrix impact etc.

Proposal 5 	Consider proposed TDL based methods only when companies approve CDL based model can’t be aligned.

## 2.3	CDL based methodologies

#### Angles of Departure and Angles of Arrival

Way Forward:

For all 4 direction combinations (AoA, AoD, azimuth, zenith) the following parameters need to be configured.

Angle spread scaling: ASdesired

Angle rotation: μϕ,desired

as a starting point.

The intention of using a “desired” mean angular rather than “model” mean angular is to have more flexibility on channel parameters adaptation. But the question here is how to choose a proper mean angular and what is the impact of this changing. For example, how to configure mean angular to have good test coverage during the PMI tests.

During the alignment stage, the original mean angular and AS in 827 model can be used. Once the alignment is complete, it could be possible to have more studies on the impact by different angular parameters setting.

It is not clear about the impact by configuring different mean angular and angular spread under a certain CDL model.

Proposal 6	RAN4 could furtherly study the performance impact by configuring different mean angular and angular spread on aligned CDL model if the time is enough.

#### SNR Definition – ‘long term average’

Agreement:

Normalization factor is constant over the duration of the simulation

#### SNR Normalisation with CDL

Way Forward:

Options:

- Option 1: Interested companies to provide information on long-term pre-EQ power measurements, for discussion whether a formula, or value, is required during RAN4#114-bis.

- Option 2: Define a normalisation method

  - Option 2a:

RAN4 to use following as normalization factor: C=NTxNRxnt=0NTx-1nr=0NRx-1E{Hnt,nr(k,l) 2} Where H(k,t)∈CNR×NT is channel coefficient at subcarrier k, OFDM symbol l. NRx is number of Rx and NTx is number of Tx ports. Hnt,nr(k,l) is the (nt, nr) element of H(k,l). E{Hnt,nr(k,l) 2} can be derived by long-term measurement in the simulation.

  - Option 2b:

Let normalized AAV beamformer vector of length Ms∙Ns (i.e. sub-array size) for one antenna port be α1,1⋯αMs,Ns=1GBF1Ms∙Nsexp-j2π∙θ1,1 ⋯ exp⁡(-j2π∙θMs,Ns) Here, amplitude normalization is separated into two factors - 1Ms∙Ns : split transmit power of each AP evenly over Ms∙Ns transmit antenna elements - 1GBF : normalize with measured transmit beamforming (power) gain GBF

In the previous meeting, the SNR normalization scaler factor is discussed, but there is no conclusion on this topic. There are two concepts should be distinguished at the first place: SNR normalization and AAV (Antenna Array Virtualization) gain normalization.

The definition of SNR used in RAN4 demodulation is as below:

The SI scope indicate that the test setup is conducted for FR1 and virtual cable for FR2. In that case, the fading channel is generated by channel emulator and added up to the transmitted signal. The SNR after channel filter could be no difference no matter TDL or CDL channel is used if the channel coefficients are normalized by the same way.

The SNR definition could be the same no matter TDL or CDL model is applied based on test setup (conducted for FR1 and virtual cable for FR2) in SI.

The actual relevant issue here is the AAV gain (or beamforming gain) introduced by AAV configurations. From the practical view, larger AAV would have better performance than no AAV configuration for MIMO scenario. Thus, the SNR of target performance would be different between AAV and no AAV configurations. One argument is that the RAN4 would keep same signal power at the receiver side and increase the AWGN level at the Tx side, so the performance can be compared regardless of Tx side antenna configuration. If we don’t want to have any AAV gain impact on the performance (which might lead to the similar performance of AAV enabled configuration as no AAV configuration especially on low layer transmission), the normalization could be considered. The reference could be no AAV configuration.

Based on our simulation analysis (increasing Tx signal power on top of constant noise power), The relationship of received signal power level among different AAV configurations is similar for different SNR levels. The averaged channel power per subcarrier is very similar among AAV configurations which is independent of SNR level, see Table 2.3-1.  The gain to AAV3 is lower than theoretical calculation by antenna elements number ratio.

Table 2.3-1 Long Term Channel and Received Signal Power Analysis

|  | SNR (dB) | 32Tx AAV 1Y |  | 16Tx AAV 1Y |  | 8Tx AAV3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | power | Gain to AAV3 | power | Gain to AAV3 | power | Gain to AAV3 |
| Avg received signal | 5 | 0.033 | 16.06 | 0.019 | 10.1 | 0.0019 | 1 |
|  | 15 | 0.303 | 16.05 | 0.191 | 10.1 | 0.019 | 1 |
|  | 30 | 9.58 | 16.05 | 6.092 | 10.21 | 0.597 | 1 |
| Avg channel power per subcarrier | Independent of SNR | 0.89 |  | 0.89 |  | 0.85 |  |

Observing the options in WF, it seems Option 2b is easier for application without changing the SNR definition. It needs extra scaling on channel model part to keep no precoding gain in received signal. But the factor should be aligned by simulations for a CDL channel with AAV configuration.

Based on our simulation results (without AAV gain normalization), the larger antenna array normally has better performance which is aligned with common sense. But the performance gain will be migrated more or less once the AAV gain normalization is added, especially for higher layer transmission. Comparing to no AAV configuration, same beam gain but much narrower beam width might decrease the performance. For layers transmitted by the relative week directions, the impact would be more obvious especially in low or medium SNR range.

The AAV gain normalization could be considered in virtualization stage to avoid changing the SNR definition.

Proposal 7 	Take Option 2b for AAV gain normalization in channel generation stage without changing definition of SNR in RAN4.

By AAV gain normalization, the performance might be degraded to be worse than no AAV configuration in some cases which is not aligned with expectation.

Proposal 8 	RAN4 to check the performance impact with/without AAV gain normalization after result alignment.

## 2.4	Comparison of methodologies

#### TDL and CDL Methodologies

Way Forward:

Options:

CDL from TR 38.753 will include randomness reduction for the purpose of the SI

- CDL (TR 38.753)

  - Option A1 : with Table 7.2.1-8 in TR 38.827

  - Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation

  - Option A3 : based on Table 7.7.1-3 in TR 38.901 with truncation 
According to the updated truncation procedure described in Annex B, truncated table to be shared before RAN4#114-bis.

- Option B1: TDL (MediaTek)

- Option B2: TDL (Huawei)

When we are talking about the comparison among methodologies, the metrics should be clarified at the first. Based on the discussions in previous meetings and the options in the WF above, following metrices could be considered for between TDL and CDL approaches, and also among different options within the same approaches:

Metrics between TDL and CDL approaches

To compare pros and cons of TDL and CDL approaches, companies have some discussion on it but don’t achieve any agreement. For our understanding, the comparison here should not only focus on the performance since different approaches have its own physical nature and hard to have a fair comparison. Furthermore, it should be careful to compare performance without the pre-condition of realistic. It will be dangerous to use a less realistic model but with better performance. Other parts, such as alignability, practicability, complexity and testability, could be used as the comparison metrices.

Alignablity is to check whether companies can get alignment on implementation by reviewing the simulation curves. Practicability is to check the channel characters are close to real network deployment. Complexity and testability are more relevant to TE implementation and to check the effort of channel parameter changing and the model profile limitations. Companies should make some conclusion on these aspects to give a highlevel view of different approaches.

Proposal 9	Consider alignability, realistic, practicability and testability as metrics for TDL and CDL approaches comparison.

As for the realistic part, the field measurement results can be referred. Ericsson and BT have delivered some channel character analysis on AoA, eigen value and PMI preference based on field measurement [5, 6, 7].

The eigen value distribution from field measurement (in Figure 2.4-1 (a)) shows LOS distribution for Location A and C due to the more dominant mode-1 value. While the eigen value distribution from 38.827 FR1 Uma CDLC model with different AAV configuration shows that eigen value mode are getting similar when increasing antenna array which means that the channel selective fading gets better. The gap between different eigen modes is smaller than field results indicate that the channel correlation in simulation is higher than field channel.

Figure 2.4-1 Eigen value distribution (a) Field measurement; (b) 38.827 FR1 Uma CDLC model

As for the PMI preference, contribution [7] proves that it could be a typical thing in the field. It could be used as a metric to compare the realistic of channel model as proposed by contribution [7].

Proposal 10 	Consider eigen mode and PMI preference into the model comparison metrices.

Metrics among options within TDL or CDL approach

Under a same approach, once companies confirm that the initial methodology is correct, then the comparison of models could more focus on improvement of performance and complexity to meet the test expectation.

Since different test cases would be expected to have different performance, the performance improvement here should depend on a certain test case. In that case, the pre-condition of this kind of comparison is that companies have agreed that the initial model is realistic and practical for this case. The better performance and less complexity is preferred, and some tradeoff might be needed based on the simulations.

Proposal 11	Consider performance and complexity as metrics for model comparison within same approach only after the initial model is concluded as realistic and practical for target test case.

#### Antenna Array Virtualisation (for CDL options)

Way Forward:

Downselection to be based upon (1. Best performance, 2. Ease of alignment, 3. Practicality)

Options:

- Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration

  - [(M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only]

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

The antenna array has very huge impact on demodulation performance especially on MIMO with large fading scenario. To have better performance, the large antenna array is expected. But a tricky question is how to do AAV configuration and CSI-port mapping. From the demodulation perspective, the AAV is not used with legacy TDL channel model but using correlation matrix to represent the general antenna impact. When we come to CDL approach, we have to deal with the complexity caused by AAV.

The first question is whether to consider AAV for CDL?

In the previous discussions, some companies argued that no need to use AAV due to three reasons:

- Hard to compare with legacy TDL requirements.

- Different antenna array configurations would lead to different performance results.

- It is not clear how to map CSI-RS ports to higher number of subarrays.

It is true that legacy TDL requirement don’t use AAV, but it is strange to compare CDL model performance to TDL model performance due to totally different approaches. Once companies could get aligned with CDL model + AAV configuration, the performance gain between CDL model and legacy TDL model is also aligned. In that case, it would be no issue to apply it for the requirement.

As for the CSI-RS port mapping, it is also true that the different companies might use different implementations. Again, it would not be an issue once the method is aligned among companies. The remaining issue is how to do mapping especially when there are more subarrays than CSI-ports. The “full connected” method used in 38.827 is a common way for all AAV configurations which simplify the CSI mapping issue. However, it is not practical in real product unfortunately. On the other hand, it seems OK for the DL demodulation requirement definition with “full connected” once it can be aligned because it can generalize the mapping.

Large AAV is necessary for high layer transmission for fading channel, but the received signal power increasement by AAV gain might be not aligned with SNR definition in RAN4 demodulation.

Although “full connected” method is not practical, this method could generalize the CSI port to AAV mapping issue.

The second question is how to choose a proper AAV configuration?

It would depend on the channel model characters and specific test case. Only one column antenna elements are considered for current subarray virtualization which is aligned with practical implementation. Regarding the array configuration, if only one subarray in vertical direction could be more investigated according to channel model. For example, more vertical subarray might be needed if channel model has limitation on horizontal strong paths for transmission. On the other hand, more horizontal subarray columns should be considered if more layers of transmission on horizontal plan is needed.

Regarding the 827 FR1 Uma CDLC model which the strongest beam directions are along the horizontal plane, it could be better to use more horizontal subarrays than vertical ones. But if other channel model is used, such as 901 CDLC model with randomness reduction, the proper antenna array might be square like to have better freedom balance in both vertical and horizontal dimensions. In our contribution for RAN4#114 [5], different antenna array configurations are compared under 38.827 FR1 Uma CDLC model and it shows that the more horizontal subarrays can improve the performance.

In real network implementation, more aspects would be considered for AAV configuration, such as power consumption, size of radios, to meet EEIRP etc. In RF study, ITU-R configurations for antenna are referred for simulations. It could be also referred for SCM study if companies want to have more practical model.

The AAV configuration could depend on the test case expectation and channel model characters if we want to have relative better performance.

ITU-R antenna configuration used by RF study can be referred in SCM study if companies want to have more practical model.

The option 1Y in WF list three AAV for different CSI ports. One reason behind of this is to simplify the port mapping. It is true that the CSI-port mapping to physical antenna array is tricky and depending on vendors implementation. But the problem is this kind of simplification might decrease the performance, such as performance of AAV 1Y (with gain normalization) is worse than AAV 3. This can be seen from many companies’ simulation results and could be an argument that why AAV 1Y is needed.

The performance of AAV 1Y with gain normalization is worse than AAV 3 in many companies’ simulation results.

Proposal 12 	For simulation results alignment to check CDL channel model implementation, RAN4 could start from no AAV configuration.

Proposal 13	After alignment on no AAV configuration, RAN4 could study a general method for CSI port to large antenna array mapping to have better performance according to test case expectation.

#### Performance Comparison Methodologies

Way Forward:

Companies welcome to bring views at next meeting

#### Channel Properties Comparison Methodologies

Way Forward:

Companies welcome to bring views at next meeting.

#### Metric for PMI tests comparison

Way Forward:

Options:

- Option 1: Gamma value to random PMI configuration.

- Other options are not preconcluded.

#### SNR Throughput Percentage measurements (for PDSCH)

Way forward:

At least 70% and 30% for alignment of PDSCH and interested companies can provide full curves if desired.

Simulation results collection excel sheet format will be updated accordingly in the next meeting allowing to capture input SNR values for target % of the max throughput for PDSCH alignment.

As we discussed in previous topics, the metrics listed here could be for the comparison within same approaches. In that case, the performance and complexity could be used. The performance comparison could be based on the legacy metrics, such as Gamma value for PMI and 70% normalize throughput for PDSCH.

Channel properties could be used to compare both extension approaches (TDL vs CDL) and models within same approach. The former is preferred since it can be used to compare with real measurement in field. Some properties are proposed by companies in RAN4#114, such as eigen values, angular power distribution, etc. They could be the start point for comparison in study.

Proposal 14 	The performance comparison after implementation alignment under same model approach (TDL or CDL) could start from legacy metric, such as Gamma value for PMI tests and 70% (and 30%) normalized throughput for PDSCH tests.

Proposal 15 	Channel properties comparison could be used for TDL and CDL approach comparison referring to real measurement results. It could start from eigen value and angular distribution.

# Conclusions

In the previous sections we made the following observations:

Observation 1	MU-MIMO scenario for demodulation requirements is more focus on the interference profile than the channel model for aggressor UEs. The network assistant information would also play a role in some cases.

Observation 2	By selecting specific locations for both victim and aggressors, the stochastics will be lacking for the interference angular and not good for a requirement definition.

Observation 3	The proposed TDL extension methods are more subjective on spatial parameter setting than CDL approach and might be suitable for function check rather than performance verification.

Observation 4	Once the alignability of CDL base methods can be approved in the study, there might no reason to take new created TDL based method.

Observation 5	It is not clear about the impact by configuring different mean angular and angular spread under a certain CDL model.

Observation 6	The SNR definition could be the same no matter TDL or CDL model is applied based on test setup (conducted for FR1 and virtual cable for FR2) in SI.

Observation 7	The AAV gain normalization could be considered in virtualization stage to avoid changing the SNR definition.

Observation 8	By AAV gain normalization, the performance might be degraded to be worse than no AAV configuration in some cases which is not aligned with expectation.

Observation 9	Large AAV is necessary for high layer transmission for fading channel, but the received signal power increasement by AAV gain might be not aligned with SNR definition in RAN4 demodulation.

Observation 10	Although “full connected” method is not practical, this method could generalize the CSI port to AAV mapping issue.

Observation 11	The AAV configuration could depend on the test case expectation and channel model characters if we want to have relative better performance.

Observation 12	ITU-R antenna configuration used by RF study can be referred in SCM study if companies want to have more practical model.

Observation 13	The performance of AAV 1Y with gain normalization is worse than AAV 3 in many companies’ simulation results.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Consider following steps for fixed PMI simulation alignment.

	Step 1: Align PMI hypothesis.

	Step 2: Align PMI histograms from companies under high SNR or high normalized throughput range (>=80%).

	Step 3: Choose the highest possibility PMI from aligned PMI histogram for performance alignment.

Proposal 2 	The study of MU-MIMO scenario could focus on the angular distribution model for interference profile rather than deriving channel models for a specified aggressor UE condition (e.g., fixed location and antenna direction).

Proposal 3 	Companies to check the Doppler impact on performance for aligned or preferred SCM models if there is time.

Proposal 4 	For TDL extension, interested companies should furtherly investigate how to improve the model to be more realistic, such as non-orthogonal steering directions, uneven power split, “tap-to-cluster” classification and correlation matrix impact etc.

Proposal 5 	Consider proposed TDL based methods only when companies approve CDL based model can’t be aligned.

_Toc194093633

Proposal 6	RAN4 could furtherly study the performance impact by configuring different mean angular and angular spread on aligned CDL model if the time is enough.

Proposal 7 	Take Option 2b for AAV gain normalization in channel generation stage without changing definition of SNR in RAN4.

Proposal 8 	RAN4 to check the performance impact with/without AAV gain normalization after result alignment.

Proposal 9	Consider alignability, realistic, practicability and testability as metrics for TDL and CDL approaches comparison.

Proposal 10 	Consider eigen mode and PMI preference into the model comparison metrices.

Proposal 11	Consider performance and complexity as metrics for model comparison within same approach only after the initial model is concluded as realistic and practical for target test case.

Proposal 12 	For simulation results alignment to check CDL channel model implementation, RAN4 could start from no AAV configuration.

Proposal 13	After alignment on no AAV configuration, RAN4 could study a general method for CSI port to large antenna array mapping to have better performance according to test case expectation.

Proposal 14 	The performance comparison after implementation alignment under same model approach (TDL or CDL) could start from legacy metric, such as Gamma value for PMI tests and 70% (and 30%) normalized throughput for PDSCH tests.

Proposal 15 	Channel properties comparison could be used for TDL and CDL approach comparison referring to real measurement results. It could start from eigen value and angular distribution.

# References

[1] 	R4-2502378, Way Forward for [114][322] FS_NR_demod_SCM, Nokia, RAN4#114

[2]	R4-2504382, Simulation results for NR SCM for demodulation, Ericsson, RAN4#114bis

[3]	R4-2501787, Discussion on Rel-19 SCM methodology for demodulation, Ericsson, RAN4#114

[4]	R4-2501788, Simulation results for Rel-19 SCM for demodulation, Ericsson, RAN4#114

[5] 	R4-2501745, Measurement results for MIMO channels in band n78, BT, Ericsson, RAN4#114

[6]	R4-2419338, Further observations on the properties of MIMO channels in typical deployments, BT, Ericsson, RAN4#113

[7]	R4-2504431, PMI statistics for measured MIMO channels in band n78, BT, Ericsson, RAN4#114bis