3GPP TSG-RAN WG4 Meeting #114bis	R4-2504385

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.10.2.2.2

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN HPUE RX RSD

Document for:	Approval

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable PC2/PC1 high power UE (HPUE) with defining RF requirements targeting at support of IoT-NTN UEs for FR1 single carrier scenario.

In this contribution, our evaluation about HPUE RX reference sensitivity degradation (RSD) requirements for IoT-NTN LTE bands is provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

2.2		Previous Meeting Agreements

Based on the WID’s objective, it indicates that it is necessary to evaluate whether RX RSD requirements would be needed when TX HPUE is enabled under FD-FDD operation.  In RAN4#114 meeting, the agreed WF [2] provided the guidance for following works. The partial contents are listed below for reference.

RAN4#114 Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based) - Proposals - 256: - 0 dB (vivo, oppo, Samsung, Ericsson) - 1.1 dB (Qualcomm) - 255: - 0 dB (oppo) - 1.2 dB (Qualcomm) - FFS (vivo) - 254: - 0 dB (Samsung, Ericsson) - WF (Ad-hoc agreement) - 256: FFS - 255: FFS - 254: 0dB Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based) - Proposals - Option 1: 0dB for n254/n255/n256 (Ericsson,…) - Option 2: non-zero value (MediaTek) - Option 3: others - WF (Ad-hoc agreement) - Keep earlier agreement [0] dB. - Further study needed

2.3		UE RF requirements

2.3.1	IoT-NTN PC2(1Tx) UE Cat-M1 RX RSD for B256, B255, and B254

According to our evaluation, because of high TX-RX duplex spacing frequency, no RX RSD is needed for IoT-NTN B256 and B254 when Cat-M1 UE enables PC2 TX. Regarding IoT-NTN B255, the RX RSD is needed when UE enables PC2 TX.

Proposal 1: Regarding IoT-NTN Cat-M1 UE operated in PC2 TX, discuss whether the FD-FDD RX RSD values as shown in table below could be used for B256 and B255.

Table 1: IoT-NTN Cat-M1 RSD from PC3 to PC2 for FD-FDD UE

| NTN Band | 1.4 MHz (dB) |
| --- | --- |
| 256 | 0 |
| 255 | 0.8 |

2.3.2	IoT-NTN PC1(1Tx) non-handheld UE Cat-M1 RX RSD for B256, B255, and B254

Proposal 2: Regarding IoT-NTN Cat-M1 UE operated in PC1 TX, discuss whether the FD-FDD RX RSD values as shown in table below could be used for B256, B255, and B254.

Table 2: IoT-NTN Cat-M1 RSD from PC3 to PC1 for FD-FDD UE

| NTN Band | 1.4 MHz (dB) |
| --- | --- |
| 256 | [3.7] |
| 255 | [4.7] |
| 254 | [3.7] |

3 Conclusion

In this paper, our views on IoT-NTN FD-FDD UE Cat-M1 RX RSD are provided.

Proposal 1: Regarding IoT-NTN Cat-M1 UE operated in PC2 TX, discuss whether the FD-FDD RX RSD values as shown in table below could be used for B256 and B255.

Table 1: IoT-NTN Cat-M1 RSD from PC3 to PC2 for FD-FDD UE

| NTN Band | 1.4 MHz (dB) |
| --- | --- |
| 256 | 0 |
| 255 | 0.8 |

Proposal 2: Regarding IoT-NTN Cat-M1 UE operated in PC1 TX, discuss whether the FD-FDD RX RSD values as shown in table below could be used for B256, B255, and B254.

Table 2: IoT-NTN Cat-M1 RSD from PC3 to PC1 for FD-FDD UE

| NTN Band | 1.4 MHz (dB) |
| --- | --- |
| 256 | [3.7] |
| 255 | [4.7] |
| 254 | [3.7] |

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- R4-2503006, “WF on HPUE UE requirement for IoT-NTN”, Vivo