3GPP TSG-RAN WG4 Meeting # 114-bis														R4-2404389

Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.1.1.3.4

Source:	Samsung

Title:	On SRS IL imbalance issue

Document for:	Discussion

1	Introduction

As one of the objectives, ”6Rx for handheld and FWA UE” under Rel-19 WI on UE RF enhancements for NR FR1/FR2 and EN-DC (Phase 4) [1], the issue of insertion loss imbalance across SRS ports has been discussed to study whether the corresponding solution is required or not. However, despite RAN4 spending considerable time trying to find a solution for the issue from Rel-18, the same question and outcome have been repeated over the last year such as whether to solve SRS IL imbalance issue in Rel-19 or not [2-7]. In this contribution, we would like to continue sharing our views while trying to conclude this unproductive discussion.

2 Discussion

The latest WF [7], like previous ones, consists of two parts [2-6], one for seeking its original justification on whether to solve SRS IL imbalance issue in Rel-19, the other is to try a compromised solution with the least impact to RAN1, RAN2, and RAN4 specifications as below.

Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1. Issue 3-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1. Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: RAN4 to take the candidate solution framework below as baseline for further discussion. Candidate solution framework: - Rel-19 UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on RAN4 specification impacts. - No RAN1 specification impact. - FFS if UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others

However, since the two sub-topics are not mutually independent, RAN4 should make a decision on the solution considering the aspect of its motivation and practical impact on the network, before thinking about a compromise way. Otherwise, the arguments for the compromise solution will continue without an outcome from Rel-18.

Observation 1:	RAN4 should make a decision on the solution considering the aspect of its motivation and practical impact on the network, before thinking about a compromise way.

Even though RAN4 can compromise with the least and simplest method in the end, in our understanding, it remains unclear how the network can recognize the SRS power gap whether it is from IL imbalance or from others, and how UEs can estimate the insertion loss value dynamically as it is not from their intention. This is because the gap between SRS ports is more dependent on the channel fading, hand grip loss, or other issues than IL. These fundamental issues cannot be removed since various underlying factors such as path differences between antennas and LPAF (LNA/PA/Filter/switch) persist affecting the imbalance even though some IL imbalance related reporting schemes or capabilities are introduced expecting the better network behavior. It should also be noted that the SRS power headroom reporting for an activated serving cell, i.e., Type 3 PHR, can be happened only if the UE is not configured for PUSCH transmissions on the serving cell, which mean that the SRS power headroom reporting with the imbalance information for the network would not happen directly nor dynamically as specified in TS 38.213.

Observation 2:	The gap between SRS ports is more dependent on the channel fading, hand grip loss, or other issues than IL. These fundamental issues cannot be removed since various underlying factors such as path differences between antennas and LPAF (LNA/PA/Filter/switch) persist affecting the imbalance even though some IL imbalance related reporting schemes or capabilities are introduced expecting the better network behavior.

Observation 3:	SRS power headroom reporting, i.e., Type 3 PHR, with the imbalance information for the network would not happen directly nor dynamically.

Then, it would give the same question about the purpose of providing assistance to the network on SRS IL compensation, e.g., New PHR or UE capability, if it cannot make any change of the SRS imbalance issue. The potential solution with the assistance information to the network for the self-compensation of SRS IL would not help improving accuracy of the downlink channel estimation.

Observation 4:	Introducing new capability for IL imbalance reporting would not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Nevertheless, we understand that RAN4 is now in a deadlock and it is time to close the time-consuming discussion with a clear conclusion. According to the latest discussion, it could see that UE could perform self-compensation of SRS IL up to its configured maximum output power for Rel-19. Having the solution without any impacts to RAN1, RAN2, and RAN4 specifications can be a good compromise given that only 1-2 meetings are left for Rel-19. The fundamental issue of SRS raised so far could be discussed in future releases alongside other SRS issues in the power domain.

Proposal 1:		As a compromise, UE could perform self-compensation of SRS IL up to its configured maximum output power without any impacts to RAN1, RAN2, and RAN4 specifications in Rel-19.

Proposal 2:		The fundamental SRS issue raised so far could be discussed in future releases alongside other SRS issues in the power domain.

3	Conclusion

In this contribution, we continued to provide our analysis and view on the proposed solutions. Following observations and proposals are provided to be considered by this meeting.

Observation 1:	RAN4 should make a decision on the solution considering the aspect of its motivation and practical impact on the network, before thinking about a compromise way.

Observation 2:	The gap between SRS ports is more dependent on the channel fading, hand grip loss, or other issues than IL. These fundamental issues cannot be removed since various underlying factors such as path differences between antennas and LPAF (LNA/PA/Filter/switch) persist affecting the imbalance even though some IL imbalance related reporting schemes or capabilities are introduced expecting the better network behavior.

Observation 3:	SRS power headroom reporting, i.e., Type 3 PHR, with the imbalance information for the network would not happen directly nor dynamically.

Observation 4:	Introducing new capability for IL imbalance reporting would not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Proposal 1:		As a compromise, UE could perform self-compensation of SRS IL up to its configured maximum output power without any impacts to RAN1, RAN2, and RAN4 specifications in Rel-19.

Proposal 2:		The fundamental SRS issue raised so far could be discussed in future releases alongside other SRS issues in the power domain.

Reference

[1] RP-241656	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

[2] R4-2406585	WF on requirements for 6Rx		RAN4#110-bis

[3] R4-2410751	WF on requirements for 6Rx		RAN4#111

[4]	R4-2414285	WF on 6Rx UE requirements		RAN4#112

[5]	R4-2417209	WF on 6Rx UE RF requirements	RAN4#112-bis

[6]	R4-2420376	WF on 6Rx requirements			RAN4#113

[7]	R4-2503014	WF on 6Rx requirements			RAN4#114