# 3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504392

# Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			TP to TR38.774: BS RF requirement overview for LP-WUS

Agenda item:		7.26.3

Document for:		Approval

# Introduction

In this paper, we provide our view on BS RF requirement impact for LP-WUS.

# Discussion

In WF[1], for power boosting, the WF is quoted as below:

## 1.1 Concept of LP-WUS power boosting and associated detail

- Adopt following content as the concept of LP-WUS power boosting.

  - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

- Clarify the declaration on the support of LP-WUS and support of LP-WUS power boosting should be separate.

- The minimum power boost level is included in the BS manufacturer declaration table, but not defined as a minimum requirement.

  - FFS on the detail of the minimum power boosting level, e.g.

    - Whether it can be different for different BS CBW

As it is declaration based, the power boosting level and CBW associated with power boosting should be also declarable. For example, the declaration text in 38.141-1 could be specified as below:

| D.x | LP-WUS/LP-SS power dynamic range | If the BS supports LP-WUS/LP-SS operation, manufacturer can declare the minimum [+X] dB of LP-WUS/LP-SS power boosting level it could support together with the channel bandwidth. The LP-WUS/LP-SS power boosting is defined as follows: The LP-WUS/LP-SS power boosting is the difference between the average power of LP-WUS/LP-SS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS/LP-SS and the NR carrier containing the LP-WUS/LP-SS REs). |
| --- | --- | --- |

The above declaration text based on WF to be agreed in conformance test.

## 1.2 Consideration on EVM requirement for LP-WUS

- For RAN4 simulation campaign and discussion in next meeting, the latest RAN1 agreements on waveform design are expected to be used.

- FFS on the following options, others are not precluded.

  - Option 1: No need to specify transmit signal quality requirement for LP-WUS.

  - Option 2: Instead of defining transmit signal quality requirement for LP-WUS, just do the verification e.g. utilizing a higher-order EVM test (64QAM) for NR with embedded LP-WUS.

RAN1 also agrees on the processing for the OOK signal and this is illustrated in Figure 3.

Figure 1: OOK-4 signal processing and subcarrier mapping before IFFT

Figure 2: OOK1 subcarrier mapping

Figure 3: ZC sequence in time and frequence domain.

- With OOK-4 (DFT-s or waveform fitting approaches) the gNB may need to support additional DFT (or another transform for LS approach) or pre-generate and store frequency domain symbols. This is illustrated in Figure 6.

Figure 4: OOK-4 subcarrier mapping

- After the DFT transform, the OOK-4 frequency domain pattern is illustrated in Figure 7. Such constellation pattern does not match any existing modulation schemes.  The measurement point for signal quality is defined after FFT according to Figure B.1-1 in TS 38.104. If this measurement point would be kept for signal quality measurement of OOK-4, RAN4 should discuss if a test with frequency domain data generated with a fix time domain OOK-4 pattern using procedure defined by RAN1 should be defined.

RAN4 discuss whether to define a frequency domain testing data for OOK-4 if the measurement point for signal quality would be kept.

According the current WID objective, the below objective still in line with the spirit of the study phase and keep the RF impact as min as possible.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Figure 5: ZC sequence in frequency domain after DFT processing before IFFT

Figure B.1-1 [3]: Reference point for EVM measurement

- Considering WID objective and keeping the min specification impact for OOK-4, we suggest to testing the tolerance of the distortion of the frequency domain data of OOK-4 by remapping the frequency constellation of overlaid ZC sequence to one of existing modulation scheme. This will mimic the distortion of the signal by the noise from hardware. As one example, for OOK4 waveform, the higher modulation (e.g 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 8) to 64QAM constellation points (right constellation in  Figure 8) for CP-OFDM symbols as illustrated in Figure 8. Figure 9 and Figure 10 illustrates the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping, It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

The performance loss for LP-WUS signal detection at WUR due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

From the distortion introduction by constellation re-mapping, it can be observed that the time domain signal quality of OOK-4 is quite robust and can be maintained without big degradation of SNR when the frequency domain OOK-4 data is distorted by the amount of EVM equivalent to the 64QAM. The frequency domain data of OOK-4 is degraded if it is distorted by the amount of the EVM equivalent to the 16QAM.

The amount of noise injected into OOK-4 signal in frequency domain after DFT can be tolerated at WUR if the noise level is equivalent to the minimum requirement of 64QM at base station.

Figure 6:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 7: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 8: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

The 64QAM modulation is mandatory tested in 38.141-1 and therefore, the LP-WUS signal quality test can be dependent on the PDSCH 64QAM and there is no need to introduce additional test. In the core specification, the LP-WUS signal quality requirement should be specified as this is new modulation scheme in Rel-19. The suggested wording in clause 6.5.2.2 as below:

The OOK4 signal quality test can be dependent on the 64QAM EVM test.

#### 6.5.2.2	Minimum Requirement for BS type 1-C and BS type 1-H

For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2.

Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for LP-WUS | Required EVM |
| --- | --- |
| OOK-4 | 8 % |

Some company last meeting claims the PAPR may increase for combined WUS and NR signal and want to introduce the test for this case. To investigate this issue, maybe more time is needed for LP-SS design. The unbalanced OOK is discussed in RAN1 for LP-SS and RAN4 can wait further RAN1 progress on this.

Wait further RAN1 progress on PAPR discussion.

TP starts here:

### 7.2.4	Transmitted signal quality

It is agreed that for overlaid OFDM sequences is designed for LP-WUS and the time sequence for overlaid OFDM symbol is ZC sequence. This applies both for OOK-1 and OOK-4. RAN1 also agrees on the processing for the OOK signal and this is illustrated in Figure 7.2.4-1. After the DFT transform, the OOK-4 frequency domain pattern is illustrated in Figure 7.2.4.2.

Figure 7.2.4-1: OOK-4 signal processing and subcarrier mapping before IFFT

Figure 7.2.4-2: ZC sequence in frequency domain after DFT processing before IFFT

- The tolerance of the distortion of the frequency domain data of OOK-4 is simulated by remapping the frequency constellation of overlaid ZC sequence to its nearest existing modulation constellation scheme. For example, for OOK4 waveform, the higher modulation (e.g 64QAM) than QPSK may be used to map the frequency domain OOK4 symbols (left constellation in  Figure 7.2.4-3) to 64QAM constellation points (right constellation in Figure 7.2.4-3) for CP-OFDM symbols as illustrated in Figure 7.2.4-3 . Figure 7.2.4-4 and Figure 7.2.4-5 illustrate the initial LLS result to evaluate the impact of OOK and OFDM sequence detection due to the remapping. It can be observed that the performance loss of such mapping on WUS detection performance (OOK and OFDM) is around 0.2 dB when mapping to regular data structure generated from 64QAM and around 0.6 dB when mapped to 16QAM.

- It is observed that the amount of noise injected into OOK-4 signal in frequency domain after DFT can be tolerated at WUR if the noise level is equivalent to the minimum requirement of 64QM at base station. When the noise level is equivalent to to minimum requirement of 16QAM, such distortion can not be tolerated at WUR.

Figure 7.2.4-3:Constellation mapping for multi-bit OOK WUS (4-bit OOK case mapped to 64QAM).

Figure 7.2.4-4: Impact of mapping WUS constellation to existing QAM on OOK detection performance.

Figure 7.2.4-5: Impact of mapping WUS constellation to existing QAM on OFDM sequence detection performance.

# Conclusions

In this contribution, we present our view on the BS RF requirement with below observations and proposals:

Observation 1 RAN4 discuss whether to define a frequency domain testing data for OOK-4 if the measurement point for signal quality would be kept.

Observation 2 WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Observation 3 The performance loss for LP-WUS signal detection at WUR due to transmitter remapping constellation to existing QAM constellation is around 0.2 dB for 64QAM remapping and 0.6 dB for 16QAM remapping.

Observation 4 The amount of noise injected into OOK-4 signal in frequency domain after DFT can be tolerated at WUR if the noise level is equivalent to the minimum requirement of 64QM at base station.

Proposal-1: The above declaration text based on WF to be agreed in conformance test.

Proposal-2:The OOK4 signal quality test can be dependent on the 64QAM EVM test.

Proposal-3:Wait further RAN1 progress on PAPR discussion.

# References

- R4-2502305, Way Forward for [114][308] NR_LPWUS, Huawei

- 38.141-1, Base Station (BS) conformance testing Part 1: Conducted conformance testing

- 38.104

- R4-2502092, TP to TR38.774: BS RF requirement overview for LP-WUS, Ericsson