3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504397

Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			WUR RF requirement of REFSESN, ASC, ASCS

Agenda item:		7.26.2.2

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of REFSENS, ASC, ASCS.

# Discussion

### FR2

Issue 2-2-1: Combining strategy for FR2 LP-WUR

Agreements:

  - For Rel-19, confirm no combination gain across two Rx chains is considered for FR2 LP-WUR. FFS whether and how to capture the impacts of Rx selection in REFSENS evaluation.

  - Confirm 1Rx at baseband for FR2 LP-WUR.

Issue 2-2-3: Spherical coverage for FR2 LP-WUR

Way forward:

  - RAN4 further discuss how to specify spherical coverage requirements for FR2 LP-WUR.

In legacy NR FR2 EIS requirement discussion, the diversity gain is set to 0. The reason is that for the same beam, the total power of power projected to theta and phi direction is the same and therefore there is no diversity gain for it. From the test perspective, only one polarization test beam is transmitted. In one case, if one polarization antenna at DUT is matched to test beam but totally mismatched with another polarization antenna (the angle between radiated electric fields of received beam and transmitted beam is 0 for one polarization antenna but 90 degree for another polarization antenna), the total power will be received at single receiver branch but there will be no diversity gain for it. For a case where the angle between radiated electric fields of received beam and transmitted beam, the projected power for both theta and phi will be -3 dB but with the diversity gain of 3 dB, the total is 0.

For WUR, as no combination gain is assumed in Rel-19, for the polarization matched case where one polarization antenna matched to polarization of the test beam, the diversity gain is 0. For a case where the angle between radiated elctric fields of received beam and transmitted beam is 45 dgree, the projected power for both theta and phi will be -3 dB and diversit gain is -3 dB as no combination gain is assumed.

Diversity gain should be -3 dB accounting the 0 combination gain in EIS test.

In theory, there is combination gain using selection also and in [2], the selection combination gain is observed. Therefore, 2 Rx should not be excluded in implementation as potential EIS improvement in future release. Afterall, the combination gain is assumed to be zero so if WUR implement 1 Rx, it has no difficulty to meet the requirment anyway. 2 Rx will be relevant to the combination gain other than zero.

both 1 and 2 Rx can be assumed in Rel-19 when combination gain is assumed to be zero.

For spherical coverage, as the coverage match between WUR and MR is the same with FR1, the spherical rank should be the same between WUR and MR for the same power class. In this sense, the EIS can be different percentile of the EIS requirment of main receiver, we think the spherical coverage percentile of min EIS should be the same between MR and WUR.

The coverage matching requires the same spherical coverage percentile of WUR and MR.

### ASCS

Issue 2-3-2: in-band power level of ASCS Test case

Agreements:

- For ASCS, allowable de-sense due to introduction of NR RBs is [0-1dB].

- [0] guard RB is the baseline.

Issue 2-3-3: channel bandwidth of ASCS Test case

Agreements:

- For ASCS core requirements, it should be applied to all NR CBW.

- For conformance test, for NR CBW can be further discussed and down-selected.

- For ASCS test configuration, the NR RBs should be fully allocated with the same PSD of LP-WUS within the channel except for 11 LP-WUS RBs and potential guard RBs (if any).

In WF [1], it is agreed to use 0 RB as guard RB between LP-WUS and NR signal. This means the SNR degradation caused by adjacent NR signal is neglectable. The NR signal is simulated with the same PSD with the wanted signal. This means the ASCS selectivity is 0 dB. As discussed by one company, the wanted signal level can be set as REFSENS. The NR signal adjacent to the wanted signal power will be set according to the same PSD of LP-WUS signal and also the # of the RB that NR signal should be transmitted. This means there is scaling factor of the 10log (# RB of NR signal / # of RB of LP-WUS). Other testing parameter could be the channel bandwidth of the MR and location of the LP-WUS signal RB.  The testing parameter is listed in Table 2 below. The x dB can be set for a case with a small adjacent interfere and for big adjacent interferer case (maximum input power). This can be discussed further with the maximum input power requirement.

ASCS can be tested with both small adjacent interferer and big interferer case with selectivity of 0 dB.

Table 2: Testing parameter for ASCS

| Wanted signal | REFSENS + [ x dB] |
| --- | --- |
| NR signal | REFSENS + 10log (# RB of NR signal / # of RB of LP-WUS) + [x dB] |
| NR channel bandwidth | 5MHz |
| LP-WUS RB allocation | # RB adjacent to the RB of NR signal |

### ACS

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB

Agreements

- Same interference signal level as MR for LP-WUR ACS requirements

  - FFS at least for one set of Rx requirements.

- The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point

  - FFS whether two sets of LP-WUS wanted signal level can be defined.

- Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation

- FFS number of guard RBs.

For the interference level, the same interference signal level as MR for LP-WUR ACS requirements should be set for both set of the Rx requirement. Otherwise, the WUR becomes a dedicated receiver and decoupled with MR and coexisting simulation will be needed.  During the study phase, the guard RB is simulated to relax the ACS requirmeent. The number of guard RB would be different between two set of Rx requriment. As the LP-WUS level using the LP-WUS REFSENS and for the WUR with worse NF, the LP-WUS REFSENS would be higher (18 dB vs 13.5 dB), this will lead to 4.5 dB ACS relaxation. For the WUR with better NF of 13.5 dB, the ACS is relaxed with 4.5 dB (13.5 – 9 ) compared to the main receiver.

The ACS is relaxed 4.5 dB for the WUR with 18 dB NF compared to ACS of the WUR with 13.5 dB NF.

The ACS is relaxed 4.5 dB for the WUR with 13.5 dB NF compared to the ACS of the MR

The same set of the inteference signal and wanted signal defined for both two set of the LP-WUS requirments. Further discuss the number of guard RB for two set of requirment.

# Conclusions

In this contribution, we present our view on WUR RF requirement for REFSENS, ASCS and ACS with below proposal and observations:

Observation 1 Diversity gain should be -3 dB accounting the 0 combination gain in EIS test.

Proposal-1: both 1 and 2 Rx can be assumed in Rel-19 when combination gain is assumed to be zero.

Proposal-2: The coverage matching requires the same spherical coverage percentile of WUR and MR.

Proposal-3: ASCS can be tested with both small adjacent interferer and big interferer case with selectivity of 0 dB.

Observation 2 The ACS is relaxed 4.5 dB for the WUR with 18 dB NF compared to ACS of the WUR with 13.5 dB NF.

Observation 3 The ACS is relaxed 4.5 dB for the WUR with 13.5 dB NF compared to the ACS of the MR

Proposal-4: The same set of the inteference signal and wanted signal defined for both two set of the LP-WUS requirments. Further discuss the number of guard RB for two set of requirment.

# References

- R4-2420528, WF on UE RF requirements for LP-WUS, vivo

- R4-2502097, TP for WUR RF requirement of REFSESN, ASC, ASCS, Ericsson