3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504399

Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			A-IoT general overview

Agenda item:		7.24.1

Document for:		Approval

# Introduction

In this paper, we present our view on system parameter.

# Discussion

## System parameter

In WID [1], both in-band and standalone are scoped and this is quoted below:

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

In WF [2], for R2D channel bandwidth, RAN4 has reached below agreements:

Issue 2-1-2: R2D transmission bandwidth definition

Agreement:

- Reuse the following definition of R2D transmission bandwidth in TR38.769

  - Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D.

- RAN4 will re-visit the two agreed bullets above, if RAN1 has the different conclusion.

Issue 2-1-3: R2D channel bandwidth definition

Agreement:

- Define R2D channel bandwidth as:

  - Channel bandwidth, BCH,R2D from a reader perspective: The frequency resources used for transmitting R2D, and guard band.

Issue 2-1-4: Values of R2D bandwidth

Agreement:

- Define R2D transmission bandwidth configuration:

  - 1PRB, 2PRB, 3PRB, 4PRB with 15KHz SCS

  - FFS on other RB numbers

- Further discuss the R2D channel bandwidth considering additional guard band.

- If RAN1 has the agreement different than above, RAN4 will revisit the agreements

Based on the WID objective, the channel bandwidth and channel raster can be also discussed in the context of the standalone and in-band operation.

For the in-band operation, coexisting has done when 1 RB is allocated for A-IoT BS and ACLR of the NB-IoT is used to simulate the leakage power from A-IoT BS to NR system. As NR UE has no in-band filter and is assumed outdoor only, there should be no difference between in-band operation and standalone operation in coexisting aspect. Therefore, the in-band and standalone operation can be treated the same when discussing the channel bandwidth.

For standalone operation, as no FDM is assumed for R2D in RAN1, it means no other R2D channel will be configured for the same A-IoT BS. However, in coexisting two Active reader is assumed with good isolation distance so multiple readers scenario is not excluded. Therefore, the spectrum efficiency is not the critical reason for guard band design of the R2D channel bandwidth. In other words, the R2D channels is not necessary to be packed tightly in the A-IoT coverage. As the filter performance can be scaled with channel bandwidth and the roll-off performance depend on the percentage ratio between guard band and channel bandwidth, we think 10% guard band could be used to design the A-IoT channel bandwidth as general guideline.

Moreover, there is offset of 100kHz in NB-IoT standalone operation in 36.104, whether this should be considered for A-IoT can be further studied.

Use 10% as ratio between guard band and channel bandwidth to design channel bandwidth as starting point

Table 1: Channel bandwidth for A-IoT

| R2D transmission bandwidth | Guard band | Channel bandwidth |
| --- | --- | --- |
| 1 RB (180kHz) | 20 kHz | 200kHz |
| 2 RB (360kHz) | 40 kHz | 400kHz |
| 3 RB (540kHz) | 60kHz | 600kHz |
| 4 RB (720kHz) | 80kHz | 800kHz |

Issue 2-1-5: R2D channel raster and channel spacing

Agreement:

- Do not define channel spacing for R2D transmission

- Define channel raster for R2D transmission.

  - Details need further discussion.

For standalone operation, the A-IoT BS should have the same NR channel raster as NR system so the spectrum can be utilized in a efficient way. For in-band operation, some discussion will be needed as in this case, the A-IoT BS may be configured in a way that RBs allocated to A-IoT BS should be orthogonal and aligned with RB grid of a macro BS.

For in-band operation, the orthogonality between RBs in A-IoT BS and RBs in macro NR BS should be guaranteed.

For standalone operation, the legacy NR channel raster should be followed

To derive the channel raster for A-IoT operating in-band within a macro NR BS, some basic concepts should be considered:

- The RF reference frequency defined by this channel raster identifies the RF channel position depending on the number of specified RBs (NRB) by the spectrum utilization for a given SCS and channel bandwidth as shown in Figure 5.2.1.3-1 [3] quoted below.

If this number is even, then it points to the subcarrier #0 (first one) of RB index #0,

If this number is odd, it points to the subcarrier #6 of RB index #0,

Figure 5.2.1.3-1 [3]: NR RF reference frequency position

- The center frequency of the A-IoT BS can be either center of one RB (frequency position between tone 5 and 6 for 1 RB, 3 RB transmission bandwidth) or the center of the two RB. It is assumed that A-IoT BS center frequency use the same definition of the NR BS.

The RB for A-IoT BS has no NB-IoT channel raster restrictions (e.g. follow the LTE DC subcarrier, 100kHz channel raster only) and as such, the channel raster can be expressed with following equations:

- NR 1channel raster and center frequency of NR channel (less than 3 GHz, so ΔF is 5kHz as shown below) is

fNR=FREF+ΔF∙(NREF-NREF0)=FREF+ΔF∙(NREFstart-NREF0)+ΔF∙Step∙m

Where:

        - m being an integer identifying NR RF reference frequency, e.g. ΔF=5kHz, Step=20 for 100kHz raster

  - # RB for A-IoT is odd and maximum # of RB of NR BS is odd, ± means the A-IoT center frequency is either on the left (-) or right (+):

fR2D=fNR±180p

Where:

        - p being an integer representing the RB index from NR reference frequency (p = 0 for the NR RB hosting the NR Reference frequency

  - # RB for A-IoT is even and maximum # of RB of NR BS is even, ± means the A-IoT center frequency is either on the left (-) or right (+):

fR2D=fNR±180p

  - # RB for A-IoT is odd and maximum # of RB of NR BS is even, there will be 6 subcarrier (90kHz) offset between A-IoT center frequency and NR BS center frequency, ± means the A-IoT centre frequency is either on the left (-) or right (+)

fR2D=fNR±(180p+90)

  - # RB for A-IoT is even and maximum # of RB of NR BS is odd, there will be 6 subcarrier (90kHz) offset between A-IoT center frequency and NR BS center frequency

fR2D=fNR±(180p-90)

It can be observed that if modular 2 of maximum # of RB of A-IoT and modular 2 of the maximum # of RB of NR BS is the same, there will be integer of the 180kHz between A-IoT channel raster and NR raster. Otherwise, if modular 2 of maximum # of RB of A-IoT and modular 2 of the maximum # of RB of NR BS is different, the frequency offset between A-IoT channel raster and NR channel raster will be integer number of 180 kHz plus or minus 90 kHz.

Depending on whether modular 2 of maximum # of RB of A-IoT and modular 2 of the maximum # of RB of NR BS is the same or not, there will be either integer number of 180 kHz or integer number of 180 kHz plus or minus 90 kHz for the frequency offset between A-IoT channel raster and NR channel raster

For in-band operation, additional A-IoT channel raster should be designed, considering the orthogonality with macro NR BS.

Issue 2-1-6: D2R bandwidth

Agreement:

- D2R channel bandwidth for A-IoT device

  - Define D2R channel bandwidth for A-IoT device considering the following aspects:

    - [1SB or] 2SB

    - D2R maximum data rate (depending on RAN1 conclusion)

    - SFO

    - FDMA, i.e., considering the different device 1 has different frequency shift

    - Reference Filter performance

    - Other aspects are not precluded

With the defined the D2R channel bandwidth, unwanted emission and occupied bandwidth should be met. We test one criterion of the occupied bandwidth RF requirement. Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. This can be simulated and Figure 1 illustrates OOK signal in time domain and PSD in frequency domain. The data rate is set 20 kbps and OOK bit length is 96 bits. It can be observed from Table 2 that 200kHz channel bandwidth should be fine with these parameters.  As there is also a simulation campaign for the SNR threshold simulation for BS REFSENS, we think the same simulation assumption could be used for simulation of the D2R channel.

Use the occupied bandwidth requirement to quantify the channel bandwidth considering the data rate and other imperfection.

Align the simulation assumption for D2R channel bandwidth simulation.

Figure 1: OOK D2R signal in time and frequency domain

Table 2: Occupied channel bandwidth test case for 20kbps data rate

| Bandwidth (kHz) | Power Containment (%) |
| --- | --- |
| 60 kHz | 98.361301% of total power |
| 80 kHz | 98.771765% of total power |
| 100 kHz | 99.019734% of total power |
| 200 kHz | 99.522660% of total power |

# Conclusions

In this contribution, we present our overview on the A-IoT general issues with below observation and proposals:

Proposal-1: Use 10% as ratio between guard band and channel bandwidth to design channel bandwidth as starting point

Observation 1 For in-band operation, the orthogonality between RBs in A-IoT BS and RBs in macro NR BS should be guaranteed.

Proposal-2: For standalone operation, the legacy NR channel raster should be followed

Observation 2 Depending on whether modular 2 of maximum # of RB of A-IoT and modular 2 of the maximum # of RB of NR BS is the same or not, there will be either integer number of 180 kHz or integer number of 180 kHz plus or minus 90 kHz for the frequency offset between A-IoT channel raster and NR channel raster

Proposal-3: For in-band operation, additional A-IoT channel raster should be designed, considering the orthogonality with macro NR BS.

Proposal-4: Use the occupied bandwidth requirement to quantify the channel bandwidth considering the data rate and other imperfection.

Proposal-5: Align the simulation assumption for D2R channel bandwidth simulation.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2502858, WF on requirements for system parameters and A-IOT device, CMCC

- TR 37.824