# 3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504400

# Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			A-IoT device requirement overview

Agenda item:		7.24.2.2

Document for:		Discussion

# Introduction

In this paper, we present our view on the A-IoT device RF requirements.

# RF requirement for A-IoT UE

## Tx requirement

In WF [6], the TX requirement relating to backscatter transmission is quoted below:

Issue 3-2-1: Transmit output power

Agreement:

- Measurement configuration including input CW power and potential other conditions need to be defined.

RAN4 defines some of measurement condition in last meeting but requirement is to be discussed. The received signal power in the first sideband of backscatter signal at the test antenna [7] can be expressed with equation (1) and this equation can be used in measurement.

Preceived=Pbackscatter∙Gr4πr2=Pt∙Gt4πr2 ∙Gr4πr2∙ηM 				(1)

ηM is defined as modulation efficiency defined in equation (2) in [5]. This parameter relates to the antenna gain and differential reflection coefficient or differential radar cross section in two different modulation states when device modulate the backscattering signal:

- ηm=2π2∙σ1-σ22=Gtag2∙λ22∙π3Γ2*-Γ1*2						(2)

Where Γi* is the conjugate-matched reflection coefficient for each of the two modulation states, i

Γi*=Zi_tag-ZA*Zi_tag+ZA								(3)

To understand the modulation efficiency performance of device 1, a λ/2 dipole is simulated for the case with and without metal plate. For the case with a metal plate the simulation referrers to [5]. In this case, λ/2 dipole antenna is placed above the 1x1 square-meter metal plate with distance d. The modulation efficiency is reproduced in Figure 1 where the device load impedance is assumed 0 in one modulation state and resistance of the impedance is assumed 0 in second modulation state. It can be observed that the surrounding objects (metal in this case) modify the antenna impedance depending on the distance d to the metal and in turn this will change the modulation efficiency. In Figure 2 and Figure 3, the RCS of two different modulation states is illustrated with different incident wave angles. It can be observed that the backscattering RCS has dependence on the incident wave angle, and this can also lead to different modulation efficiency performance. The cdf curve is present for modulation efficiency in Figure 4 for full sphere and half sphere coverage. It can be observed that half sphere cdf curve may make more sense than full sphere as the 50 percentile number becomes too low for requirement purpose. If the device would be installed above a metal surface, the half sphere above the metal may face the CW node and BS to have a better coverage so from deployment perspective, the half sphere could be good enough for requirement purpose.

The surrounding objects impact the modulation efficiency by changing the antenna impedance.

With metal material mounted behind a device 1, the modulation efficiency may depend on the incident wave angle.

Half sphere cdf curve is characterized better for the modulation efficiency for metal plate case.

Figure 1: Modulation efficiency for λ/2 dipole for the case with a metal plate

Figure 2: RCS in two different modulation states with incident wave direction of 0 degree theta

Figure 3:RCS in two different modulation states with incident wave direction of 330 degree theta

Figure 4: CDF curve of the modulation efficiency for the case where metal is present

For the case without the metal, the modulation efficiency could be decoupled with the incident wave angle. This is illustrated in Figure 5 and Figure 6. It can be observed that the RCS of two modulation states is similar for incidence wave direction of 0 degree and 330 degree cases. The sphere surface presentation of the modulation efficiency and cdf curve is shown in Figure 6. The load impedance is chosen according to Figure 7.

Figure 5: The RCS in two modulation states with different incidence wave angle (0 degree and 330 degree in elevation plane) for the case where no metal plate is present

Figure 6: CDF curve of modulation efficiency (Full sphere)

Figure 7: modulation efficiency with antenna gain=1.83

Based on analysis above, the modulation efficiency OTA requirement could be defined as similar as min EIRP of spherical coverage in FR2, but with a different metric, for example:

The minimum modulation efficiency at the Xth percentile of the distribution derived from radiated power measured in the first sideband of backscattering signal over the [full sphere] around the UE is specified in Table Y below. The requirement is verified with the test metric of modulation efficiency (Link=Spherical coverage grid, Meas=Link angle).

Table Y: spherical coverage for backscatter modulation efficiency

| Operating band | Min minimum modulation efficiency at [50] %-tile CDF (dBsm) |
| --- | --- |
| N8 | FFS |

Specify the modulation efficiency as possible backscatter requirement

Specify the spherical coverage requirement for min modulation efficiency associated with Xth percentile.

Further discuss whether the requirement should be differentiated when device is targeted to be mounted to metal surfaceVery nic

In EPC standard [8], the backscatter strength is declared by vendor:

6.3.1.3.5 Minimum operating RF field strength and backscatter strength

For a Tag certified to this protocol, the Tag manufacturer shall specify:

1. free-space sensitivity,

2.  minimum backscattered modulated power (ASK modulation) or change in radar cross-section or equivalent (phase modulation), and

3. the manufacturer’s normal operating conditions, for the Tag mounted on one or more

manufacturer-selected materials.

It can be observed that similar metric is used for declaration of the backscatter strength in EPC standard. However, as it is the declaration, so EPC does not put requirement on it.  Another observation is that EPC also require the manufacture of tag to declare the mounted materials. Regarding this, RAN4 can further discuss whether to allow the metallic mounting material behind the device so it should be tested differently.

Further discussion whether the device should be declared to be mounted on a metallic surface.

Issue 3-2-2: Modulation quality requirements

Agreement:

- Further discuss whether and how to define the modulation signal quality requirements

For modulation signal quality requirement, the SNR degradation of a signal can be contributed by intrinsic noise from the hardware (e.g. the intermodulation product from non-linear PA) and EVM can be measured for legacy system. For the backscatter signal modulation, modulation is done by switching across different load impedance and therefore, a different metric may be relevant to modulate signal quality. In modulation efficiency requirement discussion, the first sideband power is measured and modulation efficiency is derived using equation (1). It can be discussed that also the backscattered signal should be decodable so any imperfection hidden in design could be detected. In this sense, the BLER could be used as one metric to test the signal quality and this could be tested in the direction where maximum modulation efficiency is measured (beam locked mode). Such test should be complementary to the modulation efficiency test as it only relates to power but not monitoring the signal quality.

- Proposal 2: BLER can be tested for the backscattered modulation quality as a directional requirement.

Issue 3-2-3: Occupied bandwidth

Agreement:

- Further discuss whether to define the occupied bandwidth for device 1.

In our view, the SFO (sampling frequency offset) which is quite large (e.g 10%), this may shift the center frequency of the first sideband but there is no frequency error requirement in device 1 and occupied bandwidth can capture this as a “distortion” on the modulated signal.  This test can be directional and only test the maximum modulation efficiency direction.

Specify the occupied bandwidth test to test the SFO imperfection.

Issue 3-2-4: Emission requirements

Agreement:

- Consider one composite emission requirements covering the emission frequency range of SEM and spurious emission as starting point.

- Further discuss whether to define TRP or EIRP requirement for unwanted emission.

In 38.101-2, the SEM and spurious is measured in TRP. The spurious requirement is regulatory requirement so the measurement should be aligned with 38.101-2 for OTA test.

The TRP metric should be used for SEM and Spurious requirement.

## Rx requirements

For device 1 which harvests the RF energy, it may be useful to discuss whether to have EH sensitivity as one requirement. As the energy harvesting sensitivity may be different with the REFSENS, for D1T1-B, the device can be illuminated with CW or other RF signal but still have a different REFSENS specified and as such, the OTA test can be set up with the knowledge both EH sensitivity and REFSENS.

The EH sensitivity and REFSENS both should be known for the RF test.

According to the coexisting assumption in SI phase, the CW node is co-located with reader, implying the EH sensitivity also play role in the reader coverage and therefore, RAN4 needs to discuss whether an EH sensitivity should be specified. Our opinion is that if coexisting simulation is not changed, EH sensitivity should be specified otherwise the system may not work as expected. Another alternative for EH is the manufacture declare the EH but then needs further discussion in test feasibility to see if at least some range is needed to limit the test condition impact.

Energy harvesting sensitivity relevant to the reader range for D1T1-B deployment

Specify the REFSENS and declaration based on EH sensitivity if there is no impact on test feasibility design.

For the REFSENS requirement, the options are listed below from WF [6]:

  - For R2D reference sensitivity, consider following options:

    - Option 1: define reference sensitivity using legacy approach, e.g. PREFSENSdBm=-174dBm+10×log10BW+NF+IM+SNR

    - Option 2: define reference sensitivity based on link budget and/or physical limits of the receiver circuitry.

    - Other options are not precluded.

For option 1, the SNR of the R2D signal should be agreed. This may involve the LLS simulation.

For option 2, for a ISD of 20 meter case between CW node and BS, REFSENS is good enough with -26 dBm as illustrated in Figure 8. Any number higher than this is not necessary. It can even be relaxed to -20 dBm if BS transmits 38 dBm power.

Discuss whether to set the REFSENS with option 2.

Figure 8: The received power at device with different BS output power and ISD between CW node and BS

For the maximum input power, legacy requirement relates to the saturation of the receiver RF chain. For device 1, the diode will be used and it is a question if the square-law relation between input power and output voltage can be kept within a pre-defined range. As the square law is assumed in the ED modeling in LLS, if the component deviates from this behavior beyond certain input power, it should be specified as a requirement. Figure 1 gives one example of one diode from one manufacture and shows the square law only applies within a certain range of input power. RAN4 can start to collect this parameter before specifying the maximum input power.

Discuss the necessity of specifying the maximum input power by collecting the square law appliance range.

Figure 8: detector output characteristic

# Conclusions

In this contribution, we present our overview on the A-IoT device RF with below proposal and observations:

Observation 1 The surrounding objects impact the modulation efficiency by changing the antenna impedance.

Observation 2 With metal material mounted behind a device 1, the modulation efficiency may depend on the incident wave angle.

Observation 3 Half sphere cdf curve is characterized better for the modulation efficiency for metal plate case.

Proposal-1: Specify the modulation efficiency as possible backscatter requirement

Proposal-2: Specify the spherical coverage requirement for min modulation efficiency associated with Xth percentile.

Proposal-3: Further discuss whether the requirement should be differentiated when device is targeted to be mounted to metal surface

Proposal-4: Further discussion whether the device should be declared to be mounted on a metallic surface.

Proposal-5: Specify the occupied bandwidth test to test the SFO imperfection.

Proposal-6:   The TRP metric should be used for SEM and Spurious requirement.

Observation 4 The EH sensitivity and REFSENS both should be known for the RF test.

Observation 5 Energy harvesting sensitivity relevant to the reader range for D1T1-B deployment

Proposal-7: Specify the REFSENS and declaration based on EH sensitivity if there is no impact on test feasibility design.

Proposal-8: Discuss whether to set the REFSENS with option 2.

Proposal-9: Discuss the necessity of specifying the maximum input power by collecting the square law appliance range.

# References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- TR 38.769

- R4-2420470, WF on RF requirements for A-IoT, Huawei

- Pavel V. Nikitin and K. V. S. Rao, “Theory and measurement of backscattering from RFID tags” IEEE Antennas and Propagation Magazine (Volume: 48, Issue: 6, December 2006)

- Christoph Degen, Patrick Bosselmann, “On Complex Radar Cross Section and Backscatter Modulation Efficiency in RFID Systems”, 2020 14th European Conference on Antennas and Propagation (EuCAP)

- R4-2502858, WF on requirements for system parameters and A-IOT device, CMCC

- R4-2502100, A-IoT device requirement overview, Ericsson

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID