# 3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504402

# Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			RF Impact for FDD bands for PC2 RedCap

Agenda item:		7.6.1

Document for:		Approval

# Introduction

In this paper, we present our view on remaining issue for PC2 RedCap UE RF for FDD bands.

# Discussion

In WF[1], the remaining issue is the 1 Rx FD-FDD REFSENS.

Issue 1-2-3: REFSENS for 1Rx in FD-FDD

Agreement:

- Further discuss the following formats for further evaluations

  - Option 1: REFSENS (1Tx (23dBm) /1Rx) + RSD

  - Option 2: REFSENS (1Tx (26dBm) /2Rx) + ΔR

    - Evaluate the values of ΔR band by band

  - Note: Other options are not precluded

For some of FDD bands for PC2, there are additional RSD allowed and this is specified in Table 7.3.2-1c in TS 38.101-1 and quoted below. The exemplar band n1 and n3 is specified for 2 Rx relaxation.

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |
| n3 | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n7 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |  | 2.0 |
| n8 | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n13 | 0.8 | 0.9 |  |  |  |  |  |  |  |  |
| n14 | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n25 | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| n85 | 0.6 | 1.0 | 2.6 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

However, as this relaxation is for two Rx, there is a need to see how to specify the 1 Rx RSD for PC2. For RSD, the scaling factor approach for the REFSENF for 1 Rx is used in Rel-17 RedCap for PC3 based on the 2Rx REFSENS. During the Rel-17 1 Rx scaling factor discussion FD-FDD band, the Tx interference to Rx interference is considered when scaling 2 Rx to 1Rx in Table 7.3I.2-1 in TS 38.101-1. This scaling factor considers the factor that the main Rx branch may have more interference compared to the diversity branch and also the factor that the Tx interference may be higher as the channel bandwidth increased.

Table 7.3I.2-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |

The Tx leakage falling into the Rx receiving band in main Rx branch and also coupled to the diversity branch cause MSD degradation at receiver. Such Tx leakage power will be higher for PC2 than the leakage power of PC3.  As such a different scaling factor can be considered comparing to scaling factor of PC3 1Rx. No matter what relaxation number in the end to be agreed, when one comparing the 1 Rx REFSENS of PC2 and 2 Rx REFSENS PC2, the difference between the two REFSENS shall not exceed 3 dB as the theoretical MRC gain is 3 dB. The UL configuration for RENSENS will not be changed in this case.

The scaling factor or relaxation factor should not exceed 3dB.

# Conclusions

In this contribution, we present our overview on the PC2 RedCap RF overview with below observation:

Observation 1 The scaling factor or relaxation factor should not exceed 3dB.

# References

- R4-2502874, WF on NR PC2 RedCap UE in FDD bands, China Telecom