# 3GPP TSG-RAN WG4 Meeting # 114-bis					R4-2504405

# Wuhan, China,7th–11th April 2025

Source:		Ericsson

Title:			LS reply for IoT NTN

Agenda item:		7.30.2

Document for:		Approval

# Introduction

In this paper, we present our reply over the LS from RAN1[1].

# Discussion

In LS [1]

RAN1 has made the following working assumption on the TDM DMRS pattern for OCC for 3.75kHz SCS OCC for NPUSCH format 1:

For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.

Question:

From a RAN4 perspective, is it feasible to introduce support of TDM DMRS, as per the description above, in Rel-19?

Firstly, the TDM DMRS introduced in RAN1 relating to the LS statement is illustrated in below,

Figure 1: First two slots for 2 UEs using TDM DMRS over 4 slots, where UE2 blanked the first two DMRS symbols in first two slots

Figure 2: Next two slots for 2 UEs using TDM DMRS over 4 slots, where UE1 blanked the two DMRS symbols in second two slots

The slot is defined as picture below in TS 36.211 as 2ms duration for 3.75kHz SCS.

Our understanding on the feasibility on the TDM DMRS, raises a question on the OCC2 performance over the slots where DMRS symbols are blanked. In 36.102, the measurement duration is defined in frequency error requirement clause and quoted below:

“The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4B.1-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the ideally pre-compensated reference uplink carrier frequency.”

6 ms

This means the CFO is measured over 2ms slot for 3.75kHz SCS. For the slot where the DMRS symbol is blanked, the CFO estimation will be up to the bundled time slot over 6ms where the two DMRS symbols between the 6ms slots can be used to estimate the CFO.

Figure 3: The largest time gap of 6ms between two DMRS symbol

Assuming the two adjacent different DMRS symbols are used for the CFO estimation at base station, and assume the UE has 200Hz frequency error, phase rotation between two DMRS symbols are:

200Hz * 6ms * 360 degree = 432 degree.

The phase rotation over 360 degree will not be detected at the base station and as such a frequency estimation error of 166.7Hz will be created (base station will see 72 degree instead of 432 degree in this case).

200Hz -  72/360/6ms = 200Hz- 33.3Hz = 166.7Hz.

Such frequency error will generate the phase rotation for two OCC symbols:

166.7 Hz * 2* 275 us* 360 = 30 degree

It is said the phase continuity should be assured for OCC code spread within two symbols.  With 30 degree phase difference between two OCC code, the OCC orthogonality cannot be maintained and OCC2 performance may be impacted.

The largest time gap of 6ms between two adjacent DMRS symbols may introduce 30 degree phase offset between two OCC2 code when UE frequency error is 200Hz.

The phase continuity between two OCC symbol cannot be maintained when UE frequency error is 200Hz.

On the other hand, the maximum frequency error for a 6ms time gap that can be measured correctly at the base station is 1/6ms = 166.7 Hz, so if UE CFO performance is smaller than this, the CFO estimation error at the base station can be minimized and the phase continuity between OCC code can be maintained.

The phase continuity of the OCC code can be maintained if UE frequency error is less than 167 Hz during 6ms which is the largest time gap between two adjacent DMRS symbols.

Tightening the UE frequency error may not be feasible as frequency error may relate to the DL signal strength instead. It may be more feasible to perform a CFO grouping but then it is up to network implementation to select UEs with less CFO  for OCC scheduling.

The suggested LS reply is below:

RAN4 observes up to 30 degree phase offset between two OCC symbols, therefore the phase continuity of Symbol-level OCC length 2 for NPUSCH Format 1 with 3.75 kHz SCS cannot be maintained when UE frequency error is 200Hz. If UE frequency error is below 166.7 Hz, the phase continuity of the two OCC code can be maintained. RAN4 will not specify a new frequency error requirement for this purpose as RAN4 understand the frequency error may depend on DL signal strength also. In this regard, RAN4 believe it is better that the network can use a CFO grouping (left up to implementation) to maintain the phase continuity of OCC codes.

RAN4 discuss the above LS reply.

# Conclusions

In this contribution, we present our general overview for LS [1]

Observation 1 The largest time gap of 6ms between two adjacent DMRS symbols may introduce 30 degree phase offset between two OCC2 code when UE frequency error is 200Hz.

Observation 2 The phase continuity between two OCC symbol cannot be maintained when UE frequency error is 200Hz.

Observation 3 The phase continuity of the OCC code can be maintained if UE frequency error is less than 167 Hz during 6ms which is the largest time gap between two adjacent DMRS symbols.

Proposal-1: RAN4 discuss the belwo LS reply:

RAN4 observes up to 30 degree phase offset between two OCC symbols, therefore the phase continuity of Symbol-level OCC length 2 for NPUSCH Format 1 with 3.75 kHz SCS cannot be maintained when UE frequency error is 200Hz. If UE frequency error is below 166.7 Hz, the phase continuity of the two OCC code can be maintained. RAN4 will not specify a new frequency error requirement for this purpose as RAN4 understand the frequency error may depend on DL signal strength also. In this regard, RAN4 believe it is better that the network can use a CFO grouping (left up to implementation) to maintain the phase continuity of OCC codes.

# References

- R1-2501622,	LS on TDM DMRS for 3.75kHz SCS OCC,