3GPP TSG-RAN WG4 Meeting #114bis	R4-2504414

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Fundamental RX requirements for LP-WUR

Agenda item:	7.26.2.2

Document for:	Discussion

Introduction

In this document we present our views on open issues regarding REFSENS, ACS, and ASCS requirements. Following agreements were made in the last RAN4 meeting [1].

# Topic #2: REFSENS, ASCS and ACS requirements ### Sub-topic 2-1 FR1 SNR, NF and IM for REFSENS requirements Issue 2-1-1: SNR, NF and IM value for FR1 LP-WUR Agreement: - For the FR1 requirements targeting at bands <2.5GHz - The IM+NF values are - Set 1: 18dB - Set 2: 13.5dB - Send an LS to RAN1 with RAN4 decision of two sets of : NF (assume IM is 2.5dB) - Ask RAN1 to check whether coverage target can be met based on above NF information. And ask for detailed signal configuration (e.g., chip rate, length of sequence) of LP-WUS. - LS should also cc RAN2 Issue 2-1-3: Timing error impacts for OOK-based and OFMD-based SNR Agreements - Timing error contribution to RAN4 RF simulation can be contribution driven. ### Sub-topic 2-2 FR2 SNR, NF and IM for REFSENS requirements Issue 2-2-1: Combining strategy for FR2 LP-WUR Agreements: - For Rel-19, confirm no combination gain across two Rx chains is considered for FR2 LP-WUR. FFS whether and how to capture the impacts of Rx selection in REFSENS evaluation. - Confirm 1Rx at baseband for FR2 LP-WUR. Issue 2-2-2: NF for FR2 LP-WUR Way forward: - Adopt similar approach to FR1 LP-WUR, encourage companies to input analysis on IM, NF and SNR together. Issue 2-2-3: Spherical coverage for FR2 LP-WUR Way forward: - RAN4 further discuss how to specify spherical coverage requirements for FR2 LP-WUR. ### Sub-topic 2-3 ASCS simulation and requirements Issue 2-3-2: in-band power level of ASCS Test case Agreements: - For ASCS, allowable de-sense due to introduction of NR RBs is [0-1dB]. - [0] guard RB is the baseline. Issue 2-3-3: channel bandwidth of ASCS Test case Agreements: - For ASCS core requirements, it should be applied to all NR CBW. - For conformance test, for NR CBW can be further discussed and down-selected. - For ASCS test configuration, the NR RBs should be fully allocated with the same PSD of LP-WUS within the channel except for 11 LP-WUS RBs and potential guard RBs (if any). ### Sub-topic 2-4 ACS simulation and requirements Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB Agreements - Same interference signal level as MR for LP-WUR ACS requirements - FFS at least for one set of Rx requirements. - The LP-WUS level is LP-WUS REFSENS+[X]dB, X=14 as starting point - FFS whether two sets of LP-WUS wanted signal level can be defined. - Final ACS will be decided by LP-WUR REFSENS and wanted signal relaxation - FFS number of guard RBs. Issue 2-4-2: ACS requirements (same interference) and required guard RB Way forward: - FFS, also depends on discussion for Issue 2-4-1 Issue 2-4-3: ACS requirements (lower interference level) and required guard RB Way forward: - FFS, also depends on discussion for Issue 2-4-1 Issue 2-4-3: Case 1 ACS test parameter Way forward: - FFS LPWUS ACS test case. - ACS requirement applies to all NR CBW

Discussion

Timing error impact

During the last meeting it was raised that there is a need to relax the requirements based on the residual timing error as in the TR it was mentioned that there is an impact on the effective SNR based on residual timing error. However, inspecting the TR results in detail we reached on the following observations.

- There is no performance impact with a timing error of up to 4us irrespective of modulation type.

- The simulations to measure the SNR impact were done with TDLC-300ns channel and an impact of 2dB was found for timing error greater than 4 us.

Regarding timing error estimation, we did simulations with AWGN channel for different values of M and L, and following are the results.

Table 1 Timing error estimate in AWGN channel.

| M | 1 | 1 | 2 | 2 | 4 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| L | 6 | 8 | 8 | 16 | 16 | 32 |
| Timing Accuracy (us) | 2.5 | 1 | 0.3 | 0 | 0.1 | 0 |

From the simulated results, it can be concluded that for the current values of modulation being considered, there is no impact of timing error on the performance of LR and there should not be any relaxation in requirements based on it.

- The results in TR 38.869 show no performance degradation with timing error under 4us irrespective of modulation type.

Simulated results show that the observable timing error is less than 2.5 us, for all the modulations currently being considered.

Agree to have no impact on RAN4 RF requirements because of timing error.

ASCS requirement

The guard RBs for ASCS requirement have been tentatively agreed to [0] in the last meeting. Further, it was agreed to have allowable de-sense by [0 - 1] dB because of NR signal. In the past we have published results comparing performance loss because of presence of adjacent NR signal i.e., ASCS case, compared to performance without any adjacent NR signal i.e., REFSESN case [2]. We do not see any measurable performance loss because of presence or absence of NR signal. These simulations were performed with zero guard RB in the ASCS case. Thus, in our opinion the guard RB requirement should be set to 0 RB for ASCS.

No measurable de-sense by the presence of the adjacent NR signal in case of zero guard RB.

For the ASCS requirement side conditions, agree to have zero guard RB for the ASCS case i.e., there is no offset between NR signal and LP_WUS.

Further, as the interferer and LP-WUS must have same PSD, so the only question remains is what that value should be. For legacy UEs, the ACS is tested at two different power levels, one at REFSENS + 14 dB and another at a fixed level of -56.5 dBm. This enables testing filter suppression performance across both optimal and worst-case dynamic range scenarios. In our opinion this power level should be significantly higher than the REFSENS level in order to eliminate the impact of noise. Thus, we propose to use a signal level of REFSENSLR + 14 dB for the ASCS requirement tests. It has already been agreed that for ASCS case, the NR signal will have the same PSD as the wanted WUS signal. Given the WUS signal is 11 PRB wide and its signal level is specified in absolute terms, so NR signal should also be 11 PRBs wide.

NR Interferer signal will have the same PSD as the LP_WUS.

Level of LP_WUS and NR interferer should be sufficiently larger than the sensitivity level.

For the ASCS requirement side conditions, agree to have the power of LP_WUS and NR signal in dBm as REFSENSLP_WUR + 14 dBm.

For the ASCS requirement side conditions, agree to have the number of RBs of LP_WUS and NR signal to be fixed at 11 PRBs irrespective of SCS.

ACS requirement

Till now there has been no agreement on neither the ACS requirement value nor the side conditions for it. Given the limited capabilities of the LP_WUR, it is bound to have lower dynamic range compared to the MR. This makes it challenging to just reuse the legacy ACS requirements and side conditions for the LP_WUR. To figure out the requirements, LLS were conducted for the envelop-detector based LP_WUR with the simulation setup illustrated in in Figure 1. For the simulation two parameters are considered:

- The ACS interference level is swept from 0 to 31.5 dB,

- The test is performed with number of guard RBs ranging from 1 to 4.

Figure 1 Illustration of the simulation test setup.

In Figure 2 and Figure 3 the ACS simulation results are illustrated. For the AWGN channel it can be seen that the MDR=1% can be obtained when the interferer level is <17dB (guard of 1RB), <23dB (guard of 2RBs) and <28 dB (guard of 3RBs). The test with guard is 4RBs is not illustrated in the plot as the interferer can be > 31.5 dBm

Figure 2 Illustration of the simulation results for an AWGN channel with a guard band of 1-3 RB. The interfere level is swept from 0-31.5 dB.

The simulation of the ACS performance for TDLC300 channel is performed with guard of 1-4 RB. For the figure it can be seen that the MDR=1% can be obtained when the interferer level is <12dB (guard of 1RB), <17dB (guard of 2RBs) , <22 dB (guard of 3RBs) and <27 dB (guard of 4RB).

Figure 3 Illustration of the simulation results for a TDLC300 channel with a guard band of 1-4 RB. The interfere level is swept from 0-31.5 dB.

During the SI phase, there was no consensus on whether the guard RBs for ACS case have to be empty or any NR signal can go. In our opinion they should be empty as other NR signal can have PSD larger than WUS and this will impact the performance of LP_WUR. This means that these guard RBs are just waste of spectrum. Thus, there should be a balance between the ACS requirement and guard RBs. Given that the NR interferer power has been agreed to be the same as for legacy UE, the following parameters can be reused from the legacy UE requirements

- Pinterferer,

- BWinterferer,

- Finterferer.

Table 2 Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz,

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm |  | REFSENSMR + 45.5 - ACSLR dB | REFSENSMR + 42.5 - ACSLR dB | REFSENSMR + 39.5 – 10log10(BWChannel /20) - - ACSLR dB |
| Pinterferer4 | dBm |  | REFSENSMR + 45.5 dB | REFSENSMR + 42.5 dB | REFSENSMR + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

LP_WUR dynamic range will be less than the dynamic range of MR.

In AWGN channel, for an envelope detector receiver, four guard RBs are sufficient to satisfy the current MR ACS requirement.

With one guard RB, the ACS requirement has to be less than 17 dB.

There should be a balance between number of guard RBs and ACS requirement.

For the ACS requirement side conditions, agree to have two guard RBs.

Agree to have the ACS requirement as 22 dB.

Agree to have the ACS test parameters as defined in Table 2.

Conclusion

The following Observations and Proposals were made:

The results in TR 38.869 show no performance degradation with timing error under 4us irrespective of modulation type.

Observation 1: Simulated results show that the observable timing error is less than 2.5 us, for all the modulations currently being considered.

Proposal 1: Agree to have no impact on RAN4 RF requirements because of timing error.

Observation 2: No measurable de-sense by the presence of the adjacent NR signal in case of zero guard RB.

Proposal 2: For the ASCS requirement side conditions, agree to have zero guard RB for the ASCS case i.e., there is no offset between NR signal and LP_WUS.

Observation 3: NR Interferer signal will have the same PSD as the LP_WUS.

Observation 4: Level of LP_WUS and NR interferer should be sufficiently larger than the sensitivity level.

Proposal 3: For the ASCS requirement side conditions, agree to have the power of LP_WUS and NR signal in dBm as REFSENSLP_WUR + 14 dBm.

Proposal 4: For the ASCS requirement side conditions, agree to have the number of RBs of LP_WUS and NR signal to be fixed at 11 PRBs irrespective of SCS.

Observation 5: LP_WUR dynamic range will be less than the dynamic range of MR.

Observation 6: In AWGN channel, for an envelope detector receiver, four guard RBs are sufficient to satisfy the current MR ACS requirement.

Observation 7: With one guard RB, the ACS requirement has to be less than 17 dB.

Observation 8: There should be a balance between number of guard RBs and ACS requirement.

Proposal 5: For the ACS requirement side conditions, agree to have two guard RBs.

Proposal 6: Agree to have the ACS requirement as 22 dB.

Proposal 7: Agree to have the ACS test parameters as defined in Table 2.

- References

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.

- R4-2415707, Fundamental RX requirements for LP-WUR, Nokia, RAN4#112bis, Hefei, China, 14th – 18th October, 2024.