3GPP TSG-RAN WG4 Meeting #114bis	R4-2504416

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Testability aspects of LP-WUR

Agenda item:	7.26.2.4

Document for:	Discussion

Introduction

In this document we present our views on testability aspects of the LP-WUR. Following agreements were made in the last RAN4 meeting [1].

# Topic #4: Testability issues ### Sub-topic 4-1 Testability for UE RF requirements Issue 4-1-1: General framework for LP-WUR testability Way Forward: - FFS Issue 4-1-2: Separate RF test case for idle and connection mode Way forward: - RAN4 further discuss and check whether all UE types can support IDLE and/or CONNECTED mode?

Discussion

Test procedure for the LP-WUR RF performance parameters

The test procedure must make sure that it is testing only the LP-WUR performance and not the combined performance of MR and LP-WUR. This might require using high SINR for the MR whenever there is a communication between TE and MR. However, designing detailed test cases comes under RAN5 responsibility.

RAN5 defines the detailed test procedure for the conformance tests.

A higher SNR for the main radio signals can be used, so that just the performance of the LP-WUR is tested.

Test mode

As the LP-WUR is going to be a receiver which has no direct external interface to the test equipment (TE) as it can only talk with TE via the main radio (MR) unless there is a test mode to enable LP-WUR to enable such functionality. A test mode will help in simplifying the test procedure and will facilitate in faster testing. Further, as the WUS functionality will be presented in all RRC modes, so the test mode should support all RRC states. This will also help in testing the efficiency of LP-SS. The RRM test mode can be further discussed in the RRM session. However, how to design the test mode and how that will work in RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED mode can be left for RAN5 to define with RAN4 input.

RAN5 can design the details of Test mode required to test the LP_WUR with RAN4 input.

Test mode shall support RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states.

RAN4 define the test interface between the MR and the LR, to ensure the MR is able to enter deep sleep with LR test is ongoing. The MR should only be woken up when data transfer the system simulator is needed.

Testing time

To achieve a ≤1% miss detection rate (MDR) of LP-WUS in a statistically confident way will take long time. The total time taken to have a statistically significant value depends on multiple factors such as test case execution time, TE setup and response time, UE feedback time, repetition cycle, wait times between repetition, etc. As the motivation to have a higher MDR (≥1%) is to save on conformance test time, it is best for RAN5 to decide the confidence interval for the requirements in order to test them in a reasonable amount of time.

Only motivation to have a higher (≥1%) MDR is to save conformance test time.

Lot of factors impacting the time taken for test case execution are under the scope of RAN5 work.

RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

Conclusion

The following Observations and Proposals were made:

Proposal 1: RAN5 defines the detailed test procedure for the conformance tests.

Proposal 2: A higher SNR for the main radio signals can be used, so that just the performance of the LP-WUR is tested.

Proposal 3: RAN5 can design the details of Test mode required to test the LP_WUR with RAN4 input.

Proposal 4: Test mode shall support RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states.

Proposal 5: RAN4 define the test interface between the MR and the LR, to ensure the MR is able to enter deep sleep with LR test is ongoing. The MR should only be woken up when data transfer the system simulator is needed.

Observation 1: Only motivation to have a higher (≥1%) MDR is to save conformance test time.

Observation 2: Lot of factors impacting the time taken for test case execution are under the scope of RAN5 work.

Proposal 6: RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

- References

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.