3GPP TSG-RAN WG4 Meeting #114bis	R4-2504418

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	RedCap NTN UEs

Agenda item:	7.29.2.1

Document for:	Discussion

Introduction

Following agreements were made in the last RAN4 meeting [1].

## Issue 1-1: PC3 output power for (e)RedCap Agreement: - Rel-18 power boosting is applicable for NR NTN including (e)RedCap - No reference sensitivity specification impact (e.g. RSD) from power boosting. - No changes to the PC3 tolerance for HD-FDD and FD-FDD compared non-RedCap UE NR NTN - FFS on solutions on additional new output power enhancement - supported modulation orders - potential downselection between HD-FDD and FD-FDD - simultaneous support with rel-18 power boosting - Background: output power enhancement (e.g., FE IL improvement) - Other solutions not precluded Note: No change for (e)RedCap MOP tolerance to enhance the output power does not preclude any solution in other Rel-19 WI. ## Issue 1-2: HD-FDD refsens for 2 Rx Agreement: - Reference sensitivity for 2Rx HD-FDD for n255 and n254 is Operating band / SCS / Channel bandwidth. Operating Band; SCS kHz; 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm). n255; 15; -100.0; -96.8; -95.0; -93.8. 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. n254; 15; -100.0; -96.8; -95.0. 30; -97.1; -95.1. 60; -97.5; -95.4. Note: This agreement does not preclude improved reference sensitivity for other bands ## Issue 1-3: Frequency error Agreement: The NR NTN non-RedCap UE frequency error requirements are reused for NTN (e)RedCap UE ## Issue 1-4: Release independence Agreement: As a starting point in case no new RAN1/RAN2 impact is identified: - FD-FDD RedCap can be release independent from Rel-17 - FD-FDD eRedCap can be release independent from Rel-18 - HD-FDD (e)RedCap can be release independent from Rel-19

Further, RAN4 has received a reply LS from RAN2 regarding simultaneous GNSS operation with NR, with the contents copied below.

1. Overall Description: RAN2 would like to thank RAN4 for the original LS in R4-2419898. RAN2 has considered the simultaneous operation between GNSS and UL transmission in NR NTN and concluded not to work on the enhancements to the existing IDC solutions in Release 18. RAN2 might consider the extension to IDC TDM solution or autonomous denial solution in Release 19, and would like to receive more information on requirements, e.g. on the periodicity and duration of IDC free time to help the UE read the GNSS in NTN. 2. Actions: To RAN4: ACTION: RAN2 respectfully asks RAN4 to take the aforementioned RAN2 decisions into account and provide the corresponding requirements, e.g. the periodicity and duration of IDC free time to help the UE read the GNSS.

In this paper we provide our views on the simultaneous operation of the RedCap NTN UE with GNSS.

Discussion

Simultaneous operation with GNSS

As discussed during several previous meetings, it is agreeable that there are technical challenges regarding parallel operation of some GNSS variants with NR NTN in some bands. Neither all GNSS bands, nor all UEs suffer i.e., depending on UE implementation, and the GNSS signals (and frequencies) available in a given location, not all UEs will be harmed by this collision all the time.

Not all UEs will be harmed by the potential interference between UL and GNSS frequencies.

According to the RAN2 reply, they can consider extension to the IDC TDM or autonomous denial solution in Release 19 only and will not consider any enhancements in Release 18. Given RAN2 stand on older release, it makes sense to not do any further work regarding this issue in Release 18 in RAN4 too.

RAN2 will consider enhancements to the IDC framework in Release 19 only. No changes will be done for Release 18.

RAN4 shall only consider any changes regarding this issue in Release 19.

RAN2 also indicates that for although they are willing to enhance the IDC features for NTN UEs in Release 19, they will only do so if RAN4 provides proper guidance about the values to be used. The IDC feature is composed by two important IEs as described by TS 38.331:

- AutonomousDenialParameters

- IDC-TDM-Assistance-r18

The first IE configures the autonomous denial at the UE side. It contains the following parameters and allows the following values:

AutonomousDenialParameters-r18 ::= SEQUENCE { autonomousDenialSlots-r18 ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1}, autonomousDenialValidity-r18 ENUMERATED {n200, n500, n1000, n2000}

The UE is allowed to deny UL transmissions according to the values above, when configured. The procedure goes as follows:

1> if the CellGroupConfig contains the autonomousDenialParameters: 2> consider itself to be allowed to deny any transmission in a particular UL slot if during the number of slots indicated by autonomousDenialValidity, preceding and including this particular slot, it autonomously denied fewer UL slots than indicated by autonomousDenialSlots within the same cell group;

As can be seen above, the UE is allowed to drop up to 30 slots (30 ms in 15 kHz and 15 ms in 30 kHz). This is a significantly limited span of time if compared to the total time needed to update a GNSS signal. But this is just one of the aspects around the feature. This feature also allows the UE to indicate to the network, how much time it needs to solve the IDC problems (by indicating an active cycle and cycle length) as follows:

In the IDC TDM solution, the UE notifies the amount of time it needs to sort the IDC and the network might react by avoiding scheduling UL grants in the interval proposed by the UE. In this sense, there is no need to undergo autonomous denial, as much of the UL resources will not be used to schedule this UE. However, as of Rel-18, the maximum active duration of the IDC is 1600 ms within a maximum cycle length of 10240 ms. Assuming that in NTN, the largest IDC cycles are the ones to be used when there is a conflict between the UL band and one of the GNSS bands, this means that the UE requests to avoid UL transmissions at 15% of the time. In a scenario where multiple UEs request IDC, this imposes extra complexity on the NW scheduler.

On the other hand, we understand the concerns raised by other companies that 1600 ms might not be enough time to fully acquire GNSS information. As GNSS is a paramount feature of NTN from Release 17 to Release 19, both, NW and UE should strive for guaranteed timely GNSS information at the UE side. But, if the active duration is expanded, the UEs requesting the largest active duration will be compromising close to 20% of the UL duty cycle. In several cases, this is not in the best interest of the UE that in low mobility scenarios might only need to update the GNSS seldom.   One of our main concerns is that higher UL interruptions could affect

As a compromise between the complexity added to the UL scheduler and the need of NTN UEs to acquire the GNSS information, we propose the following:

Increase the maximum activeDuration-r18 to 2000ms and cycleLength-r18 to 40960 ms, provided that RRM requirements are not affected.

Agree to the attached LS provided in the Annex.

Conclusion

The following Observations and Proposals were made:

Observation 1: Not all UEs will be harmed by the potential interference between UL and GNSS frequencies.

Observation 2: RAN2 will consider enhancements to the IDC framework in Release 19 only. No changes will be done for Release 18.

Proposal 1: RAN4 shall only consider any changes regarding this issue in Release 19.

Proposal 2: Increase the maximum activeDuration-r18 to 2000ms and cycleLength-r18 to 40960 ms, provided that RRM requirements are not affected.

Proposal 3: Agree to the attached LS provided in the Annex.

- References

- R4-2502281, Way Forward for [113][310] NR_NTN_Ph3_UE_RF, Qualcomm, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.

- R4-2503119, Response LS on simultaneous operation between GNSS and NR NTN, Nokia, RAN4#114bis, Wuhan, China, 07th – 11st April, 2025.

- Annex

Title:	draft LS on NR NTN co-existence with GNSS

Response to:

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact person:

Name:	Alok Sethi

E-mail Address:	alok.sethi@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 would like to thank RAN2 for the LS reply in R2-2501568. RAN4 has discussed possible values for the periodicity and duration of the UL NTN transmission gap which may help the UE in obtaining the GNSS. The possible additional values are listed below:

- 2000ms for activeDuration-r18,

- 40960ms for cycleLength-r18

# Actions

ACTIONS:

To RAN WG2 and WG1: RAN4 respectfully asks RAN2 use the above mentioned values for modifying the IDC framework.

# Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #115	May 19 - 23, 2025	Malta

TSG-RAN4 Meeting #116	Aug 25 - 29, 2025	Bangalore, India