3GPP TSG-RAN WG4 Meeting #114bis	R4-2504423

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on the LS Reply regarding DMRS puncturing in NB-IoT over NTN

Agenda item:	7.30.2

Document for:	Discussion

Introduction

In RAN1 #120, RAN1 has evolved on the discussions regarding the implementation of OCC feature in NB-IoT over NTN, which is one of the key objectives in the Rel-19 WI IoT_NTN_Ph3-Core. However, they have reached multiple options in how to guarantee orthogonality of the DMRS for two different UEs that share the same resources when applying OCC 2.

In order to fully assess one of the DMRS allocation options, RAN1 has sent a LS to RAN4 (R1-2501622). In this document we provide our view on the RAN1 question on the feasibility of such feature.

Discussion

In their LS (R1-2501622) to RAN4, RAN1 has asked the following:

For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT.. Question: From a RAN4 perspective, is it feasible to introduce support of TDM DMRS, as per the description above, in Rel-19?

The question regards the DMRS solution presented in Figure 1. In this figure, each row represents a different UE. It is possible to see that the goal is to divide the transmission in bursts of 4 slots (“superslot”) where in two of them one of the UEs will transmit a DMRS sequence whereas the other UE will “mute” (blank) the symbols where the DMRS should be located. In the other two DMRS opportunities within the slot, the opposite will happen.

Figure 1. Example of “muting” patterns of DMRS sequences

RAN1 has asked whether the above is “feasible”. RAN4 has not fully assessed with simulations the performance of different DMRS spacing in NTN channel, which tends to vary slower than most of the macro terrestrial channels, due to a dominant component of line of sight and very low delay spreading (5 ns). However, it is expected that with smaller DMRS density the channel estimation at the UE will get poorer. In the best case scenario, i.e. when a flat channel is considered, a reduced number of transmitted sequences for channel estimation will perform on par when compared to a case where more DMRS sequences is transmitted.

UE channel estimation will suffer because of reduced number of DMRS symbols.

In the example above, instead of a density of one DMRS transmission every 7 symbols, there is variable DMRS density across the different slots. Between the second and the third DMRS symbols transmitted by each UE, there are 21 symbols instead of only seven. This will certainly account for some performance loss. On the other hand, the system seems feasible from a technical perspective, especially considering that the performance loss might not be very significant in some scenarios. The performance loss will be dependent on the fast fading behavior of the channel.

Another point to be considered for feasibility, is that in the example above, the channel for UE2 cannot be reliably estimated until the fourth symbol is received by the eNB. Given the use of OCC, network must buffer four NPUSCH slots before it can process the payload and this will not only increase the complexity but also add delay to the HARQ response.

There will be additional complexity at the network side because of the need to buffer the payload for at least four slots before network can start decoding.

Phase continuity is one important issue that has been discussed in RAN1 for this case. When TDM muting is performed, the phase continuity that has important function for decoding reliability, high coverage with high energy efficiency, will be impacted, especially for single-tone transmissions.

DMRS muting will impact the phase continuity that can have additional impact on the decoding reliability.

Based on the above, we propose:

More discussing is required in RAN4 before concluding the feasibility of TDM DMRS solutions informed by RAN1 given the additional performance loss and increased gNB complexity.

Conclusion

The following Observations and Proposals were made:

Observation 1: UE channel estimation will suffer because of reduced number of DMRS symbols.

Observation 2: There will be additional complexity at the network side because of the need to buffer the payload for at least four slots before network can start decoding.

DMRS muting will impact the phase continuity that can have additional impact on the decoding reliability.

Observation 3:

Proposal 1: More discussing is required in RAN4 before concluding the feasibility of TDM DMRS solutions informed by RAN1 given the additional performance loss and increased gNB complexity.

- References

- R1-2501622, LS on TDM DMRS for 3.75KHZ SCS OCC, Sony, RAN1#120, Athens, Greece, February 17 – February 21, 2025.