3GPP TSG-RAN WG4 Meeting #114bis	R4-2504441

Wuhan, China, April 7 – 11, 2025

Source:	Nokia

Title:	NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC

Agenda item:	7.18.3

Document for:	Discussion

Introduction

Several agreements for Rel-19 NR demodulation performance Phase 5: BS Demodulation MMSE-IRC have been made during RAN4#114 meeting. They are mainly on issues related to test parameters and simulation assumptions as documented in the way forward (WF) [1], and summarized below:

Sub-topic 2 Test parameters and simulation assumptions Channel bandwidth and waveform <Agreement> - For CP-ODFM: - For 15kHz SCS, cover 5MHz, 20MHz - For 30kHz SCS, cover 10MHz, 100MHz - Not to cover DFT-s-OFDM MCS <Agreement> - MCS 16 for 2Rx (if introduced), 4Rx and 8Rx DMRS configuration - whether to include DMRS configuration type 2 <Agreement> - Not to consider DMRS configuration type 2 Test requirement definition <Agreement> - Use the SNR based requirement definition

In addition to the agreements, [1] also documented the remaining open issues to be discussed in RAN4#114bis. This contribution describes our views on those issues by considering our simulation results presented in [2].

Discussion

Referring to the WF [1], the remaining open issues are classified into 3 subtopics, namely, Interference modelling, Test parameters and simulation, and Manufacturer declarations and applicability rules. In the following, our views on the remaining open issues are provided in each of the respective subsections.

Interference Modelling

The remaining open issue in this subtopic is on the number of interferers for 2Rx:

Number of interferers for 2Rx - For HetNet scenario, introduce BS MMSE-IRC requirements with 2Rx. - For the number of interferers for 2Rx under HetNet, further decide between the following: - Option 1: 2 interferers - Option 2: 1 interferer - For HomNet scenario, further decide among the following: - Option 1: 2 interferers - Option 2: 1 interferer - Option 3: Not to consider 2Rx requirements for HomNet scenario. - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

2Rx HetNet scenario

As stated in TR 36.884 [10], there is no significant SINR gain of MMSE-IRC to the baseline MMSE receiver in the case of 2Rx when the explicit interferer number is increased from 1 to 2. This is regardless of the scenarios (HomNet or Hetnet). As there is no significant SINR gain when the explicit interferer number is increased from 1 to 2, TR 36.884 [10] (and TS 36.104 [12]) considered only 1 interferer for 2Rx.

Our simulation results in [2] (and in [3]) confirms that, similar to TR 36.884 [10], for 2Rx case, there is no significant difference in the SNR gain when the interferer number is increased from 1 to 2.

The requirements for 2Rx in TR 36.104 are considering only 1 interferer as there is no significant SINR gain when the explicit interferer number is increased from 1 to 2.

Simulation results show that there is also no significant SNR gain in NR for 2Rx case when the interferer number is increased from 1 to 2.

RAN4 to consider 1 interferer for Hetnet scenario with 2Rx.

2Rx HomNet scenario

Referring to TR 36.884 [10], the least SINR gain observed in LTE is 3.1 dB, which is the gain of 2Rx in Homogeneous Network (HomNet), as highlighted below:

## 7.3 Summary of link level evaluation According to the above simulation campaign, the link performance gain of MMSE-IRC over MMSE can be summarized as follows: - With respect to homogeneous network, for 1x2 cases the gain is about 3,1 dB; for 1x4 cases the gain is 4,4 dB ~4,7 dB; for 1x8 cases the gain is 4,9 dB ~5,0 dB. - With respect to heterogeneous network, for 1x2 cases the gain is 5,5 dB~5,9 dB; for 1x4 cases the gain is 7,5 dB~8,3 dB; for 1x8 cases the gain is 8,4 dB~8,9 dB. Therefore, it is concluded that the MMSE-IRC receiver can bring in the significant gain over MMSE receiver.

As highlighted above, TR 36.884 [10] concludes that with the minimum SINR gain of 3.1 dB (in 2Rx HomNet), a significant SINR gain is observed. Hence, the requirements for MMSE-IRC were made in TS 36.104 [12]. While there is no specific threshold of SINR gain mentioned in TR 36.884 [10], nonetheless, it is implicitly documented that the minimum SINR gain observed is 3.1 dB, which is for 2Rx in HomNet.

In RAN4#114, companies have provided their simulation results, which were collected in [4]. In the case of 2Rx, for HomNet, the SNR gain is less than 3 dB (even the majority of results showing less than 2 dB gain), regardless of the interferer number.

In this meeting, our simulation results in [2] are again showing that the SNR gains for 2Rx are less than 2 dB in the HomNet cases. Even, with TDLB100-400, the SNR gain could fall below 1 dB, with the worst SNR gain of only 0.3 dB.

In defining the demodulation performance requirements, it is common in RAN4 to add an impairment margin on top of the average SNR of the ideal simulation results from participating companies. The additional impairment margin can be around 3 dB. Hence, the amount of impairment margin itself can already cancel out the SNR gain of MMSE-IRC over MMSE for the 2Rx. Consequently, the final requirements for the 2Rx HomNet could not separate/differentiate the MMSE-IRC from MMSE.

In TR 36.884, the observed minimum SINR gain of all cases is 3.1 dB, which is the SINR gain of 2Rx in HomNet.

Simulation results show that the SNR gain of 2Rx in HomNet is less than 2 dB, regardless of the interferer number. Even in TDLB100-400, the SNR gain could fall below 1 dB with the worst SNR gain of only 0.3 dB.

In RAN4, it is common to add an impairment margin, which can be around 3 dB, to the ideal simulation results. Such an amount of impairment margin can cancel out the SNR gain of MMSE-IRC over MMSE if the SNR gain is less than 2 dB.

RAN4 do not specify requirements for HomNet 2Rx as the SNR gain is considerably low.

Test parameters and simulation assumptions

The open issue to be addressed in this subtopic is propagation condition as below:

Sub-topic 2 Test parameters and simulation assumptions Propagation condition for the serving cell - Select between the following options: - Option 1: TDLA30-10 for HetNet and TDLB100-400 for HomNet - Option 2: TDLC300-100 for all cases - Option 3: TDLA30-10 for HetNet and TDLC300-100 for HomNet (Proposed during RAN4#114 ad-hoc session as a compromise between option 1 and option 2) - Companies are encouraged to bring results for all options above and this issue will be concluded in the next meeting.

In LTE (in [10], Table 7.1-2: Cases for link level evaluations), the studies on the gain of MMSE-IRC over MMSE were performed using the same propagation condition for both the intended cell and the interfering cell, namely, (EPA5, EPA5) and (EVA70, EVA70), for HetNet and HomNet, respectively. However, for the requirements in [12], a different propagation conditions for the intended cell and interfering cell are used, namely, (EPA5, ETU5) and (EVA70, ETU70).

If we refer to TS 38.104 [13] (e.g., Table 8.2.1.2-1), however, the propagation condition is chosen based on the MCS, namely, MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10. Hence, if we follow [13], as MCS16 has already been agreed in RAN4#114 as the MCS for the intended user, the propagation condition would be TDLC300-100.

Furthermore, in RAN4#113, for the interference signals, the following were also already agreed: MCS16 with propagation condition of TDLC300-100. This confirms that the propagation condition of the interfering signals is aligned with TS 38.104 [13], which is based on the MCS.

In LTE, to study the MMSE-IRC gain in TR 36.884, the propagation conditions were chosen according to the network type.

In NR, in TS 38.104, the propagation condition is chosen based on the MCS.

The propagation condition of the interfering signal agreed in RAN4#113 is aligned with TS 38.104, which is TDLC300-100 for MCS16.

MCS16 has been agreed to be used for the intended signal for all Rx cases, which is the same as the interfering signal. The interfering signal itself is using TDLC300-100 for all scenarios.

The SNR gain of TDLB100-400 can fall below 1 dB (even only 0.3 dB) in 2RX case.

To align with TS 38.104 and be consistent in choosing the propagation condition as for the interfering signal, RAN4 to set the propagation condition of the intended signal to be TDLC300-100.

Manufacturer declarations and applicability rules

The remaining open issues in this subtopic are below:

Sub-topic 3 Manufacturer declarations and applicability rules Manufacturer declaration on MMSE-IRC receiver - Candidate options: - Proposal 1: Introduce new manufactory declaration for BS MMSE-IRC receiver Declaration identifier; Declaration; Description; Applicability. BS type 1-C; BS type 1-H. D.xxx; MMSE-IRC receiver; Declaration of BS supporting MMSE-IRC receiver.; x; x. - Proposal 2: Do not need to introduce a new Manufacturer Declaration on MMSE-IRC Applicability - Candidate options on test requirement applicability for different BS classes and test scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS and HetNet requirements are not applicable for Wide and Medium Area BS - Other options are not precluded. - Based on test requirement applicability, further discuss whether to consider additional applicability rule on how the test is performed: - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: If a BS declared to support HomNet or HetNet and pass the tests for HomNet, can also consider the test for HetNet as passed - Proposal 3: Do not define additional applicability rule for homogenous and heterogenous network scenarios

Manufacturer declarations

As Rel-19 NR BS demodulation performance for MMSE-IRC is directly reusing the DIP from LTE, ideally, it should follow what was done in LTE concerning any related manufacturer declaration. Furthermore, as MMSE-IRC is only one form of linear interference-rejection (or interference-cancellation) algorithms, there is no need to have a manufacturer declaration which specifically mention MMSE-IRC algorithm.

In LTE, it is stated in 36.106 Section 8.2.6 as follows: “The requirements apply to the BS supporting the enhanced performance requirements type A.”

Instead of introducing a new Manufacturer Declaration on MMSE-IRC, RAN4 to follow LTE case by stating: “The requirements apply to the BS supporting the enhanced performance requirements”.

Applicability

Referring to LTE in [12], the “Note*: Not applicable for Local Area BS and Home BS.” inside the tables 8.2.6.1-1 to 8.2.6.1-6 implies that the HomNet requirements are not applicable for Local Area BS and Home BS.

In TS 36.104 for LTE, the note in the requirement tables implies that HomNet requirements are not applicable for Local Area BS and Home BS.

As in LTE, RAN4 to adopt the following for NR: “HomNet requirements are not applicable for Local Area BS.” Furthermore, RAN4 to add the following: “HetNet requirements are not applicable for Wide and Medium Area BS”.

Conclusion

In this contribution, we provide our views on Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC. The following Observations and Proposals were made:

Observation 1: The requirements for 2Rx in TR 36.104 are considering only 1 interferer as there is no significant SINR gain when the explicit interferer number is increased from 1 to 2.

Observation 2: Simulation results show that there is also no significant SNR gain in NR for 2Rx case when the interferer number is increased from 1 to 2.

Proposal 1: RAN4 to consider 1 interferer for Hetnet scenario with 2Rx.

Observation 3: In TR 36.884, the observed minimum SINR gain of all cases is 3.1 dB, which is the SINR gain of 2Rx in HomNet.

Observation 4: Simulation results show that the SNR gain of 2Rx in HomNet is less than 2 dB, regardless of the interferer number. Even in TDLB100-400, the SNR gain could fall below 1 dB with the worst SNR gain of only 0.3 dB.

Observation 5: In RAN4, it is common to add an impairment margin, which can be around 3 dB, to the ideal simulation results. Such an amount of impairment margin can cancel out the SNR gain of MMSE-IRC over MMSE if the SNR gain is less than 2 dB.

Proposal 2: RAN4 do not specify requirements for HomNet 2Rx as the SNR gain is considerably low.

Observation 6: In LTE, to study the MMSE-IRC gain in TR 36.884, the propagation conditions were chosen according to the network type.

Observation 7: In NR, in TS 38.104, the propagation condition is chosen based on the MCS.

Observation 8: The propagation condition of the interfering signal agreed in RAN4#113 is aligned with TS 38.104, which is TDLC300-100 for MCS16.

Observation 9: MCS16 has been agreed to be used for the intended signal for all Rx cases, which is the same as the interfering signal. The interfering signal itself is using TDLC300-100 for all scenarios.

Observation 10: The SNR gain of TDLB100-400 can fall below 1 dB (even only 0.3 dB) in 2RX case.

Proposal 3: To align with TS 38.104 and be consistent in choosing the propagation condition as for the interfering signal, RAN4 to set the propagation condition of the intended signal to be TDLC300-100.

Observation 11: In LTE, it is stated in 36.106 Section 8.2.6 as follows: “The requirements apply to the BS supporting the enhanced performance requirements type A.”

Proposal 4: Instead of introducing a new Manufacturer Declaration on MMSE-IRC, RAN4 to follow LTE case by stating: “The requirements apply to the BS supporting the enhanced performance requirements”.

Observation 12: In TS 36.104 for LTE, the note in the requirement tables implies that HomNet requirements are not applicable for Local Area BS and Home BS.

Proposal 5: As in LTE, RAN4 to adopt the following for NR: “HomNet requirements are not applicable for Local Area BS.” Furthermore, RAN4 to add the following: “HetNet requirements are not applicable for Wide and Medium Area BS”.

- References

- R4-2502272, Way Forward for [114][323] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#114, China Telcom, Athens, Greece, January 17-21, 2025

- R4-2504442, Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC, 3GPP RAN4#114bis, Nokia, Wuhan, China, April 7-11, 2025

- R4-2501843, Simulation Results – NR BS Demodulation Performance Requirements for MMSE-IRC, 3GPP RAN4#114, Nokia, Athens, Greece, February 17-21, 2025

- R4-2502028, Simulation results summary for BS requirement with MMSE-IRC receiver, 3GPP RAN4#114, Samsung, Athens, Greece, February 17-21, 2025

- R4-2413570, WF on NR demodulation performance: Phase 5, 3GPP RAN4#112, China Telcom, Maastricht, Netherlands, August 19-23, 2024

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#112bis, China Telcom, Hefei, China, October 14-18, 2024

- R4-2419780, Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#113, China Telcom, MediaTek, Orlando, USA, November 18-22, 2024

- R4-2418934, Simulation results summary for BS requirement with MMSE-IRC receiver, 3GPP RAN4#113, Samsung, Orlando, USA, November 18-22, 2024

- R4-2416442, NR Demodulation Performance Phase 5: BS demodulation performance requirements for MMSE-IRC, 3GPP RAN4#112bis, Nokia, Hefei, China, October 14-18, 2024

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016

- R4-2415994, Simulation results summary for BS requirement with MMSE-IRC receiver, 3GPP RAN4#112bis, Samsung, Hefei, China, October 14-18, 2024

- 3GPP TS 36.104, Evolved Universal Terrestrial Radio Access (E-UTRA) - Base Station (BS) radio transmission and reception, v18.5.0, 03-2024

- 3GPP TS 38.104, NR - Base Station (BS) radio transmission and reception, V18.6.0, 06-2024

- 3GPP TS 38.101-4, NR - User Equipment (UE) radio transmission and reception, Part 4: Performance requirements, V18.6.0, 12-2024