3GPP TSG-RAN WG4 Meeting #114bis	R4-2504445

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	SSB index detection requirements in Less than 5 MHz operation in NTN.

Agenda item:	7.10.3.4

Document for:	Discussion

Introduction

In RAN #103, RAN plenary has approved the RAN 4 WI for NTN features [1]. The WI was then updated in RAN #104 [2]. One of the topics for this WI is the operation of NTN in less than 5 MHz. For the most part, the core requirements for this objective was finalized in the previous meetings, with only one remaining open issue to be agreed in this meeting:

Issue 1-1-1: Measurement(SSB index detection requirements) Agreements: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by [3 or 4] and [3] for intra-frequency and inter-frequency cases respectively. Notes: the final value can be confirmed in the next meeting and the corresponding value will be updated in the draftCR.

Discussion

Our position relative to the issue described in the introduction has always been that in NTN the measurements are already delayed by several factors (such as DL coverge enhancement feature to be introduced in Release 19 or the meausrements across multiple satellites). As such, we always strived for requirements that would guarantee the UE to complete the measurements in timely manner.  Specially considering that in the case of NTN the satellite might be available for very limited span of times, and in certain scenarios (such as earth moving cells) a significant amount of mobility events are to be conducted, and shortened interruption times are preferred.

As companies have previously not agreed on our proposals to have a secondary set of requirements, with shorter requirements to complete the measurements, we would go for the compromise that intra-frequency measurements should only be extended by 3 samples as we..

Both, intra and inter-frequency measurements to be extended by 3 samples for the SSB index detection requirements.

Conclusion

In this paper we discussed the last open issue in the WF from previous meeting, related to the SSB index detection requirements for UEs operating in less than 5 MHz. The following proposal was made.

Proposal 1: Both, intra and inter-frequency measurements to be extended by 3 samples for the SSB index detection requirements.

- References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair (Huawei), RAN #103, Maastricht, Netherlands, March 18-21, 2024

- RP-241985, " Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105