3GPP TSG-RAN WG4 Meeting #114bis	R4-2504446

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	RRM Performance Requirements for less than 5 MHz over NTN

Agenda item:	7.10.3.5

Document for:	Discussion

Introduction

In RAN #103, RAN plenary has approved the RAN 4 WI for NTN features [1]. The WI was then updated in RAN #104 [2]. One of the topics for this WI is the operation of NTN in less than 5 MHz whose objectives are presented below:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

Throughout the first meetings in this WI good progress was achieved by reusing most of the requirements from the less than 5 MHz WI for terrestrial networks with the necessary adaptation for NTN. There are a few open issues left, one of them, which is the topic of this document is related to requirements for SSB time index detection.

Discussion

The principle used for Less than 5 MHz test cases in terrestrial networks (Rel-18) was that, the performance requirements (the definition of test cases) should rely as most as possible on legacy requirements, and only the differential (delta) of the test cases shall be introduced. The only test cases introduced were:

| TC # | Description |
| --- | --- |
| IDLE-1 | Cell reselection, Unknown target cell |
| HO-1 | SA FR1-FR1 Handover, Intra-frequency, Unknown target cell |
| RLM-1 | Radio Link Monitoring (SSB-based, FR1): DRX, Out-of-sync, 12 PRBs |
| RLM-2 | Radio Link Monitoring (SSB-based, FR1): DRX, In-sync, 12 PRBs |
| RLM-3 | Radio Link Monitoring (SSB-based, FR1): Non-DRX, Out-of-sync, 15 PRBs |
| RLM-4 | Radio Link Monitoring (SSB-based, FR1): Non-DRX, In-sync, 15 PRB |
| BFD-1 | BFD and link recovery (SSB-based, FR1): DRX, 15 PRB |
| MR-1 | SA event triggered reporting, SSB based, Time period for time index detection: Intra-frequency, non-DRX, no gaps, 15 PRBs |
| MR-2 | SA event triggered reporting, SSB based, Time period for time index detection: Inter-frequency, DRX, gaps, 15 PRBs |

Observe that some of the test cases above are “duplicated” to cover for operations within 12 and 15 PRBs.  But in RAN 4 WF agreed on R4#112bis, the following compromise agreement was reached:

#### Issue 1-1-1-1: Channel bandwidth assumption Agreements - 3MHz with 15 PRBs #### Issue 1-1-1-2: SSB/PBCH assumption Agreements - 12 PRBs #### Issue 1-1-1-3: PDCCH transmission bandwidth assumption Agreements - 15 PRBs

Therefore it is our view that the test cases shall be limited to 15 PRB cases. Besides, there are no NTN requirements related to BFD. Therefore, we propose:

Add test cases for Cell Reselection, Handover, Soft/Hard Satellite switching, Radio Link Monitoring for less than 5 MHz over NTN.

Add a description on Annex  A.3.36.2	of TS 38.133 about the applicability of such test cases.

Conclusion

In this paper, we provided our views on the performance test cases of Less than 5 MHz operation in NTN by means of the following proposals:

Proposal 1: Add test cases for Cell Reselection, Handover, Soft/Hard Satellite switching, Radio Link Monitoring for less than 5 MHz over NTN.

Proposal 2: Add a description on Annex  A.3.36.2	of TS 38.133 about the applicability of such test cases.

- References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair (Huawei), RAN #103, Maastricht, Netherlands, March 18-21, 2024

- RP-241985, " Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105