3GPP TSG-RAN WG4 Meeting #114bis	R4-2504448

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on IoT NTN TDD requirements

Agenda item:	7.31.5

Document for:	Discussion

Introduction

In the plenary meeting RAN #105, a new WI for Rel-19 was approved to introduce a TDD frame operation for NTN [1]. The work item is designed to provide extended support of 3GPP NB-IoT features leveraging existing satellite systems deployed in Non-Geostationary Satellite Orbit (NGSO). It also aims at enabling low-complexity payloads for satellite operations that are non-reliant on a diplexer.

To achieve the aforementioned goals, the WID was designed to introduce an NTN-compatible TDD frame, that while based on minimum necessary changes on the NB-IoT NTN FDD specification provides a new approach to TDD periodicity.  RAN4 has analysed the potential impacts of such frame and provided their view on [2].

In the plenary meeting RAN #106, the WID was updated to provide the objectives for RAN4 based on identified challenges [3]. In this document we discuss the potential RRM impacts.

Discussion

In the previous RAN4 meeting, several technical aspects were raised pointing for the need to update requirements in TS 36.133 when the UE is using the newly introduced TDD pattern for NTN in the 1616-1626.5 MHz MSS allocated band. In this contribution we will address some of the open issues and provide our views and proposals regarding those.

General Aspects

Several general aspects were discussed in the previous meeting, including discussions on the applicable coverage enhancement (whether apply it or not) and if so, the maximum number of repetitions to be considered. Another important aspect discussed was related to the applicable DRX Cycle and the current patterns and how they might impact the measurement requirements once they are not aligned with the transmission patterns of the cell.

## Issue 0: General aspects Agreement (online): For TDD NB-IoT in NTN, the following aspects will be further considered in future RAN4 meetings: - The target SNR - Applicable coverage (whether the EC needs to be considered) - Applicable Rmax - Applicable DRX cycle - Note: The DRX cycle might not be non-uniformly periodic due to the H-SFN not matching with the new TDD pattern. This issue will be addressed by other working groups. In the meantime, RAN4 assumes the DRX cycle is uniformly periodic.

Coverage Enhancement

- In the previous meeting, RAN4 had no visibility from the other working groups (RAN1 and RAN2) whether DL coverage enhancement features were part of the scope of work and whether we should take them into consideration when creating the requirements. This situation was clarified in RAN2 # 129, where RAN2 has reached the following agreement:

RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

Therefore, the legacy coverage enhancement feature is part of the scope of the current work item, and therefore should be addressed accordingly. However, due to the difference of the frame pattern, which provides “active” and “non-active”  times in the serving cell, there are still question marks regarding the “segmentation”, transmission gaps, and the maximum number of repetitions to be considered during a DL or UL transmission.

RAN1 has assumed tacitly that Coverage Enhancement feature applies for NB-IoT NTN TDD, but with no agreements yet whether a transmission pattern goes beyond one or more DL/UL occasions:

Agreement For segmented pre-compensation, RAN1 to further discuss at least the following options: - Option 1: Dropped/postponed NPUSCH / NPRACH are counted for segment determination - Option 2: A new pre-compensation segment is started at the beginning of each uplink burst - Option 3: UL segment should only be defined in consecutive U subframes - Option 4: Segmented pre-compensation does not apply for NB-IoT NTN TDD. - FFS: How to handle the UL segment gap.

Based on the above, it is our view that:

Coverage Enhancements requirements are applicable for NB-IoT NTN TDD.

Wait for further RAN1 definition to decide whether the requirements for coverage enhancements require updating.

Applicable DRX Cycle

Neither RAN1 nor RAN2 has fully addressed the problem that the DRX Cycle periodicities and the frame pattern periodicities are not multiple of each other, and therefore, there might be mismatch between the active periods of the UE and the DL opportunities used by the serving cell. For example, for the case of cell evaluation for cell reselection, where DRX Cycles may have periodicities that are multiple of 320 ms, and therefore, non-multiple of 90 ms. This would indicate that the active part of the DRX Cycle slips slightly over time every cycle of N frames and cannot be matched with the active time on this cell.  However, RAN2 has identified that there might be some problems that need addressing regarding the (e)DRX cycles:

Agreements:

- RAN2 will continue studying paging aspects based, on RAN1 progress

- RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

- RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

- RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

Wait for RAN1/2 progress related to the mismatch between (e)DRX cycle and the frame pattern.

Cell Reselection.

In Release 18, it was established that the cell reselection requirements are only applicable if the UE is provided with satellite assistance information for the neighbor cells in NGSO scenarios. The reason behind this agreement is that, due to the large differential doppler pre-compensation, the UE is not expected to find the neighbor cells in due time in the absence of further information that allows for the UE to find the neighbor satellite.

When the new TDD pattern is introduced in NTN, a cell active time in DL corresponds to less than 10% of the total operation time. Therefore, if the UE is not informed about the DL times of the said cell, it might not be capable of tracking (tuning) the measurements in time to find this cell within a reasonable window of time. Because of this, we think it should be clarified that the UE shall be provided with neighbor cell information relative to the active times.

The UE shall be informed about the active periods of the neighbor cells configured to be measured in the new TDD frame pattern.

Another problem is related to the scaling factors used in NTN when adapted to the new frame pattern, because the Doppler may be significantly different between serving and neighbor satellites in LEO (NGSO) scenarios, the Cell Reselection measurements have been adapted to account for this difference.  In the Cell Reselection requirements, the total time for the UE to complete the measurements is scaled by Ksatellite, where Ksatellite is defined as the scaling factor for measurements corresponding to multiple NGSO satellites.

The scaling factor was introduced to account for the fact that, because of the significant Doppler difference between serving and target satellites to be measured, the UE can either measure one or the other, which would lead to “shared” measurement opportunities. In this sense it makes sense to expand the time to detect (and to measure, and to evaluate) a cell, based on the fact that the measurement opportunities are split.

However, in the new TDD frame pattern, this no longer happens if the cells to be measured belong to non-overlapping DL transmission opportunities. For example, in Figure 1, consider a UE that is allocated to DL-UL 1 on the serving satellite, but enabled to also measure DL 2 of the neighbor satellite. From this image, it is clear that there is no need to split the opportunities across the different satellites.

Figure 1

Figure 1

Adopt Ksatellite equal to 1 for the cells measured in DL opportunities of a neighbor satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite.

On the scaling factor related to the punctured frame pattern.

There is also some consideration on whether there is a need to scale the time to detect/measure/evaluate due to the fact that there are less PSS/SSS opportunities for the UE to measure for the same amount of time (1 every 90 ms instead of 1 every 10 ms (PSS) or 1 every 20 ms(SSS)). However, the current numbers on the specification were based on simulation results back in Release 13. For this case the number of DRX cycles used to define the measurement requirements assumed that the number of instances of NPSS and NSSS per DRX cycle is equal to [5] :

  - DRX Cycle 1.28s: 5

  - DRX Cycle: 2.56s: 10

  - DRX Cycle 5.12s: 10

  - DRX Cycle: 10.24s: 20

In the 1.28 s cycle, there are at least 14 occasions of DL transmission where the UE might be capable of acquiring the NPSS and NSSS, which is more than the minimum assumption above. Assuming this is the most stringent case, then,

The UE has enough opportunities to acquire NPSS and NSSS within a DRX Cycle, according to the simulation assumptions.

Cell Measurements in Connected Mode

Similarly to the issue discussed in clause 2.2.1 of the present document, there were also discussions related to the cell measurement in connected mode, where the proposals were to extend the measurement period due to the diminished number of opportunities for the UE to acquire the NPSS and NSSS of the target cell.

## Issue 2: Cell measurement in connected mode (8.14A) Further discuss the following: For the cell measurement period in connected mode for TDD NB-IoT in NTN, - For intra-frequency measurement, the delay requirements shall be scaled by N=9 - For inter-frequency measurement, the minimum length of measurement occasions is defined as the max {existing value, periodicity of corresponding RS}. - Revise the existing cell measurements condition “at least one 1 DL subframe per radio frame has to be available at the UE during the measurement period.” - Serving cell measurement requirements apply provided the UE is provided with the configuration about the “active DL” periods.

Intra-frequency measurements

DRX not configured

When the DRX is not configured, the requirements are:

- Tdetect_intra = 1400 ms

- Tmeasure_intra = 800 ms for NRS-based measurement

- Tmeasure_intra = 1600 ms for NSSS-based measurement

First of all, the detection procedure depends on UE identifying the PSS and SSS of the cell to be measured.  From our understanding, the PSS has periodicity of 10 ms, and therefore, it is always available during the DL opportunities of each cell when the new TDD frame pattern is utilized. There will be at least 15 PSS occasions within 1400 ms.

During link budget analysis, RAN1 has performed investigations on the PSS and SSS detection, and the following results were obtained:

RAN1 makes the following observations on downlink synchronization: Case 1: For N=9, D=8: - For NPSS: - For one-shot detection (90ms delay), 9 sources provided simulation results, with a median required SNR of 1.30dB. The link budget margin is (median margin): - 4.21 dB for LEO-1200 (0dBi antenna gain) - -1.89 dB for LEO-600 (-5.5dBi antenna gain). - For two-shot detection (180ms delay), 3 sources provided simulation results, with a median required SNR of -1.20dB. The link budget margin is (median margin): - 6.71dB for LEO 1200 (0dBi antenna gain) - 0.61dB for LEO-600 (-5.5dBi antenna gain). - For four-shot detection (360ms delay), 3 sources provided simulation results, with a median required SNR of -3.80dB. The link budget margin is (median margin): - 9.31dB for LEO-1200 (0dBi antenna gain) - 3.21dB for LEO-600 (-5.5dBi antenna gain). - NOTE: The assumptions for combining for 2-shot and 3-shot detection are different across companies. - For NSSS: - For one-shot detection (180ms delay), 5 sources provided simulation results, with a median required SNR of -2.50dB. The link budget margin is (median margin): - 8.01dB for LEO-1200 (0dBi antenna gain) - 1.91dB for LEO-600 (-5.5dBi antenna gain)

For 3 shot detection of the PSS, the median SNR required according to the simulation results is -3.8 dB. It is well known that RAN4, when designing the minimum requirements commonly utilize -6 dB as the threshold value for the measurement requirements. We also acknowledge that we miss simulation results for the NTN channel model in this WI for such SINR. However, the results above it shows that with several PSS opportunities, it is likely that PSS can be successfully collected. Likewise, for SSS, one-shot detection was possible with a required SINR of -2.5 dB. Within a few SSS opportunities there should be possible to also detect the SSS. At current stage we see no need to further increase the time to detect based on the PSS and SSS detectability.

For the time to measure, within 800 ms there are up to 10 NRS opportunities and within 1600 ms there are up to 10 NSSS opportunities. Therefore, we see no reason to extend the time to measure as well.

However,  as there is diminished opportunities for measurement, it means that the UE should wake up at every DL opportunity of the cell to be measured. If the UE has to measure cells across multiple DL opportunities (see Figure 1), the UE will be awake for most of the time just to collect measurements of the cells to be measured, which goes against the principle of NB-IoT to limit power consumption. Because of this, as a compromise, we can scale the measurements when the UE has to measure 3 or more different DL opportunities, in order to give the UE more chance to save power and space its measurements in time.

Do not increase the Tmeasure and Tdetect for intra-frequency measurements when DRX is not configured.

As a compromise, scale Tmeasure and Tdetect , when DRX is not configured, by a factor of [2], when the UE measures cells across 3 or more different patterns of DL opportunities.

DRX configured

When DRX is in use, the time to detect is set to 6 DRX Cycles, whereas the time to measure is set 5 DRX Cycles. In special for the largest DRX Cycles, the number of measurement opportunities is sufficient for the UE to acquire the information of neighbor cells with no need for extending the minimum requirement. However, the main goal when DRX is configured is to save UE power, and for the smallest DRX Cycles, the UE will have to rely on almost every transmission opportunity to have sufficient number of samples to detect or measure the target cell. Using the same principle as above, we propose:

Scale Tmeasure and Tdetect, when DRX is configured, for intra frequency measurements by a factor of [2] when the UE measures cells across 3 or more different patterns of DL opportunities.

Inter-frequency measurements

For the inter-frequency measurements, requirements state that:

- The UE is not required to monitor NPSS/NSSS more frequent than once per 40ms. (Time to Detect) - The UE is not required to monitor NRS more frequent than once per 20ms for NRS-based measurement and NSSS more frequent than 40 ms for NSSS-based measurement. (Time to measure)

Therefore, increasing the interval between collected samples to 90 ms, would not represent an increase in the scaling factor by a factor of 9. At worse case scenario, that would represent an increase of 4.5 times in the NRS based measurements. Hence, increasing the time by a scaling factor of 9 cannot be accepted.

Besides, the requirements were already established based on the number of samples to be collected (factors N and M in the respective formulas for time to detect and time to measure). Therefore, they scale naturally with the time for the UE to collect the samples. So it is natural that assuming that the number of samples do not need to be enlarged, we propose that:

Do not scale Tmeasure and Tedetect for inter-frequency measurements.

For the length of the measurement occasions, the pre-conditions for the requirements applicability also establish that:

- Length of MOdetect_inter_NB1-NC is at least 200 ms. - Length of MOmeasure_inter_NB1-NC is at least 50 ms.

For the measurement window opportunities of the time to detect, we believe it can be made smaller than 200ms. As there are more well defined opportunities for measuring in the new frame pattern (due to inactivity in the serving cell), it might be useful to establish smaller MO to avoid overlapping with the DL opportunities of the serving cell, while allowing. One potential approach is to have the minimum length of measurement equal to the interval between the end of one DL occasion X (X=1,2,3 or 4) to the start of the next DL occasion X.

For inter-frequency measurement, the minimum length of measurement occasions is [82] ms.

Measurement conditions

For all cases above, regardless whether the requirements are scaled or not, the UE might not be capable of tracking the neighbor cells (each with a DL active time inferior to 10% of the TDD frame) if it is not provided firstly with assistance information on the DL periodic patterns.

Requirements are applicable if the cells is provided with assistance information on the DL periodic patterns of the cells to be measured.

Another issue is related to the side conditions for the signals to be measured. In Clause 8.14A, it is established that the measurements of the serving cell are applicable provided that some side conditions are fulfilled:

The UE shall meet all applicable requirements specified in clause 8.14A under the following conditions: - at least 1 DL subframe per radio frame of serving NB-IoT cell is available at the UE during measurement period. - Valid information for the serving satellite has been provided

The first condition might be a non-reachable condition for the new IoT TDD frame. The reason is that only one or two frames are partially transmitted at every 90 ms. Therefore, the condition might never be fulfilled if the radio frames not transmitted by the network are considered as unavailable. The only scenario where this condition can be achieved depends on whether RAN1 adapts the radio frame counting such that only active radio frames are counted or considered valid ones.

Revise the condition where at least one 1 DL subframe per radio frame has to be available at the UE during the measurement period to: “at least 1 DL subframe per active DL radio frame of  serving NB-IoT cell is available at the UE during measurement”

The proposal above is not limited to connected mode measurements in the neighbor cells, but also to other measurement evaluations where the “1 DL subframe per radio frame” is not a reachable condition (clauses 7.23A, 8.14A, 9.1.22A, etc).

RRC Re-establishment and RLM

There are also open issues related to RLM and RRC Re-establishment, also addressing the more sparse conditions for measurements. They are listed as follows:

## Issue 3: RRC re-establishment (6.5A) Agreement (online): In the core requirement, For the delay requirement (TUL_grant + TUE_re-establish_delay_NB-IoT) of RRC re-establishment for TDD NB-IoT in NTN, - No update to TUL_grant - For the following parameters in TUE_re-establish_delay_NB-IoT - No update: - TPRACH_NB-IoT - TSI_NB1-NC [and TSI_NB1-EC] - Consider update, and the details are FFS: - Tsearch_NB1-NC,i [and Tsearch_NB1-EC,i] for unknown cell ## Issue 5: RLM (7.23A) Agreement (online): Further discuss the following aspects for RLM for TDD NB-IoT in NTN: - FFS update the hypothetical NPDCCH format (including Rmax) specified in Table 7.23A.2-1 upon RAN1 design in future meetings. - Evaluation period when DRX is not in use: - Option 1: Reuse the existing requirement defined in 7.23A.2.1, unless RAN1 changes cause impact to this requirement - Option 2: The evaluation period defined in 7.23A.2.1 is scaled - Option 2a: scaled by N=9 - Option 2b: scaled by FFS - Option 2c: no scaling, and define requirement based on 8ms DL subframes - Evaluation period when DRX is in use: - FFS whether the evaluation period defined in 7.23A.2.1 needs to be scaled.

Based on the discussions provided above for the connected mode measurements, we maintain as a principle that the time to complete the measurements do not require scaling, although this might compromise the UE power saving in certain situations. So, in consistency with the proposals and analysis made above:

Do not update Tsearch_NB1-NC,I  for RRC re-establishment

Dot not update RLM requirements in clause 7.23A.2.1

DL-UL Switch

Finally, there is one additional issue discussed in the previous WF that needs addressing. It is related to the DL-UL switch times in the RRC connection redirection.

## Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) Agreement (online): For the latency requirement (TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH) of RRC Connection redirection to Non-Anchor Carrier for TDD NB-IoT in NTN, - No update to TRRC_procedure_delay, TUL_grant, and TNPUSCH - Update the following parameters - Tperiod_DL_bitmap: to be updated based on RAN1 design - TDL-UL switch: to be updated based on RAN1 design

The requirements for RRC Connection Redirection to Non-Anchor Carrier establish the maximum delay in time for the UE to complete the transition (RRC Redirection) to the Non-Anchor Carrier upon receiving the reconfiguration indication through the IE “CarrierConfigDedicated-NB”.  This delay depends on several components, as described below:

6.9A.2 Requirements The UE shall be capable of performing the RRC connection redirection to the non-anchor carrier within Tconnection_redirect_non-anchor. The time delay (Tconnection_redirect_non-anchor) is the time between the end of the last subframe in the repetition period of NPDSCH containing the IE, “CarrierConfigDedicated-NB” received on the anchor carrier and the end of the last subframe in the repetition period of NPUSCH transmitted on the target non-anchor carrier. The time delay (Tconnection_redirect_non-anchor) shall be less than: Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - Tperiod_DL_bitmap: It is the periodicity of the downlink subframe configuration for downlink transmission on the non-anchor carrier. It is configured via IE DL-Bitmap-NB [2] and can be 10 ms or 40 ms. TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 8 ms. - TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier. When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended.

As we highlight above, two of the parameters need to be revisited during this WI, as they are impacted by the periodic pattern introduced.

The Tperiod_DL_bitmap defines the periodicity of the DL subframe configuration on the non-anchor carrier. It could be previously configured for 10 or 40 ms. However, in the current WI, due to periodic patterns with cycle periods of 9 radio frames, the “off time”  of a cell might be close to 90 ms. Therefore, the value of the parameter requires updating.

Update Tperiod_DL_bitmap to a value compatible with a period of 90 ms.

Also the TDL-UL switch  which measures the time between the end of the last subframe in the repetition period of the NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the NPUSCH transmitted on the non-anchor carrier requires updating. Currently the delay value is set to 8 ms. But in practice this value might be much longer than 8 ms, as there might be a gap between UL and DL active frames in the TDD periodic pattern.

In fact, regarding the DL-UL switch, RAN1 has reached a significant agreement that might be used as a baseline for establishing the new requirement:

Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [48…50] ms at the ULSRP. - NOTE: This corresponds to the largest separation between DL and UL Iridium slot pairs. Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA.

Update TDL-UL switch to a value equal to the guard period between DL-UL  subframes to be introduced in specifications by RAN1.

Conclusion

In this paper we analysed the open issues on the IoT TDD work item and have reached conclusions that support the following list of proposals and observations:

Proposal 1: Coverage Enhancements requirements are applicable for NB-IoT NTN TDD.

Proposal 2: Wait for further RAN1 definition to decide whether the requirements for coverage enhancements require updating.

Proposal 3: Wait for RAN1/2 progress related to the mismatch between (e)DRX cycle and the frame pattern.

Proposal 4: The UE shall be informed about the active periods of the neighbor cells configured to be measured in the new TDD frame pattern.

Proposal 5: Adopt Ksatellite equal to 1 for the cells measured in DL opportunities of a neighbor satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite.

Observation 1: The UE has enough opportunities to acquire NPSS and NSSS within a DRX Cycle, according to the simulation assumptions.

Proposal 6: Do not increase the Tmeasure and Tdetect for intra-frequency measurements when DRX is not configured.

Proposal 7: As a compromise, scale Tmeasure and Tdetect , when DRX is not configured, by a factor of [2], when the UE measures cells across 3 or more different patterns of DL opportunities.

Proposal 8: Scale Tmeasure and Tdetect, when DRX is configured, for intra frequency measurements by a factor of [2] when the UE measures cells across 3 or more different patterns of DL opportunities.

Proposal 9: Do not scale Tmeasure and Tedetect for inter-frequency measurements.

Proposal 10: For inter-frequency measurement, the minimum length of measurement occasions is [82] ms.

Proposal 11: Requirements are applicable if the cells is provided with assistance information on the DL periodic patterns of the cells to be measured.

Proposal 12: Revise the condition where at least one 1 DL subframe per radio frame has to be available at the UE during the measurement period to: “at least 1 DL subframe per active DL radio frame of  serving NB-IoT cell is available at the UE during measurement”

Observation 2: The proposal above is not limited to connected mode measurements in the neighbor cells, but also to other measurement evaluations where the “1 DL subframe per radio frame” is not a reachable condition (clauses 7.23A, 8.14A, 9.1.22A, etc).

Proposal 13: Do not update Tsearch_NB1-NC,I  for RRC re-establishment

Proposal 14: Dot not update RLM requirements in clause 7.23A.2.1

Proposal 15: Update Tperiod_DL_bitmap to a value compatible with a period of 90 ms.

Proposal 16: Update TDL-UL switch to a value equal to the guard period between DL-UL  subframes to be introduced in specifications by RAN1.

- References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024

- R4-2419795,  Way Forward for [113][317] IoT_NTN_TDD,   Moderator (Qualcomm Incorporated),  RAN4 #113, Orlando, US, 18th – 22nd November, 2024

- RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, RAN#106, December 2025

- R1-2409038, “Feature lead summary #1 on IoT-NTN TDD mode” , Qualcomm, RAN1#118bis

- R4-79AH-0138, “Cell detection simulation results”, Nokia, Hong Kong, China, 28rd – 30th June 2016