3GPP TSG-RAN4 Meeting  #114bis	R4-2504450

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.5.2

Source:	Apple

Title:	On methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

At the last RAN4 meeting, a WF was agreed that captures the agreements and open issues [2]. In this contribution, we share our views on the open issues:

- Signaling aspects

- UE Fallback behavior

# 2	Discussion

Signaling aspects

At RAN4#113, it was captured in the WF [2] that companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting. At the last meeting, there was no conclusion reached, as shown in the WF [3] below:

# Open issues ### How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers Option 1: Legacy band combination reporting scheme + Rx sharing capability indication - FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”. Other options are not precluded

As such, we provide our analysis below, which is updated based on the discussion at the last meeting.

Usually, the CA configurations supported by a UE are reported to the network when the UE camps on a cell and receives capability inquiry from the network. In the case of RF chain switching (between common RF chains and separate RF chains), however, the UE may need to update its supported CA configuration.

There are at least three alternatives as shown in the table below.

Table 1: Three alternatives of signaling solutions with pros and cons analysis

|  | How it works | Pros | Cons |
| --- | --- | --- | --- |
| Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication | UE reports to network that it is capable of Rx chain sharing for the supported band combination by additional signaling on top of existing band combination reporting. An example is the UE supports CA_n7(2A)-n25(2A) with Rx sharing capability and supports CA_n7A-n25(2A) without. Note: UE indication of Rx sharing can or cannot be supported based on some triggering condition is needed. | The current CA capability reporting is reused. | New UE capability is needed to indicate support of Rx chain sharing. |
| Alt. 2: Band combination capability update with the supported number of CCs | The UE first reports a high CA combination assuming non-continuous CCs can be handled by common RF chain. If some triggering conditions are met, such as when the blocker power is too high or self-interference due to TX leakage is too high, the UE can inform the network that only fewer CCs can be supported. Similarly, the UE first reports it supports CA_n7(2A)-n25(2A), and later if the triggering conditions are met, either the UE can use UAI or other mechanism to release CCs to CA configuration of CA_n7A-n25(2A) or the network can release CCs to CA configuration of CA_n7A-n25(2A) Note: UE indication of Rx sharing cannot be supported based on some triggering condition is needed. | The current CA capability reporting is reused. No new UE capability is introduced. |  |
| Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling | A new syntax/notation, say CA_n25(f_2A), is used for indicating support of fragmented carrier operation in for CA in band n25 with common RF chain. Depending on some triggering conditions, the UE may indicate it can or cannot support it. Note: UE indication of Rx sharing can or cannot be supported based on some triggering condition is needed. | It is visible and clear in RAN4 specification that a CA configuration is supported by UE with RF sharing capability. | A new syntax/notation is needed for both RAN4 and RAN2. |

We also note that there are more aspects related to UE capability and implementation options. Accordingly, more consideration is needed for the signaling design.

Proposal 1: In terms of signaling, at least three alternatives can be considered for band combination reporting for UE capable of Rx chain sharing, with the analysis of pros and cons in Table 1. More alternatives can be further considered.

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling

As detailed signaling is specified in RAN2, it is recommended to share RAN4’s pros and cons analysis with RAN2 for their views. This is also in line with the description in the SID [1]:

NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS

Proposal 2: It is recommended to send an LS to RAN2 to seek their views.

UE fallback behavior

UE fallback behaviors were discussed at the last meeting, e.g., in [4][5]. The issue is to consider UE fallback behavior in inter-band CA.

Before fallback, it is assumed that the UE works in in the inter-band CA case, where single RX chains A1 and A2 are used to support CC1 and CC2 in band A, and single RX chains B1 and B2 are used to support CC3 in Band B. When the UE cannot support CC1 and CC2 with single RX chain, as shown below, there are two fallback cases, Case 1 and Case 2:

- Case 1: Release one CC, either CC1 or CC2, in Band A

- Case 2: Release CC3 in Band B

Fallback Case 1: Release one CC, either CC1 or CC2, in Band A

Fallback Case 2: Release CC3 in Band B

It may be more straightforward to consider Case 1, since if a UE can use single RX chain to support two CCs in a band, it is expected to support just one CC.

On the other hand, there could be other considerations from the network/UE side. For example, if CC1 and CC2 channel bandwidth is larger than CC3 channel bandwidth, to support higher DL throughput, it is beneficial to fall back to CC1 + CC2 CA.

Therefore, which fallback case to support is proposed to be left to UE capability and network configuration.

Proposal 3: Which fallback case to support in inter-band CA is proposed to be left to UE capability and network configuration.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: In terms of signaling, at least three alternatives can be considered for band combination reporting for UE capable of Rx chain sharing, with the analysis of pros and cons in Table 1. More alternatives can be further considered.

- Alt. 1: Legacy band combination reporting scheme + Rx sharing capability indication

- Alt. 2: Band combination capability update with the supported number of CCs

- Alt. 3: New band combination definition e.g. CA_n25(f_2A) + new set of signaling

Proposal 2: It is recommended to send an LS to RAN2 to seek their views.

Proposal 3: Which fallback case to support in inter-band CA is proposed to be left to UE capability and network configuration.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2420337, "WF on fragmented DL carriers study," MediaTek

R4-2502873, "WF on fragmented DL carriers study," Moderator (Mediatek Inc.), OPPO, vivo, ZTE, Nokia, Ericsson, Xiaomi, Apple, Samsung, Huawei, Murata, CHTTL

R4-2501802, "On signalling aspect of fragmented carriers," Huawei, HiSilicon

R4-2501150, "Discussion on remaining signalling aspects for DL fragmented carriers study," MediaTek