3GPP TSG-RAN4 Meeting  #114bis	R4-2504452

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.5.1

Source:	Apple

Title:	TP on UE RF requirements impact for fragmented carriers

Document for:	Discussion

# 1	Introduction

This contribution provides text proposals to the TR [1] to capture two aspects:

- Clarification that adjusted requirements for UEs supporting fragmented carriers with common RF chain architecture, if any, shall be separated from the existing requirements.

- The analysis of UE RF requirement impact for fragmented carriers from our contribution in [2], where both UE self-interference and blocker impact are analyzed.

# 2	Text Proposal

<<Start of Change 1 for TR 38.755>>

# 7	Study on RF requirements for the inter-operator co-located BS scenario

## 7.0	Requirement clarification

It is important to establish a common understanding on the applicability of the new/additional requirements for UE supporting fragmented carriers with shared RF chains. As is well understood, the current RF requirements for intra-band non-contiguous CA were derived based on a partially shared RF chain architecture captured in TR 36.823. Given the blocker in the gap, it is evident that a shared RF chain architecture cannot meet such requirements. That is the reason why the SI scope explicitly contains the bullet “Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking.”

To be clear, whatever the outcome of the SI is, the existing requirements should still apply to UEs using partially shared RF chain architecture to support fragmented carriers (or intra-band non-contiguous CA). With that, it is reasonable to accept that the adjusted requirements for UE supporting fragmented carriers with shared RF chains should be separated from existing requirement. Of course, there should be a UE capability or other means to indicate such UEs.

## 7.X	RF requirements evaluation from company B

With common RF chain architecture, the TX noise at SCC from the PCC remains the same as the analog BB filter cannot provide any help. However, besides TX noise, there is TX leakage at TX band that may benefit from analog filtering before it goes into digital BB. In this case, since common RF chain architecture uses a wide analog BB filter to cover the overall frequency span from the lower edge of CC1 to the upper edge of CC2, as compared to the separate RF chain architecture where two separate narrow BB filters to cover CC1 and CC2, there is less filter rejection. As a result, both the PCC and SCC performance may take some hit.

We first provided analysis on the impact of self-interference, i.e., the NF increase due to AGC adjustment (i.e., RX gain backoff). Table 1 and Table 2 show our analysis for CA configuration CA_n3(2A) (5MHz PCC+ 5MHz SCC, Wgap = 65MHz) and CA_n25(2A) (5MHz PCC + 5MHz SCC, Wgap = 55MHz), respectively.

Table 1. Link budget of Self-interference for CA_n3(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - |
| LPF attenuation | 13.4 | 1 | 1 | - | - |
| LPF output (dBm) | -36.4 | -102 | -97.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -101 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.5 | - |
| Noise floor (including ADC noise) | - | - | - | -90.5 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -98.1 | - | - |
| SNR after ADC (dB) | - | - | - | - | -11.5(PCC) |
|  |  |  |  |  | -7.5(SCC) |

Table 2. Link budget of Self-interference for CA_n25(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -96.5 | -91.5 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -100.5 | -95.5 | - | - |
| LPF attenuation | 12.9 | 1 | 1 | - | - |
| LPF output (dBm) | -35.9 | -101.5 | -96.5 | - | - |
| Noise floor for basic REFSENS | - | - | - | -100.5 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 11 | - |
| Noise floor (including ADC noise) | - | - | - | -89.5 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -98.1 | - | - |
| SNR after ADC (dB) | - | - | - | - | -12(PCC) |
|  |  |  |  |  | -7.7(SCC) |

Next, we consider the impact of blocker in the gap in addition to the self-interference. In the analysis, we consider two cases, where the image of the blocker overlaps and does not overlap with a wanted CC.

Case 1: The image of the blocker does not overlap with a wanted CC

Based on our analysis, the results are shown in Table 3 and Table 4 for CA_n3(2A) and CA_n25(2A), respectively.

Note the following in the analysis:

- In ACS Case 1, both the receiver sensitivity (REFSENS) and blocker power are increased by the proposed ΔRIBNC adjustment due to self-interference.

- In IBB Case 1 and Case 2, only the receiver sensitivity (REFSENS) is increased by the proposed ΔRIBNC adjustment due to self-interference, but the blocker power is not. This is because in ACS Case 2, the wanted signal power is fixed at -56.5dBm and the blocker power is fixed at -25dBm in ACS Case 2, -56dBm in IBB Case 1, and -44dBm in IBB Case 2, respectively.

Table 3. Link budget of Blocker for CA_n3(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 13.4 |  |
| NF increase due to AGC adjustment (dB) | 10.5 |  |
| Proposed ΔRIBNC adjustment due to self-interference | 10.5dB for PCC | 6.5dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | 13 | 13 |
| ACS Case 2 | 4.1 | 4.1 |
| IBB Case 1 | 5 | 5 |
| IBB Case 2 | 5 | 5 |

Table 4. Link budget of Blocker for CA_n25(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 12.9 |  |
| NF increase due to AGC adjustment (dB) | 11 |  |
| Proposed ΔRIBNC adjustment due to self-interference | 11dB for PCC | 6.7dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | 13 | 13 |
| ACS Case 2 | 4.1 | 4.1 |
| IBB Case 1 | 5 | 5 |
| IBB Case 2 | 5 | 5 |

Based on the analysis, with the proposed ΔRIBNC adjustment due to self-interference, the ACS/IBB requirements can be met with some margin. However, we note that this conclusion may depend on the CA band combination. For instance, if the duplex gap between UL and DL bands are wide enough, in other words, the TX leakage is of little issue and little ΔRIBNC adjustment is needed to meet the REFSENS requirement, there may be some adjustment needed for ACS/IBB requirements.

Case 2: The image of the blocker overlaps with a wanted CC

When the image of the block overlaps with a wanted CC, the results are shown in Table 5 and Table 6 for CA_n3(2A) and CA_n25(2A), respectively.

Note the following for the analysis:

- Image rejection ratio at the UE is assumed to be 25dB.

- Other assumptions are the same as in Case 1 where the image of the blocker does not overlap with a wanted CC.

- For the two CA configurations, CA_n3(2A) (5MHz PCC + 5MHz SCC, Wgap = 65MHz) and CA_n25(2A) (5MHz PCC + 5MHz SCC, Wgap = 55MHz), because the PCC and SCC have the same channel bandwidth, the image of the blocker does not overlap with a wanted CC. Therefore, the analysis shown below are aimed to provide an initial assessment of the impact. Further analysis is needed to address the CA configurations where the image of the blocker does overlap with a wanted CC.

Table 5. Link budget of Blocker for CA_n3(2A)

| Image rejection ratio (IRR) (dB) | 25 |  |
| --- | --- | --- |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -6.5 | -6.5 |
| ACS Case 2 | -6.5 | -6.5 |
| IBB Case 1 | 0.5 | 1.2 |
| IBB Case 2 | -11.5 | -10.8 |

Table 6. Link budget of Blocker for CA_n25(2A)

| Image rejection ratio (IRR) (dB) | 25 |  |
| --- | --- | --- |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -6.5 | -6.5 |
| ACS Case 2 | -6.5 | -6.5 |
| IBB Case 1 | 1.5 | 2.2 |
| IBB Case 2 | -10.5 | -9.8 |

It can be seen that the SNR for ACS Case 1, ACS Case 2 and IBB Case 2 is 5-10dB below -1dB, showing the image of the blocker causes strong interference to the receiver.

Another way to address the image interference issue is to limit the blocker to wanted CC power ratio of 26dB. If this is the case, SNRs for ACS/IBB Case 1 and 2 in Table 5 and Table 6 will be -1dB or higher.

<<End of Change 1 for TR 38.755>>

# 4	References

TR 38.755 V0.0.1

R4-2504451, "On UE RF requirements for fragmented carriers," Apple