3GPP TSG RAN WG4 Meeting #114-bis	R4-2504461

Wuhan, China, 07 April 2025 – 11 April 2025

Agenda Item:	7.12.4

Source: 	Ericsson

Title:	Discussion on Remaining Issues Related to Rel-19 ATG CA in RRM Core

Document for:	Discussion

# Introduction

In the last meetings, we discussed and agreed on several general issues regarding the enhancement of the ATG CA. The approved WF of RAN4#114 can be found in [1]. This contribution discusses the remaining open issues related to ATG CA enhancement.

# Discussion

In this section, we discuss the important open issues for this meeting.

## General Issues

## UE antenna type for inter-band CA

During the RF session of RAN4#114, the following was agreed regarding case 4-

Agreement: - ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam is possible for specific band combination. - Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation. - Option 1: Case 4 UE implementation is possible for CA_n3-n39 that frequency ranges between two bands are close to each other. - Option 2: Case 4 UE implementation is possible for CA_n3-n39 that DL frequency separation between two bands are less than [X=FFS, e.g, 500 MHz].

It is important to note that, according to RF session, one common Rx beam is possible for specific band combination. The type of antenna array used for inter-band CA highly depends on the frequency range of the operating bands and the implementation of the UE. For Case 4, UE implementation is possible for CA involving bands n3 and n39, as the frequency ranges between these two bands are relatively close to each other. It is still under FFS what the minimum frequency separation needs to be between two bands for Case 4 to be applicable.

RF- Session agreed that one common Rx beam is possible for specific band combination where Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation.

The RF session in Release 19 will not specify a particular band combination for Case 4. Therefore, RAN4 should carefully capture Case 4 in the specifications. According to the directives in the Foreword of 38.133, the modal verb "can" should be used to express the requirements related to Case 4 specification as can in the spec. indicates that it is possible.

According to the 38.133 Foreword directives, modal verb ‘can’ should be used to capture case 4 related requirements.

Furthermore, we may need to carefully capture the common beam management (CBM) related requirements, as the requirements set for case 4 may not be universally applicable and may require introducing specific conditioning in many cases.

RAN4 should be cautious when capturing case 4 or CBM-related requirements, as the requirements set for case 4 are not applicable to all cases and often require specific conditioning.

## UE Antenna Type Signaling

In RAN4#114, it is agreed to introduce a new UE capability to indicate common and independent Rx beams. As both the PCC and SCC use antenna arrays, with the current UE capability and/or IE, it is not possible to differentiate whether the UE is using a common Rx Beam or an independent Rx beam. Hence, in the last meeting, we agreed to introduce additional UE capability to differentiate common and independent Rx beam.

Issue 1-1-2: UE Antenna Type signaling Agreement: - To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed - FFS for the details.

The UE can inform the network via a 1-bit indicator (as depicted in Table 1) whether it uses separate Rx beams or a single Rx beam for PCell and SCell measurements in an NR ATG CA. This distinction is particularly relevant for co-located PCell and SCell scenarios: if the UE uses a single Rx beam (i.e., case 4), PCell measurements can reliably represent SCell conditions, enabling the network to reuse measurements and reduce overhead. However, with separate Rx beams (i.e., case 5), measurement discrepancies may occur even for co-located cells, requiring independent measurement handling. Therefore, the indicator is required to ensure accurate network interpretation and optimized RRM decisions for ATG Dl CA.

| Value | Antenna type |
| --- | --- |
| 0 | Antenna arrays |
| 1 | Antenna arrays with only one antenna panel |

Table 1 Example of UE indicating antenna type

At least One bit indicator needs to be introduced for additional UE capability to indicate common or separate Rx beam.

This requires discussion in RAN2. We believe an LS should send out to RAN2. A draft of the LS can be found in Annex A.

RAN4 should send an LS to RAN2 based on LS draft in Annex A.

## UE Capability on Simultaneous Reception with One Common Rx Beam

In the specific scenario of ATG co-located DL CA, as described in Case 4, the UE possesses two separate Rx chains: one dedicated to the PCC and another for the SCC. However, ATG UE can form one common Rx beam at any given time. Given this single-beam limitation, the UE inherently faces significant constraints when receiving signals simultaneously across different frequencies or numerologies in inter-band.

With an antenna array on the bands of PCC and SCC, and one common Rx beam, ATG UE inherently faces significant constraints when receiving signals simultaneously across different frequencies or numerologies in inter-band.

In this case, ATG UE with one Rx beam requires the capability to concurrently perform inter-frequency or different-numerology measurements (e.g., synchronization signal blocks or SSB-based measurements) while receiving data signals (PDSCH/PDCCH). Without this capability, the UE would be unable to efficiently utilize its limited beam resources, as it would need to sequentially switch its single Rx beam between reception and measurement tasks, thereby reducing overall efficiency and reliability. Thus, supporting simultaneous Rx Data reception is crucial for optimizing resource utilization, improving beam management accuracy, and enhancing overall system performance for UEs constrained to a single common Rx beam in co-located ATG DL CA scenarios. RAN4 should discuss and support new UE capability to simultaneously perform the following tasks without requiring measurement gaps:

- Receive scheduled DL data/control signals (PDSCH/PDCCH) on a serving cell.

- Concurrently perform measurements on Synchronization Signal Blocks (SSBs) transmitted on inter-frequency or intra-frequency cells that have a different numerology (subcarrier spacing) from the scheduled serving cell.

RAN4 should discuss and support new UE capability to simultaneously perform the following tasks without requiring measurement gaps:

- Receive scheduled DL data/control signals (PDSCH/PDCCH) on a serving cell.

- Concurrently perform measurements on Synchronization Signal Blocks (SSBs) transmitted on inter-frequency or intra-frequency cells that have a different numerology (subcarrier spacing) from the scheduled serving cell.

## Signaling Characteristics

## SCell Addition/Activation Delay

Upon reviewing the current SCell activation delay requirements, we found that SCell activation from CBM and IBM is not specified for the FR1 UE antenna array. Therefore, we propose to discuss and capture the following points.

#### Conditions for CBM vs IBM

RF Session agreed that an ATG UE with an antenna array on the PCC and SCC bands using a common Rx beam is possible for specific band combinations. In TS 38.133, CBM is not specified for SCell addition/activation delay in FR1, leaving a gap in how such configurations should be handled. Given that a common Rx beam allows the UE to reuse PCell measurements for the co-located SCell, applying CBM can reduce measurement overhead and activation latency. However, in scenarios where the UE supports independent Rx beams, beam-specific measurements are required to ensure accuracy. Therefore, RAN4 should discuss and clearly capture the conditions under which CBM or independent beam management (IBM) should be applied.

CBM and/or IBM conditions are not specified in the legacy spec. Without specifying the CBM and/or IBM condition for FR1 array, SCell activation for UE antenna array type is not possible.

RAN4 should discuss and capture the conditions when CBM and IBM should be applied for SCell activation.

According to the WID and RF agreement, a representative inter-band CA example in FR1 is CA_n3-n39, where band n3 spans 1805–1880 MHz and band n39 spans 1880–1920 MHz. With a minimum channel bandwidth of 5 MHz, the frequency separation between these bands remains within 100 MHz. In principle, CBM assumes that beam characteristics, such as directionality and gain patterns, remain sufficiently similar across PCC and SCC to allow the reuse of measurements (e.g., SS-RSRP, CSI-RSRP) from one carrier to the other. However, beam behaviour is frequency-dependent, and even within FR1, a 100 MHz separation, especially in mid-band frequencies, can cause divergence in beam alignment due to differences in antenna beamwidth and steering. While 100 MHz may still be acceptable for some UEs equipped with carefully designed wideband antenna arrays, for robust and standards-aligned CBM operation, a reduced separation (e.g., ≤50 MHz) may be safer to ensure that a common Rx beam delivers consistent gain and directionality across both carriers. Therefore, it is proposed that CBM may be supported if the frequency separation between co-located inter-band DL CA bands is within a threshold, tentatively 50 to 100 MHz, beyond which the UE should default to IBM. RAN4 should evaluate and define a clear separation threshold, documenting when CBM or IBM applies, considering UE antenna architecture, carrier frequency proximity, and the need for accurate and reliable measurement behaviour.

RAN4 should evaluate and define a clear separation threshold, documenting when CBM or IBM applies, considering UE antenna architecture, carrier frequency proximity, and the need for accurate and reliable measurement behaviour.

RAN4 should discuss and agree following conditions to differentiate between CBM and IBM.

- -If, the frequency ranges of intra-band co-located DL CA bands are less than or equivalent to X MHz, then the ATG UE may support CBM with an antenna array on the PCC and SCC in FR1.

- -Otherwise, ATG UE with an antenna array on the bands of PCC and SCC will adhere to IBM in FR1.

RAN4 should discuss and agree on the DL frequency separation (i.e., X MHz) between the two bands for FR1 CBM should be

- Option 1: ≤50 MHz.

- Option 2: ≤100 MHz.

- Option 3: Between 50-100MHz, FFS.

#### SCell Addition/Activation Delay for Common Rx Beam

To support SCell Activation Delay for Common Rx Beam, the following scenarios need to be captured in the specification.

- Scenario 1: Known SCell with CBM

- Scenario 2: PCell information can be reused for unknown SCell with CBM

In Scenario 1, it is assumed that the ATG UE is equipped with an antenna array that can support both bands, where the frequency ranges of both bands are less than or equal to X MHz (i.e., if Proposal 4 is agreed upon, then X would be less than 100 MHz). The Scell is already known to the UE but is initially deactivated. Since the SCell and PCell have co-located beams and operate in close frequency bands, the same beam refinement process can be used. UE does not require additional beam acquisition for SCell activation, and common Rx beam steering reduces activation delay by minimizing the need for additional CSI-RS measurements. In such a scenario, ATG UE can skip Cell search and L1-RSRP measurement. Hence, the Tactivation_time can be set to 3ms.

For Known SCell with CBM ATG UE can skip Cell search, and L1- RSRP measurement.

RAN4 should discuss and agree that If the target SCell is known by the UE and frequency ranges of both bands are less than or equivalent to X MHz with common beam management the Tactivation_time time can be 3ms .

In Scenario 2, it is assumed that the ATG UE is equipped with an antenna array that can support both bands where the frequency ranges of both bands are less than or equal to X MHz. The SCell is unknown and not pre-configured to the UE, but the UE can reuse the PCC beam information. Since the SCell is not pre-configured, the UE must perform an SSB search to detect it. The beam management delay is zero because the UE uses the PCell beam information to determine the beam for the SCell. Hence, if the target SCell is unknown, frequency ranges of both bands are less than or equal to X MHz then Tactivation_time is 3ms+TFirstSSB+ 2ms.

For unknown SCells, CBM beam information can be reused, but the UE still needs to perform the SSB search in the SCell.

RAN4 should discuss and agree that if the target SCell is unknown, frequency ranges of both bands are less than or 	equal to X MHz Tactivation_time is 3ms+TFirstSSB+ 2ms.

## SCell Addition/Activation Mechanism

In the last meeting, we discussed the possibility of introducing distance-triggered and/or condition-based SCell addition and activation mechanism proposals and decided to discuss further in the RAN4#114-bis meeting.

Issue 1-3-3: SCell Addition/Activation mechanism FFS - Proposal 1: Consider distance triggered SCell addition - Proposal 1-1: To save power and avoid throughput loss caused by measurement for SCell addition and/or activation, introduce distance triggered SCell addition and/or activation, which can be achieved through network configuration of SCell addition and/or activation condition. After the condition is fulfilled at UE side, UE can add and/or active the SCell automatically, and then sent the SCell addition/activation confirmation message to network. - Proposal 1-2: The distance triggered SCell addition and/or activation needs to be discussed with the UE antenna type. - Proposal 2: RAN4 should wait for the decision of RF session on case 4 before proceed with the Scell addition/activation mechanism discussion. - Proposal 3: further optimization methods for SCell addition and activation can be considered - Define the corresponding enhanced requirements with some conditions, where the measurements of SCell addition and scheduling restrictions on neighbor cell measurements on candidate SCC can be omitted, and it depends on UE to apply these requirements.

In the distance triggered SCell addition and/or condition based SCell addition mechanism, UE initiates the SCell activation and addition. However, to our understanding SCell activation should be fully controlled by the network since network knows whether and when to trigger the DL SCell based on the data buffer traffic and cell quality. That is why historically TS 38.133 outlines only network assisted SCell activation and addition. Therefore, we do not see a strong justification for introducing an additional mechanism, specifically with UE assisted mechanism.

Distance triggered SCell addition and/or Condition based SCell addition mechanism is UE initiated SCell activation and addition mechanism.

DL SCell activation should be controlled by the network since network knows whether and when to trigger the SCell based on the data buffer, traffic and cell quality information.

Furthermore, both proposals, as we understand them, are beneficial for ATG UE power saving. ATG UE is not power limited. Thus, RAN4 should not introduce any UE- initiated SCell activation mechanism.

RAN4 should not support any UE initiated DL SCell activation mechanism.

## SCell Deactivation Mechanism

Even though we think the distance triggered SCell addition and/or condition based SCell addition mechanism is not needed for SCell Activation for ATG CA, but we think this method is beneficial for ATG deactivation. In the legacy procedure, the UE monitors SCell scheduling using a timer. The timer can be predefined or configured by NW. If UE doesn’t receive the DL/UL scheduling and the timer expires, UE will deactivate the SCell accordingly. Additionally, if we introduce distance-based deactivation, the UE monitors SCell scheduling based on the distance condition. The distance condition can be configured by the network. if UE doesn’t receive the DL/UL scheduling and the distance condition is met, UE will deactivate the SCell accordingly.

Even though we consider that distance-triggered or condition-based SCell addition mechanisms are not necessary for SCell activation in ATG CA scenarios, we believe that such mechanisms can be beneficial for SCell deactivation. In the legacy procedure, the UE monitors SCell activity based on a timer either predefined or configured by the network. If the UE does not receive DL or UL scheduling for the SCell before the timer expires, it proceeds to deactivate the SCell.

However, relying solely on a timer can be suboptimal in ATG environments where the UE experiences high mobility, such as in aircraft scenarios. In such cases, the UE may move rapidly out of the effective coverage area of the SCell, yet the deactivation timer may not reflect this change promptly. This could result in the UE maintaining an inactive or unreachable SCell for an extended period, leading to inefficient resource utilization and potential signaling delays.

To address this, we propose to introduce a distance-based deactivation mechanism. In this approach, the UE would continue to monitor the DL/UL scheduling for the SCell, while also evaluating in parallel whether a distance condition configured by the network is met. If the UE fails to receive scheduling information and the distance threshold is crossed (indicating the UE has likely moved beyond the intended SCell coverage area), the UE can proactively deactivate the SCell.

This mechanism enhances responsiveness and ensures that SCell deactivation more closely aligns with the UE’s actual radio conditions and spatial movement. It also reduces unnecessary SCell tracking and measurement operations, resulting in improved power efficiency and more scalable behavior in high-speed ATG scenarios. Hence, RAN4 should introduce a distance-based DL SCell deactivation mechanism.

- RAN4 should introduce a distance-based DL SCell deactivation mechanism.

## Interruption Requirements

The following open issues related to interruption requirements were discussed in the RAN4#114 meeting and should be addressed in this meeting.

Issue 1-3-1-2: Whether Interruptions at NR SRS carrier based switching requirements need to be introduced FFS - Proposal 1: The NR SRS carrier based switching interruption requirement can be introduced. (HW)

Carrier-based switching for SRS, as specified by 3GPP, explicitly addresses scenarios where the UE temporarily switches its uplink from one configured carrier to another activated carrier to send SRS. In the Rel-19 ATG CA scenario, the UE cannot directly send SRS on the SCell because no uplink resources are allocated for it. Therefore, if the network requires the UE to transmit SRS signals to evaluate the uplink condition relevant to the SCell (e.g., uplink beam or channel quality estimation for a related frequency), the network explicitly configures SRS transmissions on another suitable carrier that does support UL (e.g., PCell or other activated UL carriers). Furthermore, the network can statically configure the SRS transmission on a fixed UL carrier, such as the PCell, without requiring any dynamic switching. Additionally, carrier based switching for SRS is a separate feature and outside of the scope of the WID for RRM. Hence, we believe there is no specific need to introduce NR SRS carrier based switching interruption requirement for NR ATG CA.

RAN4 should not introduce NR SRS carrier based switching interruption requirement for NR ATG CA.

## Measurement Procedure

    - Deactiated SCell measurement

In the RAN4#114 meeting, the following agreement was made regarding deactivated SCell measurement.

Issue 1-4-4: Deactivated SCell measurement Agreement: - For ATG UE antenna types except Case 4, L1/L3 measurement sharing factor is not needed - FFS: For ATG UE antenna type Case 4, L1/L3 measurement sharing factor is needed

In the case of UE antenna type Case 4, the UE only have one Rx beam, which prevents it from conducting simultaneous L3 measurements of neighboring cells and L1 measurements of all serving cells. As a result, the measurement resources must be shared among all L1 measurements for the serving cells and the L3 measurements. Therefore, for ATG UE antenna type Case 4, a sharing factor for L1 and L3 measurements is necessary.

RAN4 to discuss and agree that for ATG UE antenna type Case 4, a sharing factor for L1/L3 measurements is needed.

    - Scheduling Availability/Restrictions

The following agreements were made regarding the scheduling availability and restrictions of ATG UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 in RAN4#114 meeting. It is also agreed to further study Scheduling Availability/Restrictions for case 4.

Issue 1-4-7: Scheduling Availability/Restrictions of ATG UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 Agreement: - For ATG UE antenna types except Case 4 - For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for RLM, BFD, CBD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols. - When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the bands in which PCell is configured. - FFS for ATG Case#4 UE

UE measurements such as RLM, BFD, CBD, intra-frequency/inter-frequency measurements, L1-RSRP, and L1-SINR may be significantly impacted by the absence of measurement gaps. Measurement gaps are typically required when these measurements involve signals using different frequencies or subcarrier spacings (numerologies) compared to the scheduled PDSCH/PDCCH reception signals.

The capability simultaneousRxDataSSB-DiffNumerology-Inter-r16 specifies whether a UE can concurrently perform inter-frequency SSB-based measurements without requiring measurement gaps while actively receiving data and control channels (PDSCH/PDCCH) from its serving cell. If a UE does not support simultaneous measurement capability, scheduling restrictions become necessary to ensure accurate measurements and avoid potential interference between measurement activities and data reception.

Specifically, in scenario 4, the UE has two separate Rx chains, one for the PCC and one for the SCC, but shares only a single Rx beam. Given the limitations of ATG UE capabilities, which do not permit simultaneous measurement and data reception across different numerologies or frequencies, performing neighbor cell measurements in one band will consequently require scheduling restrictions on the serving cell in other bands. These scheduling restrictions are necessary to maintain measurement accuracy, ensure link reliability, and prevent degradation in measurement quality due to simultaneous reception limitations.

RAN4 should discuss and agree on the following Scheduling restriction options that arises due to UE limitations in simultaneously performing measurements and data reception across different numerologies without measurement gaps:

- Option 1: For scheduling availability of UE performing measurements (RLM, BFD, CBD, intra/inter-frequency measurements without MG, L1-RSRP, and L1-SINR measurements) with a different subcarrier spacing from the PDSCH/PDCCH, and ATG UE does not support simultaneous reception. When inter-band carrier aggregation in FR1 is configured,

- For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, scheduling restrictions arising from measurement activities on a given PCell should apply to the SCell in a different band on the symbols fully or partially overlap with the aforementioned restricted measurement symbols.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for ATG UE DL CA and made the following observations and proposals.

Observation 1	RF- Session agreed that one common Rx beam is possible for specific band combination where Antenna array type for inter-band CA is highly depending on frequency range of operating bands and UE implementation.

Observation 2	According to the 38.133 Foreword directives, modal verb ‘can’ should be used to capture case 4 related requirements.

Observation 3	RAN4 should be cautious when capturing case 4 or CBM-related requirements, as the requirements set for case 4 are not applicable to all cases and often require specific conditioning.

Observation 4	At least One bit indicator needs to be introduced for additional UE capability to indicate common or separate Rx beam.

Observation 5	With an antenna array on the bands of PCC and SCC, and one common Rx beam, ATG UE inherently faces significant constraints when receiving signals simultaneously across different frequencies or numerologies in inter-band.

Observation 6	CBM and/or IBM conditions are not specified in the legacy spec. Without specifying the CBM and/or IBM condition for FR1 array, SCell activation for UE antenna array type is not possible.

Observation 7	For Known SCell with CBM ATG UE can skip Cell search, and L1- RSRP measurement.

Observation 8	For unknown SCells, CBM beam information can be reused, but the UE still needs to perform the SSB search in the SCell.

Observation 9	Distance triggered SCell addition and/or Condition based SCell addition mechanism is UE initiated SCell activation and addition mechanism.

Observation 10	DL SCell activation should be controlled by the network since network knows whether and when to trigger the SCell based on the data buffer, traffic and cell quality information.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 should send an LS to RAN2 based on LS draft in Annex A.

Proposal 2	RAN4 should discuss and support new UE capability to simultaneously perform the following tasks without requiring measurement gaps:

 Receive scheduled DL data/control signals (PDSCH/PDCCH) on a serving cell.

 Concurrently perform measurements on Synchronization Signal Blocks (SSBs) transmitted on inter-frequency or intra-frequency cells that have a different numerology (subcarrier spacing) from the scheduled serving cell.

Proposal 3	RAN4 should discuss and capture the conditions when CBM and IBM should be applied for SCell activation.

Proposal 4	RAN4 should evaluate and define a clear separation threshold, documenting when CBM or IBM applies, considering UE antenna architecture, carrier frequency proximity, and the need for accurate and reliable measurement behaviour.

Proposal 5	RAN4 should discuss and agree following conditions to differentiate between CBM and IBM.

-If, the frequency ranges of intra-band co-located DL CA bands are less than or equivalent to X MHz, then the ATG UE may support CBM with an antenna array on the PCC and SCC in FR1.

-Otherwise, ATG UE with an antenna array on the bands of PCC and SCC will adhere to IBM in FR1.

Proposal 6	RAN4 should discuss and agree on the DL frequency separation (i.e., X MHz) between the two bands for FR1 CBM should be

Option 1: ≤50 MHz.

Option 2: ≤100 MHz.

Option 3: Between 50-100MHz, FFS.

Proposal 7	RAN4 should discuss and agree that If the target SCell is known by the UE and frequency ranges of both bands are less than or equivalent to X MHz with common beam management the Tactivation_time time can be 3ms .

Proposal 8	RAN4 should discuss and agree that if the target SCell is unknown, frequency ranges of both bands are less than or  equal to X MHz Tactivation_time is 3ms+TFirstSSB+ 2ms.

Proposal 9	RAN4 should not support any UE initiated DL SCell activation mechanism.

Proposal 1	RAN4 should introduce a distance-based DL SCell deactivation mechanism.

Proposal 2	RAN4 should not introduce NR SRS carrier based switching interruption requirement for NR ATG CA.

Proposal 3	RAN4 to discuss and agree that for ATG UE antenna type Case 4, a sharing factor for L1/L3 measurements is needed.

Proposal 4	RAN4 should discuss and agree on the following Scheduling restriction options that arises due to UE limitations in simultaneously performing measurements and data reception across different numerologies without measurement gaps:

 Option 1: For scheduling availability of UE performing measurements (RLM, BFD, CBD, intra/inter-frequency measurements without MG, L1-RSRP, and L1-SINR measurements) with a different subcarrier spacing from the PDSCH/PDCCH, and ATG UE does not support simultaneous reception. When inter-band carrier aggregation in FR1 is configured,

For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, scheduling restrictions arising from measurement activities on a given PCell should apply to the SCell in a different band on the symbols fully or partially overlap with the aforementioned restricted measurement symbols.

# References

- R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420338, WF on NR ATG UE RF, CMCC

- R4-2415002, (NR_ATG_enh-Core) Discussion on RRM core requirements for R19 ATG, CMCC

- R4-2417716, Discussion on enhancements for Rel-19 ATG RRM requirements, CATT

# Annex A: LS to RAN2: UE Antenna type signalling

Title:	LS on UE capability for UE Antenna Type Signalling in ATG

Response to:

Release:	Release 19

Work Item:	NR_ATG_enh-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Istiak Hossin

E-mail Address:	istiak.hossain@ericsson.com

Attachments: -

# 1	Overall description

RAN4 has discussed and agreed that 5 antenna type combinations can be used for ATG DL CA. The agreed antenna types are the following:

1. ATG UE with omnidirectional antennas on the bands of PCC and SCC.

2. ATG UE with an omnidirectional antenna on the band of PCC and an 	antenna array on the band of SCC.

3. ATG UE with an antenna array on the band of PCC and an omnidirectional 	antenna on the band of SCC.

4. ATG UE with an antenna array on the bands of PCC and SCC, and one 	common Rx beam assumed.

5. ATG UE with an antenna array on the bands of PCC and SCC, and two 	simultaneous separate Rx beams assumed.

To differentiate between different UE array antenna types (i.e. case 4 and case 5) for Inter-band co-located CA, an additional UE capability is needed to indicate whether the Rx beam is common or independent. It is agreed that the UE can inform the network about the antenna type used for measurements in an NR ATG cell group via a 1-bit indicator or explicit signaling in UE capability information IE. The indicator specifies whether the common or independent Rx beam is used for PCell and SCell measurements, as depicted in Table 1.

| Value | Antenna type |
| --- | --- |
| 0 | Antenna arrays |
| 1 | Antenna arrays with only one antenna panel |

Table  Example of UE indicating antenna type

# 2 Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to take above information into account and capture the UE antenna type signaling in the required specification/s.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #115	            19 – 23 May 2025,	Malta

TSG RAN WG4 Meeting #116	            25 – 29 August 2025, India