3GPP TSG-RAN WG4 Meeting #114bis	R4-2504465

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.30.2

Source:	MediaTek Inc.

Title:	Discussion on RF requirement impact for IoT NTN phase 3

Document for:	Approval

- Introduction

In RAN#104 plenary meeting, the WID of “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was approved in [1]. One of the main objectives is to enable multiplexing of multiple UEs via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH and to update RF requirements accordingly if needed.

In this contribution, our views on the RF requirements impact are provided for discussion.

- Discussion

2.1		RF requirement impact for IoT-NTN phase 3

Some of the main Rel-19 WID [1] objectives for IoT NTN are shown below for reference.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account

In RAN4#114 meeting, the agreed WF [2] provides the following works for considering RF requirement impact. The contents are shown as follows.

RF core requirements NOTE1: Enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH. Issue 2-1: UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2 - Agreement: - No RF impact on OCC2 for IoT NTN. Issue 2-2: Whether there would be UE RF requirement impact when OCC schemes would cross gap in UL transmission. - Agreement: - No RF impact for <1ms gap length. - FFS for longer gaps. Issue 2-3: Whether UE would need to re-synchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs. - Agreement: - Further discuss whether solutions (e.g., CFO grouping) for solving frequency inaccuracy between OCC UEs would have potential RF impact.

To maintain successful UE UL OCC scheme, some conditions could be needed within the operation time window.

Proposal 1: Regarding OCC schemes with UE CFO grouping, consider whether the following conditions should be met within the UE NPUSCH OCC operation time window:

-	UL LO frequency does not changed.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-	Doppler frequency and time pre-compensation event trigger is not within OCC operation window.

3 Conclusion

In this paper, our views on RF requirements impact for NPUSCH with OCC feature are provided as follows:

Proposal 1: Regarding OCC schemes with UE CFO grouping, consider whether the following conditions should be met within the UE NPUSCH OCC operation time window:

-	UL LO frequency does not changed.

-	Modulation order does not change.

-	No network commanded TA takes effect.

-	There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-	Doppler frequency and time pre-compensation event trigger is not within OCC operation window.

4 Reference

- RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc.

- R4-2502373, “Way Forward for [114][315] IoT_NTN_Ph3”, MediaTek Inc.