3GPP TSG-RAN WG4 Meeting #114bis					R4-2504467

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Discussion on LP-WUS RRM Simulations

Agenda item:	7.26.4.1

Document for:	Discussion

Introduction

This contribution provides simulation results for LP-SS and proposes a simulation methodology

Full set of simulation results is presented in our contribution R4-2504466

Discussion

Summary of the simulation results

We simulated initially TDLC with ED and with the sequences selected in the last meeting. What we noticed is that from the results it was not clear which part of the results was because of number of samples, and which part was caused by the sequence selected. Furthermore, the TDLC channel was used which causes a bias to the results compared to AWGN.

The following tables describe the accuracy achieved with 2-5 samples with LP-SS when ED is used in AWGN channel with and without interference.

Figure 1: AWGN with and without interference. ED LR and IQ LR

The following tables describe similar results with ED, and for comparison IQ LR.

Figure 2: TDLC300ns with and without interference. ED LR and IQ LR

The following interferer is assumed per LP-SS binary sequence:

Figure 3: OOK interfering cell and pattern

From the results we observed that the number samples not impact significantly to the observed accuracy and timing error.

Number of samples does not significantly impact the accuracy of the LP-SS measurements across different scenarios

Use 2 LP-SS samples as a baseline to define RAN4 core requirements.

Conclude on the measurement accuracy in core requirements with simulations with final sequence design once RAN1 has agreed the LP-SS design.

For alignment purposes, would be better to use AWGN simulation results instead of TDLC. Also it is not clear whether the test cases use TDLC instead of AWGN.

Discuss if simulation results should be aligned with AWGN across companies.

OOK interfering cell

Currently RAN4 is simulating with an interference pattern that is selected from the available patterns from RAN1. Selecting overly good pattern will bias the RAN4 simulation results, and eventually the UE requirements as well as selecting opposite pattern will cause problems. In real deployment scenario it is highly likely that there will be some time offset between neighbouring OOK patterns, hence the deployment in the scenario is not realistic.

Discuss whether using OOK interferer is realistic or not, and if time offset should be applied.

Simulation timeline

For the RAN4#114 meeting, companies were requested to provide simulations for high level alignment. Running simulations can require a quite a lot of effort from companies and there are sometimes unexpected delays. The timeline between the next meeting is quite tight, so it would be better to aim to align simulations in August so there is enough time for companies to run simulations and align them to get good quality output.

Aim to collect aligned simulations across companies at RAN4#116 meeting.

Conclusion

The following Observations and Proposals were made:

Observation 1: Number of samples does not significantly impact the accuracy of the LP-SS measurements across different scenarios

Proposal 1: Use 2 LP-SS samples as a baseline to define RAN4 core requirements.

Proposal 2: Conclude on the measurement accuracy in core requirements with simulations with final sequence design once RAN1 has agreed the LP-SS design.

Proposal 3: Discuss if simulation results should be aligned with AWGN across companies.

Proposal 4: Discuss whether using OOK interferer is realistic or not, and if time offset should be applied.

Proposal 5: Aim to collect aligned simulations across companies at RAN4#116 meeting.