3GPP TSG-RAN WG4 Meeting #114bis		 			                      R4-2504477
Wuhan, China, 7th – 11st April, 2025
 
Agenda Item: 7.11.2

Source:	SKY Perfect JSAT

Title: Discussion on Ku-band co-existence study

Document for:	Discussion and Decision

- Introduction

This document proposes satellite parameters in the coexistence study between TN and NTN in the Ku-band. Additionally, it provides considerations for applying the coexistence study results to the Core Requirement.

At RAN4#114 in WF R4-2502990[1], it was agreed to manage the agreed items related to Ku-band coexistence studies as a tdoc.

### Sub-topic 2-1: Baseline simulation assumptions ### In order to coordinate the simulation assumptions, a Tdoc containing the aggregate agreements from all the WF will be maintained. For this meeting that will be in R4-2502315 which will be noted.

The satellite parameters summarized in R4-2502315[2] are as follows:

- 6a.2.2.1 Satellite parameters Set-1 satellite parameters are listed in Table 6a.2.2.1-2 according to TR 38.821 [6]. The satellite max Tx power can be calculated by the equation as below: maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi Table 6a.2.2.1-1: NRB configuration per BandWidth size and SCS Configuration Ku-band; NRB (100MHz BW). SCS 30 kHz; 273. Table 6a.2.2.1-2: Set-1 satellite parameters for co-existence study Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Satellite antenna pattern; Section 6.4.1 in [2]; Section 6.4.1 in [2]; Section 6.4.1 in [2]. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (Note 1); Ku-band (i.e. 11 GHz for DL); 9.1 m; 0.91 m; 0.91 m. Satellite EIRP density; 43 dBW/MHz; 13 dBW/MHz; 7 dBW/MHz. Satellite Tx max Gain; 58.5 dBi; 38.5 dBi; 38.5 dBi. 3dB beamwidth; 0.177 deg; 1.768 deg; 1.768 deg. Satellite beam diameter (Note 2); 110 km; 37 km; 19 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (Note1); Ku-band (i.e. 14 GHz for UL); 7.14 m; 0.714 m; 0.714 m. G/T; 30.9 dB K-1; 11.4 dB K-1; 10.9 dB K-1. Satellite RX max Gain; 58.5 dBi; 38.5 dBi; 38.5 dBi. NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB.. Table 6a.2.2.1-3: Other parameters for NTN satellite Parameters; NTN satellite; Remark. Carrier frequency; 14GHz(UL) / 11GHz(DL). The number of active UE (UL); 10 UEs and 27RBs per UE for GEO and LEO. The number of active UE (DL); 1; Same with TN. Traffic model; Full buffer. DL power control; NO. UL power control; See Session 6a.2.6.2. NTN satellite Noise Figure in dB; See Table 6a.2.2.1-4. Handover margin; 3dB. Table 6a.2.2.1-4: NTN satellite Noise Figure in dB Satellite; GEO; LEO 600; LEO 1200. G/T (dB K-1); 30.9; 10.9; 10.9. NF (dB); 3; 3; 3.

This document highlights the differences between the currently assumed GEO parameters and what we, as GEO operator, consider to be more realistic values in Ku-band, and proposes updated parameters accordingly. It also clarifies the combinations of VSAT and SAN models to be used in simulations and provides recommendations on which combinations should be evaluated.

- Analysis and Discussion

  - Satellite Parameters and VSAT-Satellite combination patterns

The currently assumed GEO satellite parameters differ from those of the GEO satellites that our company is operating or planning to operate. A comparison between the parameters of our envisioned GEO satellites is provided below.

Table 2.1-1: Comparison with realistic Satellite parameters: addition to R4-2502315 Table 6a.2.2.1-2: Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | GEO(realistic) | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 9.1 m | 2.5 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 39.0 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.177 deg | 0.400 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 250 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 2.5 m | 0.714 m | 0.714 m |
| G/T |  | 30.9 dB K-1 | 12.5 dB K-1 | 11.4 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 40.0 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |  |

Table 2.1-2: Comparison with realistic SAN parameters: addition to R4-2502315 Table 6a.2.2.1-4: NTN satellite Noise Figure in dB

| Satellite | GEO | GEO(realistic) | LEO 600 | LEO 1200 |
| --- | --- | --- | --- | --- |
| G/T (dB K-1) | 30.9 | 12.5 | 10.9 | 10.9 |
| NF (dB) | 3 | 3 | 3 | 3 |

At the RAN4 #114 meeting, our company performed a link budget analysis based on satellite parameters assuming realistic GEO satellites, and proposed a maximum EIRP of 88.2 dBm for VSAT terminals as a UE RF requirement (R4-2501691[3]). This proposal was agreed upon WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF (R4-2502288[4]). The parameters used in the link budget were then generalized and presented as representative values for GEO satellites in the above tables.

The currently assumed GEO satellite parameters are generally based on those used in the standardization of L/S-band in Rel-17 and Ka-band in Rel-18, with some adjustments made to account for differences in frequency. However, when comparing these parameters with those of the GEO satellites envisioned by our company, we find that the currently assumed GEO satellite parameters are larger antenna sizes, higher G/T, and higher antenna gain, resulting in discrepancies from more realistic values. We believe that coexistence studies should be conducted using realistic parameters.

Proposal 1: As shown in the table below, the GEO satellite parameters should be updated to more realistic values in Ku-band, and it is proposed that these be used for the coexistence study.

Table 2.1-3: Revised Table 6a.2.2.1-2: Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 2.5 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 39.0 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.400 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 2.5 m | 0.714 m | 0.714 m |
| G/T |  | 12.5 dB K-1 | 11.4 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  | 40.0 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

On the other hand, SHARP proposes four VSAT models for coexistence studies in RAN4#114bis meeting. It is proposed that coexistence studies be conducted using these four models as well.

The proposed VSAT models consist of the following four types: a 45×45 phased array antenna, a 78× 78 phased array antenna, a 100 cm aperture parabolic antenna, and a 60 cm aperture parabolic antenna. Some of these antenna models are assumed to be used with GEO satellites only, while others are assumed to be used with both GEO and LEO satellites. In order to clarify the combinations of satellite orbit types and VSAT models for which coexistence studies should be conducted, it is proposed to add a table summarizing these combinations to the “Co-existence simulation assumptions for NR NTN Ku band (Running document).”

Proposal 2: Create the table 6a.2.2.2-X of R4-2502315 to clarify the combinations of VSAT models and satellite models to be used for coexistence study. In the following matrix, we identify (via the X check mark) 4 configurations for VSAT models and 3 configurations for SAN that should be included in the simulations.

Table2.1-4:Additional proposal of Table 6a.2.2.2-X: VSAT-SAN combination patterns

| VSAT models SAN | 45 x 45 Phased Array | 60 cm Parabolic | 78 x 78 Phased Array | 100 cm Parabolic |
| --- | --- | --- | --- | --- |
| GEO | X | X | X | X |
| LEO 600 km | X | X | N/A | N/A |
| LEO 1200 km | X | X | N/A | N/A |

Handling of Coexistence Study Results

At RAN4#112, it was decided that the frequencies for TN and NTN in the coexistence study would use the same frequencies, at 11 GHz and 14 GHz. The details of the WF from R4-2413519[5] are provided below.

- Issue 2-4: Coexistence frequencies - Use 11GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band. - Issue 2-10: TN Parameters As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from: - The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz - The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

Additionally, at RAN4#112, the scenarios for the coexistence study were agreed upon as follows in the WF.

- Issue 2-2: Coexistence scenarios Adopt the interference scenarios in [Intelsat] R4-2411507 for the coexistence studies

Observation 1: It has been agreed in the WF that the coexistence study will be conducted using 11 GHz and 14 GHz for both TN and NTN.

At RAN4#112bis, the initial simulation results were presented in R4-2416140[6], showing that for Scenario No.5, a high ACS value is required for VSAT.

In response to this result, at RAN4#112bis, it was agreed in the WF that relaxation might be necessary when deriving core requirements based on the VSAT ACS simulation results from the coexistence study [5].

- Sub-topic 2-22: TN parameters NTN to NTN won’t be considered. Continue with the TN - NTN coexistence study noting that the applicability of the VSAT ACS simulation result may have to be relaxed for the core requirement derivation. Use the IMT parameters from TR38.922 version 0.3.0 for the IMT study on 15 GHz as a starting point.

Observation 2: It has been agreed in the WF that relaxation might be necessary when deriving core requirements based on the VSAT ACS simulation results from the coexistence study.

We verify the actual frequency allocation of TN, which is subject to coexistence studies. The frequency allocation status is summarized in R4-2411507[7], and an excerpt is provided below.

- 4.3 Interferers Figure 1 Priority 1 Configuration and Interfering Carriers And Figure 2 Priority 2 Configuration and Interfering Carriers

For the uplink in the 13.75-14.5 GHz range, the only TN frequency allocation under consideration is 14.8-15.35 GHz, as discussed at WRC-23, meaning that TN and NTN frequencies are separated by more than 300 MHz.

For the uplink in the 12.75-13.25 GHz range, there is currently no TN frequency allocation adjacent to NTN.

Observation 3: The only TN frequency allocation subject to coexistence studies with the NTN uplink in the 13.75-14.5 GHz range is 14.8-15.35 GHz, meaning that TN and NTN frequencies are separated by more than 300 MHz. For the uplink in the 12.75-13.25 GHz range, there is currently no TN frequency allocation adjacent to NTN.

For the downlink in the 10.7-12.75 GHz range, the 10.0-10.5 GHz was decided to allocate on a shared basis to 12 counties in the Region 2 for 5G/6G Services, as discussed at WRC-23, meaning that TN and NTN frequencies are separated by more than 200 MHz. On the other hand, in the Region 1 and 3, there is currently no TN frequency allocation close to the downlink in the 10.7-12.75 GHz range, and no allocation is even being considered.

Observation 4: The only TN frequency allocation subject to coexistence studies with the NTN downlink in the 10.7-12.75 GHz range is 10.0-10.5 GHz allocated to 12 countries in Region 2, meaning that TN and NTN frequencies are separated by more than 200 MHz.

As confirmed above, since the TN and NTN frequencies are separated by more than 300 MHz in the case of NTN Uplink and more than 200 MHz in the case of NTN downlink, the impact of NTN VSAT and SAN on the TN is expected to be significantly smaller. This differs greatly from the results of a coexistence study conducted under the assumption of using the same frequency or the frequencies adjacent to each other (i.e.: no frequency separation).

Observation 5: Since the TN and NTN frequencies are separated by more than 300 MHz in the case of NTN Uplink and more than 200 MHz in the case of NTN downlink, the impact of NTN VSAT and SAN on the TN is expected to be significantly smaller. This differs greatly from the results of a coexistence study conducted under the assumption of using the same frequency or the frequencies adjacent to each other (i.e.: no frequency separation).

As stated in Observation 2, it has been agreed that relaxation might be necessary when deriving core requirements based on the VSAT ACS simulation results from the coexistence study. Considering that the frequencies are not actually adjacent in practice, this relaxation could also be applied to other coexistence study scenarios.

Proposal 3: When considering ACS/ACLR for VSAT UE and SAN based on the coexistence test results, RAN4 should consider that the actual frequency assignments of TN and NTN are separated by more than 300 MHz for NTN Uplink and 200 MHz for NTN Downlink. Given this separation, it should be noted that relaxation may be necessary when deriving core requirements from the simulation results.

- Conclusions

Proposal 1: As shown in the table below, the GEO satellite parameters should be updated to more realistic values in Ku-band, and it is proposed that these be used for the coexistence study.

Table 6a.2.2.1-2: Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 2.5 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 39.0 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.400 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 2.5 m | 0.714 m | 0.714 m |
| G/T |  | 12.5 dB K-1 | 11.4 dB K-1 | 10.9 dB K-1 |
| Satellite RX max Gain |  | 40.0 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Proposal 2: Create the table 6a.2.2.2-X of R4-2502315 to clarify the combinations of VSAT models and satellite models to be used for coexistence study. In the following matrix, we identify (via the X check mark) 4 configurations for VSAT models and 3 configurations for SAN that should be included in the simulations.

Table 6a.2.2.2-X: VSAT-SAN combination patterns

| VSAT model SAN | 45 x 45 Phased Array | 60 cm Parabolic | 78 x 78 Phased Array | 100 cm Parabolic |
| --- | --- | --- | --- | --- |
| GEO | X | X | X | X |
| LEO 600km | X | X | N/A | N/A |
| LEO 1200km | X | X | N/A | N/A |

Observation 1: It has been agreed in the WF that the coexistence study will be conducted using 11 GHz and 14 GHz for both TN and NTN.

Observation 2: It has been agreed in the WF that relaxation might be necessary when deriving core requirements based on the VSAT ACS simulation results from the coexistence study.

Observation 3: The only TN frequency allocation subject to coexistence studies with the NTN uplink in the 13.75-14.5 GHz range is 14.8-15.35 GHz, meaning that TN and NTN frequencies are separated by more than 300 MHz. For the uplink in the 12.75-13.25 GHz range, there is currently no TN frequency allocation adjacent to NTN.

Observation 4: The only TN frequency allocation subject to coexistence studies with the NTN downlink in the 10.7-12.75 GHz range is 10.0-10.5 GHz allocated to 12 countries in Region 2, meaning that TN and NTN frequencies are separated by more than 200 MHz.

Observation 5: Since the TN and NTN frequencies are separated by more than 300 MHz in the case of NTN Uplink and more than 200 MHz in the case of NTN downlink, the impact of NTN VSAT and SAN on the TN is expected to be significantly smaller. This differs greatly from the results of a coexistence study conducted under the assumption of using the same frequency or the frequencies adjacent to each other (i.e.: no frequency separation).

Proposal 3: When considering ACS/ACLR for VSAT UE and SAN based on the coexistence test results, RAN4 should consider that the actual frequency assignments of TN and NTN are separated by more than 300 MHz for NTN Uplink and 200 MHz for NTN Downlink. Given this separation, it should be noted that relaxation may be necessary when deriving core requirements from the simulation results.

- References

- R4-2502990, Way Forward for [114][313] NR_NTN_Ku_Band_General, RAN4 #114

- R4-2502315, Co-existence simulation assumptions for NR NTN Ku band (Running document), RAN4 #114

- R4-2501691, Discussion on UE RF requirements for NR NTN Ku band; SKY Perfect JSAT, RAN4 #114

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#114

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General; Moderator (Eutelsat Group), RAN4#112

- R4-2416140, NTN Ku-band – coexistence study; Ericsson

- R4-2411507, Ku Band Co-existence: Intelsat, RAN#112