3GPP TSG-RAN WG4 Meeting #114-bis 	R4-2504478

Wuhan, China, April 7th – 11th, 2025

Agenda Item: 7.7.3

Source: MediaTek inc.

Title: Discussion on low-band CA switching

Document for: Discussion

# Introduction

The low band carrier aggregation has been one of the network operators’ interests. The benefit on low band is mainly due to the large coverage compared to that using higher frequencies. Nevertheless, although RAN4 RF spec has already covered the low bands CA band combination, yet this has never been implemented in real field. This is mainly due to challenging task to implement low-band CA on handheld UE especially when there are some combinations has almost no frequency gap between UL transmission and DL reception channels. To overcome such obstacle and to better utilize the low band fragmented spectrums owned by operator, a new WI was approved in RANP#106 [1], in which, 25 companies agreed to introduce low-band CA via switching.

This contribution provides discussion on the low-band CA with the new carrier switching feature.

# Discussion

The main objectives of the work item is to introduce the physical procedures and requirements to enable low band CA via switching, detailed below:

### Objective of SI or Core part WI or Testing part WI Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4]

The PHY procedure as described in the WID objective is that a UE is configured with two low band carriers, where the UE operates on a single carrier at a time. The switching configuration is set by RRC signalling (semi-static). The procedure can be depicted as in Figure 1 below. It should be noted that the PCell of CA is within carrier 1, while the SCell is within carrier 2.

Figure 1: UE carrier switching procedure in low band CA.

RAN4 has already discussed several RRM aspects related to the low band CA operation and the agreements have been captured in the WF [2]:

Issue 1-1-1: RRM work scope

- Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion. - Switch delay requirements Agreement: - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM. - Synchronization requirements for FDD and synchronization accuracy at BS antenna connectors or radiated interface boundaries Agreement: - No need to define new cell phase synchronization requirement in RRM spec for R19 LB CA via switching. - SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used. - other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application. - Potential impacts to the PCell related requirement - FFS

Activated/deactivated SCell L3 measurement: When SSB based SCell operation is used for R19 LS CA visa switching, for activated/deactivated SCell, UE shall perform L3 measurements for SCell. Hence, RAN4 to reuse existing activated/deactivated SCell L3 measurement requirements for SDL L3 measurements.

- When SSB based SCell operation is used for R19 LS CA visa switching, for activated/deactivated SCell, RAN4 to reuse existing activated/deactivated SCell L3 measurement requirements for SDL L3 measurements.

Other SDL SCell requirements: To our understanding, the main motivation of this WI is to enable the low band CA operation with minimum change in RRM requirements, hence, enabling this CA combination in the field. Thus, RAN4 to reuse the existing SCell activation/deactivation delay and related interruption requirement for R19 LB CA. In addition, RAN4 to reuse the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement for R19 LB CA. Nevertheless, further discussion is needed to justify the new requirements, if any.

- RAN4 to reuse the existing SCell activation/deactivation delay and related interruption requirement for R19 LB CA, further discussion is needed to justify the new requirements, if any.

- RAN4 to reuse the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement for R19 LB CA, further discussion is needed to justify the new requirements, if any.

Issue 1-1-2: Assumptions for R19 LB CA via switching

- (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us

For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Due to the limited time of this WI, RAN4 to focus on discussing and defining requirements for SDL SCell operation for SSB-based Scell, i.e. RRM to deprioritise the scenario of SSB-less Scell based.

- RAN4 to focus on discussing and defining requirements for SDL SCell operation for SSB-based Scell, i.e. RRM to deprioritise the scenario of SSB-less Scell based.

MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Based on agreement from the previous meeting, RRM group down-selected the values to either 4.67us or 3us. Given that both values are within the CP length, hence there is no major difference between these two values on the 5G system performance.

- The received timing difference between the two bands at UE is within a value smaller or equal to CP length.

Issue 1-2-1: Interruption requirement (if needed) for carrier switching between FDD CC and SDL CC [3]

Recommended WF - RAN4 to discuss interruption requirements due to switching procedure after RF has agreements on the time duration of switching - slot level or symbol level interruption requirement for carrier switching in R19 LB CA - whether to define requirements (such as interruption requirements) for a configuration where, in addition to the LBCA CCs to be used with a switching pattern, there may be an additional CC which can be used without switching

In order to enable the switching procedures from RAN4 point of view, one of the main aspect is switching delay and location in time. RAN4 RRM looked into this issue in the previous meeting and companies are discussing interruption requirements on slot level or symbol level. It should be pointed out that slot level is too slow for such feature and might have major negative impact the system performance and hence hinder the successful implementation of such feature in real field. Therefore, it is suggested that the interruption shall be defined as within symbol level only. Also, RAN4 shall discuss the location of the interruption and to limit the interruption location outside the PDCCH symbols.

- RAN4 RRM to define interruption requirements for carrier switching in R19 LB CA on a symbol level only.

- RAN4 RRM to limit the interruption location for carrier switching in R19 LB CA outside the PDCCH symbols.

# Summary

In summary:

Proposal 1: When SSB based SCell operation is used for R19 LS CA visa switching, for activated/deactivated SCell, RAN4 to reuse existing activated/deactivated SCell L3 measurement requirements for SDL L3 measurements.

Proposal 2: RAN4 to reuse the existing SCell activation/deactivation delay and related interruption requirement for R19 LB CA, further discussion is needed to justify the new requirements, if any.

Proposal 3: RAN4 to reuse the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement for R19 LB CA, further discussion is needed to justify the new requirements, if any.

Proposal 4: RAN4 to focus on discussing and defining requirements for SDL SCell operation for SSB-based Scell, i.e. RRM to deprioritise the scenario of SSB-less Scell based.

Proposal 5: The received timing difference between the two bands at UE is within a value smaller or equal to CP length.

Proposal 6: RAN4 RRM to define interruption requirements for carrier switching in R19 LB CA on a symbol level only.

Proposal 7: RAN4 RRM to limit the interruption location for carrier switching in R19 LB CA outside the PDCCH symbols.

# References

- RP-243317, “New WID on low band carrier aggregation via switching”, Madrid, Spain, Dec 2024.

- R4-2502588, “WF on RRM requirements for NR_LBCA_Sw”, Apple, Athens, GR, Feb 2025.

- R4-2500527, “Topic summary for [114][213] NR_LBCA_Sw”, Apple, Athens, GR, Feb 2025.