3GPP TSG-RAN WG4 Meeting #114-bis 	R4-2504479

Wuhan, China, April 7th – 11th, 2025

Agenda Item: 7.23.3.1

Source: MediaTek inc.

Title: Discussion on SBFD UE-to-UE CLI

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2] and collection of companies proposals in Topic summary [3]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion on the RRM impact for CLI handling mechanisms

In legacy 5G NR TDD, the RRM specification cover the CLI measurements delay and accuracy for SRS-RSRP and CLI-RSSI. The UE reports these measurements to the gNB to help the dynamic allocation of uplink/downlink in time. Nevertheless, these measurements are based on L3 measurements, i.e. it requires L3 filtering, which has longer measurement delay compared to L1 measurements. Therefore, in SBFD Rel-19 WID objective, new CLI measurements and reporting shall be introduced for L1 based UE-to-UE CLI with new measurement quantities, such as L1-SRS-RSRP, L1-CLI-RSSI.

# Discussion on the RRM impact for CLI handling mechanisms

In the previous meeting, RAN4 RRM reached agreement with related further study point, which are captured in the WF.

#### Issue 1-1-3: Measurement resources - Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSRI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types. #### Issue 1-1-5: Scheduling restriction

Issue 1-1-3: Measurement resources: 
Provided that the measurements methods, resources should be defined by RAN1 WG while measurements accuracy and delay requirements should be defined by RAN4 RRM. Thus, RAN4 should consider the agreements made in RAN1:

Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting - Agreement: there is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19. Agreement For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

Meanwhile, RAN4 made the following agreement:

- Agreements - Define measurement requirements assuming wideband reporting for L1-SRS-RSRP and L1-CLI-RSSI, and do not need to consider subband reporting. FFS if UE is able to measure both DL subbands simultaneously in case of DUD and how such measurement capability affects measurements period and accuracy requirements. - RAN4 to define requirements for L1-CLI-RSSI and L1-SRS-RSRP measurement for semi-static, aperiodic and periodic resource types

A similar issue has been discussed in RAN1 on whether UE is able to measure CSI-RS resources on both DL subbands simultaneously in case of DUD. RAN1 reached the following agreement for that issue:

Agreement[RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

Therefore, following the same logic for that issue of CSI processing timeline from RAN1, RAN4 can assume that the UE to measure both DL subbands sequentially in case of DUD. Nevertheless, RAN4 to discuss whether measurement period and accuracy is impacted.

- A similar issue of CSI processing timeline was discussed in RAN1 resulting by scaling the processing timeline with a factor of two.

- For L1-SRS-RSRP and L1-CLI-RSSI, UE is not expected to measure both DL subbands simultaneously, yet, RAN4 to further study whether measurement period and accuracy is impacted.

Issue 1-1-5: Scheduling restriction:

Agreements (from Thu AH session) - For L1-SRS-RSRP, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restriction does not apply to the symbols with SRS if UE is able to support FDM-ed DL reception and SRS measurement if RAN1 will define optional UE capability for FDMed DL reception and SRS measurement; otherwise (if UE does not support the capability or RAN1 does not define the capability), scheduling restriction applies. - The restricted symbols at least include symbols for measurement resource and Y symbols before symbols for measurement resource and Y is re-using same number as R16 requirements, - Note: this time has accounted for the UL/DL switching time. - FFS whether there is restriction on symbols after the symbols for measurement resource considering UL/DL transition. - For L1-CLI-RSSI Methods #1, - For UL, scheduling restrictions always apply, for both FR1 and FR2. - For DL, scheduling restrictions do not apply except when CLI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2. - The restricted symbols at least include symbols for measurement resource, FFS whether restriction on additional symbols is needed. - For L1-CLI-RSSI Method #3, scheduling restriction requirements are FFS

For the measurements of L1-CLI-RSRP, given that the UE is configured to perform measurements (i.e. DL behaviour) within UL subband, therefore, the scheduling restriction should cover at least twice the UL/DL transition period. In addition, in the previous meeting, RAN4 agreed to define restricted symbol before the reference signal and FFS the symbols after the reference signal resources. Yet, it should be pointed out that the UL/DL switching is about 13us, which is larger than a CP, hence, the number of restricted symbols should be rounded up to one symbol.

- the number of restricted symbols after the symbols for measurement resource should be an integer number and consider UL/DL transition, hence, the number of symbols should equal to one symbol.

For L1-CLI-RSSI Method #3, given that Method#3 is based on performing measurement within UL subband then RAN4 should follow the requirements defined for Method#2 as baseline.

- L1-CLI-RSSI Method #3, RAN4 to follow the requirements defined for Method#2 as baseline for defining scheduling restriction for Method #3.

# Discussion on remaining issues

Issue 1-1-2: Rx beam:

- Recommended WF - Agree on: The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period. - Further discuss: - RAN4 can capture the original statements of RAN1 agreements. If necessary minor change based on the statement of RAN1 agreements can be considered. - directly refer to RAN1 spec instead of repeating all details in RAN spec

In the previous meetings, the above way forward was captured in the topic summary. Regarding the first bullet ‘The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period’, seems agreeable. Besides, regarding the second bullets, our preference is that RAN4 spec directly refer to RAN1 spec instead of repeating all details in RAN4 spec.

- The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

- RAN4 spec directly refer to RAN1 spec instead of repeating all details in RAN4 spec.

Issue 1-1-6: Measurement restriction:

Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements - For L1-SRS-RSRP, measurement requirements for L1-SRS-RSRP do not apply if L1-SRS-RSRP measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - For L1-CLI-RSSI Method #1, measurement requirements for L1-CLI-RSSI do not apply if L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, FFS whether to consider exception case - Option 1: do not consider exception case, i.e. measurement requirements for L1-CLI-RSSI do not apply - Option 2: consider exception case, i.e. measurement requirements for L1-CLI-RSSI apply when UE can perform L1-CLI-RSSI measurement and other measurement at same time. - For L1-CLI-RSSI Method #3, - If the measurement timing is same as L1-SRS-RSRP, as baseline, measurement restriction requirements are same as L1-SRS-RSRP. - If the measurement timing is same as L1-CLI-RSSI Method #1, as baseline, measurement restriction requirements are same as L1-CLI-RSSI Method #1. - FFS whether any adaptation is needed on top of baseline. Recommended WF for collision between different CLI measurements - FFS whether to capture in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. - FFS how to handle collision between L1-SRS-RSRP and L1-CLI-RSSI measurement resources - Option 1: prioritize L1-SRS-RSRP measurement - Option 2: prioritize L1-CLI-RSSI measurement - Option 3: other

For collision between L1 CLI measurements and L1/L3 RRM measurements, given that the reference timing for L1-SRS-RSRP is based on UL timing, hence, if the CLI L1-SRS-RSRP measurement is overlapped with another measurements the UE might not be able to perform both measurements at the same time. Therefore, the following can be supported: ‘For L1-SRS-RSRP, measurement requirements for L1-SRS-RSRP do not apply if L1-SRS-RSRP measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps’. For L1-CLI-RSSI Method #1, although the reference timing follows the DL timing yet the concern in here is that the single sample for measurements might be impacted by the other measurement that happens simultaneously. Thus, our preference is: ‘For L1-CLI-RSSI Method #1, measurement requirements for L1-CLI-RSSI do not apply if L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, FFS whether to consider exception case’. In addition, L1-CLI-RSSI Method #3, the reference timing is based on DL, yet the measurement happens in the UL subbands, which means the UE might require scheduling restriction to perform the measurements. Thus, the following shall be supported: ‘Measurement restriction for Method #3 follow Method #2 and Method #1 as baseline’.

For collision between different CLI measurements, it is acceptable to capture that in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

- For L1-SRS-RSRP, measurement requirements for L1-SRS-RSRP do not apply if L1-SRS-RSRP measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

- For L1-CLI-RSSI Method #1, measurement requirements for L1-CLI-RSSI do not apply if L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

- For L1-CLI-RSSI Method #3, as baseline, measurement restriction requirements are same as Method#1 and Method#2.

- For collision between different CLI measurements, it is acceptable to capture that in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Issue 1-1-7: Measurement capability:

Provided that measurement capability is based on the RAN1 design and agreements, hence, RAN4 not to discuss measurement capability requirements unless requested by other WGs.

- RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Issue 1-1-8: Report mapping:

RAN1 sent an LS with the following information [4]:

The following working assumption was made in RAN1 on L1-based CLI reporting since RAN1 specifications must capture the format and bitwidths of the CLI reports. Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.. RAN1 assumes that the reporting range and granularity for L3 SRS-RSRP measurement in Table 10.1.22.1.2-1, TS38.133 can be reused for L1 SRS-RSRP. In addition, the reporting range and granularity for L3 CLI-RSSI measurement in Table 10.1.22.2.2-1 of TS38.133 can be reused for L1 CLI-RSSI. For a CLI report, RAN1 agreed that the absolute value of the most interfered (largest) measured L1-SRS-RSRP or L1-CLI-RSSI is reported. RAN1 also agreed to use differential reporting for L1 SRS-RSRP and L1 CLI-RSSI when the number of reported measurement resources is larger than one, in the same manner as for existing L1-RSRP/SINR reporting. RAN1 would like to ask RAN4 whether the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI are reasonable. RAN1 also would like to check with RAN4 whether there is any issue with the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI.

RAN4 shall confirm the reuse of R16 existing reporting range and granularity for L3 SRS-RSRP measurement in Table 10.1.22.1.2-1, TS38.133 and for L1 SRS-RSRP. Also, RAN4 can confirm the reuse of R16 reporting range and granularity for L3 CLI-RSSI measurement in Table 10.1.22.2.2-1 of TS38.133 for L1 CLI-RSSI.

However, RAN4 to further study the necessity of introducing differential L1 SRS-RSRP and differential L1 CLI-RSSI.

- RAN4 shall confirm the reuse of R16 existing reporting range and granularity for L3 SRS-RSRP and CLI-RSSI measurements in Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1, TS38.133 and for L1 SRS-RSRP and L1 CLI-RSSI, respectively.

- RAN4 to further study the necessity of introducing differential L1 SRS-RSRP and differential L1 CLI-RSSI.

Issue 1-1-12: Requirement applicability:

In the previous meeting, RAN4 discussed applicability of L1-SRS-RSRP and L1-CLI-RSSI measurement requirement in terms of frequency location of measurement resources. In our view, RAN4 should follow RAN1 agreement in terms of measurements resources, hence, L1 CLI measurement resources must be within the DL active BWP of the UE, where L1-SRS-RSRP confined within UL subband, while L1-CLI-RSSI based method #1 and #3 to be confined within DL and UL subband, respectively.

Agreement (from RAN1#118) For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

- L1 CLI measurement resources must be within the DL active BWP of the UE, where L1-SRS-RSRP confined within UL subband, while L1-CLI-RSSI based method #1 and #3 to be confined within DL and UL subband, respectively.

# Summary

In this contribution, we have the following proposals:

Observation １: A similar issue of CSI processing timeline was discussed in RAN1 resulting by scaling the processing timeline with a factor of two.

Proposal 1: For L1-SRS-RSRP and L1-CLI-RSSI, UE is not expected to measure both DL subbands simultaneously, yet, RAN4 to further study whether measurement period and accuracy is impacted.

Proposal 2: the number of restricted symbols after the symbols for measurement resource should be an integer number and consider UL/DL transition, hence, the number of symbols should equal to one symbol.

Proposal 3: L1-CLI-RSSI Method #3, RAN4 to follow the requirements defined for Method#2 as baseline for defining scheduling restriction for Method #3.

Proposal 4: The method used by the UE to determine the TCI state to be used in the L1 CLI measurement will not affect the measurement period.

Proposal 5: RAN4 spec directly refer to RAN1 spec instead of repeating all details in RAN4 spec.

Proposal 6: For L1-SRS-RSRP, measurement requirements for L1-SRS-RSRP do not apply if L1-SRS-RSRP measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

Proposal 7: For L1-CLI-RSSI Method #1, measurement requirements for L1-CLI-RSSI do not apply if L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps.

Proposal 8: For L1-CLI-RSSI Method #3, as baseline, measurement restriction requirements are same as Method#1 and Method#2.

Proposal 9: For collision between different CLI measurements, it is acceptable to capture that in RAN4 spec that UE is not expected to perform measurement using Method #1 and Methods#3 in the same OFDM symbol.

Proposal 10: RAN4 not to discuss measurement capability requirements unless requested by other WGs.

Proposal 11: RAN4 shall confirm the reuse of R16 existing reporting range and granularity for L3 SRS-RSRP and CLI-RSSI measurements in Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1, TS38.133 and for L1 SRS-RSRP and L1 CLI-RSSI, respectively.

Proposal 12: RAN4 to further study the necessity of introducing differential L1 SRS-RSRP and differential L1 CLI-RSSI.

Proposal 13: L1 CLI measurement resources must be within the DL active BWP of the UE, where L1-SRS-RSRP confined within UL subband, while L1-CLI-RSSI based method #1 and #3 to be confined within DL and UL subband, respectively.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2502699, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Athens, Greece, Feb 2025.

- R4-2500536, “Topic summary for [114][222] NR_duplex_evo”, Huawei, HiSilicon, Athens, Greece, Feb 2025.

- R1-2501620, “LS on L1 based UE-to-UE CLI measurement and reporting”, RAN1, Athens, Greece, Feb 2025.